| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-8 |  | We have to test a epilator with AC Adapter (Power supply : Electric) which can’t work without the Adapter. Non-rechargeable appliance. The epilator that, works at 12 V DC, is using AC/DC adapter (Output 12 V DC, 400mA) , (Input 100-240V, 7W) as the power supply. Q: According to which standard we shall test this Adapter. Other than IEC 60335-2-8. | You can test the complete appliance (including item 7) to IEC 60335-2-8 taking into account Annex G of Part 1. In this case the appliance is treated as a mains operated appliance (supplied in two parts) with the hand-held part being a part of class III construction. It is also acceptable to test item 7 to IEC 61558-2-6 and the handle part to IEC 60335-2-8 as a class III appliance. Jordan 3: Confirmed | 61(Rimini/Chairman)02a – Jordan 3 | 61/4285/RM | Jordan |
| 60335-2-8 | 4.101, 6.2, 7.12 | For IEC 60335-2-8:2002: 1. Why subclause 4.101 “Before and during the tests, cutting heads are lubricated with a few drops of light oil” is deleted in the new edition? 2. How to understand subclause 6.2? Or parts of class III construction need not be labeled with IP code? 3. Can “in water” of subclause 7.12 be changed to “under water”? | IEC 60335-2-8:2002 1. BECAUSE AT THE TYPE TEST LEVEL THE APPLIANCE MUST COMPLY WITH THE STANDARD WITHOUT THE ARTIFICAL INTERVENTION OF LUBRICATION OIL. 2. CORRECT 3. NO | None | 61(Jeju/Chairman)2a | China |
| 60335-2-8 | 6.2 | Here we have another confusion need your guide. EN 60335-2-8, 6.2 says: But as far as I know in the IEC60083 plug, there are only in Australia E6624 plug and in Germany DIN49440 plug is plashproof ( IPX4) plug Is that the standard’s intention that only these two kind of plug configuration can be used in shaver direct plug in transformer ? I’m curious then how to handle if product used in other countries, See attached photo, do adaptors no, 1, 2, 3 comply with 6.2 ? | The requirement in 6.2 should not cause a problem, because the transformer with pins for insertion into a socket outlet must be class II according to 6.1. You then mount the transformer with pins for insertion into a socket outlet as detailed in 15.1.2 4th paragraph and carry out the IPX4 test accordingly. Then according to 15.1 The appliance shall then withstand the electric strength test of 16.3 and inspection shall show that there is no trace of water on insulation which could result in a reduction of clearances and creepage distances below the values specified in clause 29. NOTE The external enclosure is carefully wiped to remove any surplus water before inspection. Care has to be taken when dismantling to avoid displacing any water within the appliance. If you follow the standard there should be no problem in carrying out the tests. For the pin configuration applying in various countries you need to review the country’s particular national deviations relating to plugs that can be fitted to class II appliances. This means that plugs other than those identified can also be used. China 1: Confirmed but add “This means that plugs other than those identified can also be used.” | 61(Oslo/Chairman)02a | 61/3827/RM | China |
| 60335-2-8 | 7.1 | a) In subclause 7.1, it requires that, Appliances shall be marked with "rated power input in watts or rated current in amperes". For battery-powered shavers, does this requirement apply or not apply? Your reply will be well appreciated. b) For example, a 3V battery powered shaver, the measured input current is 540mA and the calculated power input is 1.6W, some manufacturers say it is meanless to mark these values ( power input or input current) on the shavers, so they don't make them on the shaver. It seems for this situation marking of power input or input current gives no information. What do you think | - Yes it applies if the battery must be taken out of the shaver for recharging or if the battery is non-chargeable. If the battery is recharged in the appliance, then Annex B applies and as per 7.15 of Annex B the mains connected part must be marked with the complete appliance “rated power input in watts or rated current in amperes" - If it is not marked it is not in compliance with the standard. Also how do you apply normal operation (3.1.9 of part 2-8) and how do you test for clause 10. China 2 – After discussion the TC agreed that it was not necessary to mark the rated current and rated power input for battery operated appliances using either batteries that are non-rechargeable or batteries that are charged outside of the appliance. Secretary to review IEC 60745 annex related to battery operated tools to determine what may be applicable to battery operated appliances, and prepare DC for review by NCs. | None | 61/3777e/RM | China |
| 60335-2-8 | 15.2 | Question: There are some power plugs with incorporated AC/DC converters available on the market for shavers which bear the IP X4 or IPX7 classification. Some of these power plugs with incorporated AC/DC converters the comply as a standalone unit with the IPX4 or IPX7 classification (see picture), but when combined with a socket outlet do not comply with the IPX4 or IPX7 classification. Questions: 1a) Can a product with a certain IP class classification be equipped with a plug with a lower IP classification? 1b) Does it make a difference if the particular standard of the product requires a certain IP classification? 2a) Is the IP classification of a plug always related to the combination of plug in the socket? 2b) Or is the IP classification of a plug (for example plug with incorporated AC/DC converter) related to the plug as standalone. Rationale: (See question) Proposal (if any): Q1a- YES in 6.2 of Part 2-8 IPX4 is required for plug-in in appliances with means that it is assumed that IP degree corresponds only to the enclosure and not to the pins. Q1b- it depends on correspondent Part 2 Q2a- YES, when needed it shall be required the use of a specific socket-outlet. Q2b- NO, but in some Part 2 it is not required IP for the plug (i.e. part 2-8) It is pointed that some plug-in transformers, with IP classification and for general use, need to have a marking and instructions to indicate that are only for indoor use when may be used for household appliances and the plug cannot meet the IP degree with the standard IP socket outlet. EXPLANATION: This decision is similar to OSM/HA 459 | Q1a – Confirmed. See the requirement in 15.1. Q1b – In general, No, for appliances with pins for insertion into a socket outlet. See the requirement in 15.1. Q2a – This is not a question for TC 61. It should be referred to SC23B since it asks about “a plug.” Q2b – The answer to this question depends on whether the power supply with pins for insertion into a socket-outlet is for general use or specific use. If it is for general use then the question should be referred to TC 96. If it is for specific use then see the answer to Q1a and Q2a. The statement about plug-in transformers following the answer to question 2b should be deleted. | 61(Seattle/CTL Liaison)02d - 04/10 (0787) | 61/4080ea/RM | CTL Liason |

Attachment 61(Oslo/Chairman)02a –

1(Germany plug)                              2 (EN 50075 plug)         3 (UK plug)

Attachment 61(Seattle/CTL Liaison)02d - 04/10 (0787):

Attachment 61(Rimini/Chairman)02a – Jordan 3: