| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-24 | 7.1 | Question: Which one Is the correct interpretation on the requirement “Appliances shall be marked with the maximum rated wattage of lamps, in watts” (5th dashed item of sub-clause 7.1 of IEC60335-2-24:2002) in the followings? a) Lamps whose replacement by the user is easily available or intended by the appliance manufacturers shall be marked with their maximum rated wattage in watt and the others are not necessary. b) All the lamps shall be marked with their maximum rated wattage in watt regardless of their replacement by the user. Rationale: The requirement of the marking of the maximum rated wattage of lamps in watt is to avoid excessive current in the power supply circuitry to the lamps in the appliance due to the wrong lamps replacement by the user. See also the draft amendment, 61C/490/CDV Proposal (if any): a) is correct | According to the current standard (b) is correct. The proposal in the 61C CDV was to cover LEDs on printed circuit boards. The text may be changed before stage IS is reached. | None | 61/4212e/RM | CTL Liason |