| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-41 | Foreword; 22.101; 7.1; 11.7 | I'm working with one of China Household Appliance TCs. Recently I find some inconsistencies in IEC 60335-2-41 and its Amend.1 as follows: Regarding IEC 60335-2-41: 1. In FOREWORD, in "additional differences exist in some countries", -6.1: The pumps with the two kinds of working conditions have the same class "01"(Japan". 2.In the "Note" of subclause 22.101, "15.101" should be 15.1.1, since there is no "15.101"subclause. Regarding Amend.1: 1, In subclause 7.1, "shall not be less than 35C" in the first sentence is inconsistent with "exceeds 35C" in the second sentence. 2.In subclause 11.7, the marking condition in these two paragraphs are the same, ie."If the pump is marked with a maximum period of operation", but the test procedure is different. Which test should be carried out? | 1. This was the wording provided by the Japanese National Committee, however I suspect the second occurrence of “OI” should be “O”. 2. Your are correct, thank you for pointing out this error, it will be corrected in the next edition of the standard. However, it makes no difference to the normative elements in the standard. Regarding Amend. 1 1. There is no inconsistency. Please read the text carefully. All pumps must be marked with the maximum liquid temperature. This marked value cannot be less than 35 oC. If it is greater than 35 oC, then it can also be marked with a maximum period of operation when pumping liquid at this higher temperature. If it is not marked with a maximum period of operation when pumping liquid at this higher temperature then it means that it can pump such high temperature liquid continuously. 2. They are not the same. If a pump is for continuous operation, then only the first paragraph is relevant. If a pump is marked with a maximum period of operation then both paragraphs are applied. The first with the liquid at 35 oC which is carried out until steady conditions are reached and the second with the liquid at the marked temperature which is carried out for a time period as specified in the text. | None | 61(Beijing/Chairman)2a | China |
| 60335-2-41 | 15.1.1 | I'm writing to ask a question still concerning IEC60335-2-41. In the second dash of Subclause 15.1.1, here we have two understandings, first, 1.3 times the pressure occurring at maximum depth or 1.3 times the pressure at 15m; second, 1.3 times the pressure occurring at maximum depth or the pressure at 15m. Which one is correct? Please give us a reply. | The second dash item means that you test at a pressure of 1,3 times the pressure occurring at the maximum operating depth or 1,3 times the pressure occurring at 15 m whichever is greater. | None | 61(Durban/Chairman)2a | Hong Kong |
| 60335-2-41 | 25.7 | I have question from THAI manufacturer concern the requirement of IEC60335-2-41:2002+A1 Clause 25.7. Water pump, as attach file, is usually connected to water tap and place on the floor during use. Is it stationary appliance or fixed appliance? Or ? According to Clause 25.7, The word “Fixed pumps” is BOLD but no definition. Are these kinds of pump can classified as “Fixed pumps” ? and What kind of cord shall be used between ordinary polychloroprene sheathed cord (code designation 60245 IEC 57) and heavy polychloroprene sheathed cord (code designation 60245 IEC 66)? | The term Fixed appliance is defined – see definition 3.5.4 in Part 1. In the Parts 2 the word “appliance” is sometimes replaced another noun – in the case of Part 2-41 the word “pump” is used – so the term “fixed pump” is defined as per 3.5.4 in Part 1 (see also the text in the Foreword that states: “Words in bold in the text are defined in Clause 3. When a definition concerns an adjective, the adjective and associated noun are also in bold”) So taking the above explanation into account and your statement that it “is usually connected to water tap and place on the floor during use” means that it is not a fixed pump (appliance). If it weighs 18 kg or more then it would be classified as a stationary pump (appliance – see 3.5.3) and if it weighs less than 18 kg it would be classified as a portable pump (appliance – see 3.5.1) From the above you should be able to derive the type of cord to be used based on the weight of the pump and if it is for outdoor use or indoor use only. Thailand 1: Confirmed. | None | 61/4080ea/RM | Thailand |