| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-9 60335-2-78 |  | Particular requirements for toasters, grills, roasters and similar appliances And on document IEC 60335-2-78:1995, 1st edition: Particular requirements for outdoor barbecues: As far as the German National Committee is understanding the content of IEC 60335-2-9 all appliances described in the scope of the standard are only for indoor use. In our opinion IEC 60335-2-78 is particularly designed only for barbecues for outdoor use. Attention is drawn to the fact that some appliances which are within the scope of IEC 60335-2-9 like rotary grills, radiant grills, raclette grills, portable ovens, waffle irons, etc., are often used outdoor. This misuse of the above mentioned appliances is not intended in the standard. Their design is often not in accordance with the basic requirements for an outdoor use. IEC 60335-2-78 which is only for outdoor barbecues applicable also does not cover such appliances. We are informed test houses have for the time being some problems to approve appliances for outdoor use depending on part -2-9. Because there is a lack in the standard they make use of some clauses of part -2-78. 2 61/1567/Q Therefore we are of the opinion that the scope of IEC 60335-2-9 should be expanded for appliances for outdoor use and necessary requirements should be added in the relevant clauses. As an alternative solution IEC 60335-2-78 could be expanded for the aforementioned appliances with some specific modifications. If TC 61 is also of the opinion that an improvement of one of the relevant part 2s of the standard is advisable the German National Committee is prepared to provide a detailed proposal in due time. In that case we would like to know which part 2 has to be modified. | In response to the question of interpretation, Part 2-78 is the applicable Part 2 for portable outdoor cooking appliances as the standard contains specific requirements that would be applicable to portable cooking appliances for outdoor use. | None | 61/1673/RM | Germany |
| 60335-2-9; 60335-2-78 | Scope | The amendment 1 of IEC 60335-2-9 introduces the outdoor appliance requirements. This amendment should normally cover all outdoor appliances and especially barbecue for which all requirements of IEC 60335-2-78 have been rewrited in amendment 1. My trouble is in notes 101 and 103 which include barbecues for indoor use only and exclude barbecues for outdoor use. We have now an amendment which is intended to cover appliances for outdoor use and which excludes barbecues for outdoor use. Does it means that for barbecues intended for in and outdoor use we shall use IEC 60335-2-9 and IEC 60335-2-78 ?? Clarification is needed . | You are correct - this matter was discussed at great length during the development of am 1 to part 2-9. It was agreed that it was completely covered by 4th paragraph of the introduction. | None | 61(Delft/Chairman)2a | France |