| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-9 60335-2-78 |  | Particular requirements for toasters, grills, roasters and similar appliances And on document IEC 60335-2-78:1995, 1st edition: Particular requirements for outdoor barbecues: As far as the German National Committee is understanding the content of IEC 60335-2-9 all appliances described in the scope of the standard are only for indoor use. In our opinion IEC 60335-2-78 is particularly designed only for barbecues for outdoor use. Attention is drawn to the fact that some appliances which are within the scope of IEC 60335-2-9 like rotary grills, radiant grills, raclette grills, portable ovens, waffle irons, etc., are often used outdoor. This misuse of the above mentioned appliances is not intended in the standard. Their design is often not in accordance with the basic requirements for an outdoor use. IEC 60335-2-78 which is only for outdoor barbecues applicable also does not cover such appliances. We are informed test houses have for the time being some problems to approve appliances for outdoor use depending on part -2-9. Because there is a lack in the standard they make use of some clauses of part -2-78. 2 61/1567/Q Therefore we are of the opinion that the scope of IEC 60335-2-9 should be expanded for appliances for outdoor use and necessary requirements should be added in the relevant clauses. As an alternative solution IEC 60335-2-78 could be expanded for the aforementioned appliances with some specific modifications. If TC 61 is also of the opinion that an improvement of one of the relevant part 2s of the standard is advisable the German National Committee is prepared to provide a detailed proposal in due time. In that case we would like to know which part 2 has to be modified. | In response to the question of interpretation, Part 2-78 is the applicable Part 2 for portable outdoor cooking appliances as the standard contains specific requirements that would be applicable to portable cooking appliances for outdoor use. | None | 61/1673/RM | Germany |
| 60335-2-9; 0335-2-78 | Scope | The amendment 1 of IEC 60335-2-9 introduces the outdoor appliance requirements. This amendment should normally cover all outdoor appliances and especially barbecue for which all requirements of IEC 60335-2-78 have been rewrited in amendment 1. My trouble is in notes 101 and 103 which include barbecues for indoor use only and exclude barbecues for outdoor use. We have now an amendment which is intended to cover appliances for outdoor use and which excludes barbecues for outdoor use. Does it means that for barbecues intended for in and outdoor use we shall use IEC 60335-2-9 and IEC 60335-2-78 ?? Clarification is needed . | You are correct - this matter was discussed at great length during the development of am 1 to part 2-9. It was agreed that it was completely covered by 4th paragraph of the introduction. | None | 61(Delft/Chairman)2a | France |
| 60335-2-9 | 21.101 | I'd like to ask a question regarding IEC60335-2-9:2008, 21.101 Surfaces of hotplates of glass-ceramic or similar material shall withstand the stresses liable to occur in normal use. Compliance is checked by the following test. The surface of the hotplate shall not be broken and the appliance shall withstand the electric strength test of 16.3. Here I’d like to ask how to understand the term “broken”, it just means that the hotplate is broken apart, or in addition it includes that there is a crack on the hotplate. | Broken also means “that there is a crack on the hotplate”. China 1: Confirmed. Add “This does not mean a surface imperfection” to the response. | None | 61/4080ea/RM | China |
| 60335-2-9 | 22.108 | 1) IEC 60335-2-9, clause 22.108: Taking into account the definition of a barbeque (2.111), should the requirement apply only to radiant grills (2.106) rather than all grills. | 1) IEC 60335-2-9, Subclause 22.108. The response was that the requirement applies to all grills. | None | 61/2093/RM | Australia |
| 60335-2-9 | Subclause 22.108 | Taking into account the definition of a barbeque (2.111), should the requirement apply only to radiant grills (2.106) rather than all grills. | The requirement applies to all grills. | None | 61/2093/RM |  |
| 60335-2-9 | 22.111 | Question: According 22.111 heating elements in breadmakers shall be located so that they are not exposed to dough that may rise over the edge of the dough container during normal use of the appliance. The standard in 6ª ed seems to be not clear enough because it is not specified if the recipe in the instructions may be modified by increment only some ingredients or if is required to increment all of them (whole recipe quantity). When there is a doubt that the heating element is located that overflow may affect it, if the second of the above possibility is agreed for testing, the test specification can not get an overflow because the recipe of dough and not by constructional requirements. Rationale: (See question) Proposal (if any): In breadmakers, if due to the location of the heating elements after an initial inspection there are doubts concerning the possible exposure to overflowing dough, the ingredients shall be increased keeping the correct proportions of the initial recipe. In the case where it is not possible to cause overflowing due to the characteristics of the appliance, then the dough is specially prepared separately in a sufficient amount and then put into the breadmaker vessel in a quantity sufficient to cause an overflow. EXPLANATION: CLC/TC61 confirmed in Sofia on November 2009 and issued the I.S. 2009/02. This decision is similar to OSM/HA 452 | The decision sheet is not necessary. The note in the standard allows increase in any of the ingredients in incremental amounts. | None | 61/4080ea/RM | CTL Liason |
| 60335-2-9 | 22.113 | Clarification is needed regarding the intent in 22.113 of IEC 60335-2-9. In this clause it is stated that: Toasters having an ejector mechanism shall be constructed so that they switch off automatically after the normal toasting time even if the ejector mechanism is blocked. Compliance is checked by the following test. The toaster is supplied at rated voltage and the ejector mechanism is prevented from releasing. On the completion of the normal toasting time, heating elements shall be automatically disconnected from the supply by at least an all-pole disconnection, micro disconnection. However, a single pole, micro-disconnection is allowed, provided heating elements are not accessible to the test probe 12 of IEC 61032 Question: Is the intent only to cover the case where the ejector mechanism is blocked by the bread being toasted or it is also intended to cover the case where the operating level is held down by an external source such as a cupboard door or similar. | The intent of the discussion that lead to this new clause means that both cases are to be considered in turn but not simultaneously. Australia 1: Confirmed | None | 61/3927e/RM | Australia |
| 60335-2-9 | 19.4 & 19.5 | According to our understanding, a sandwich maker is a contact grill by definition and since contact grills now are included in 19.1 by Am 1, they shall be subjected to the tests of 19.4 and 19.5. This question is forwarded since we have been informed of different interpretations. We would like to have a confirmation of that our understanding is correct. | The opinion of the SENC is agreed. | None | 61/2898/RM | Sweden |
| 60335-2-9 | 19.101; 19.102; 19.103; 22.105; 22.108; 22.109; 22.110; 3.1.9.109; 3.105; 3.106; 30.1; 31 | During preparation of the French version of IEC 60335-2-9 the FRNC detected a number of errors and possible inconsistencies in the English version of the standard. The errors and inconsistencies detected are as follows : - Differences in some countries: 22.107 of Ed 4 has become 19.101 in Ed 5 19.101 “ 19.102 “ 19.102 “ 19.103 “ 22.106 “ 22.105 “ 22.110 “ 22.108 “ 22.111 “ 22.109 “ 22.112 “ 22.110 “ - 3.1.9.109 is the same text as in 3.1.9.106 - 3.105 and 3.106: “visible glowing” shall read “visibly glowing” - 30.1 The reference of the referred test shall be 19.102 (19.101 of Ed 4 has become 19.102 in Ed 5). - Clause 31 (in amendment A1) It seems to be the only Part 2 where an angle of 80°- 85° is specified for drawing the pin on the coating surface. Is that necessary? Do we have to modify the other publications (2-40, 2-59, 2-78, 2-82, 2-83, 2-84, 2-86, 2-87, 2-88, 2-95, 2-97 and 2-103, and may be some others)? | - accept the changes for the "Differences in some countries" (correct the placement of 19.101 ISCN) - delete reference to raclette appliances from 3.1.9.106 - change visible to visibly (bold text) in 3.105 and 3.106 - change 19.101 to 19.102 in 30.1 (SEE ALSO SWEDEN 1 WHERE AN ALTERNATIVE ACTION WAS PROPOSED) - clause 31 of Amdt 1: Take no action at this time. The above could be handled via a corrigendum. | None | 61(Kuala Lumpur/Chairman)2a | France |
| 60335-2-6; 60335-2-9; 60335-2-15 | 19.102; 19.104; 15.102 | 2a) Question: In clauses 19.102 of IEC 60335-2-6:2002 and 19.104 of IEC 60335-2-9:2002), it is required to short-circuit the thermal control for the test. Note 5 of Sub-clause 19.1 of IEC60335-1:2006 explains that instead of short-circuiting, controls may be rendered inoperative. In induction hobs, the thermal control of hob elements is normally a NTC controlled by the micro-controller. Short-circuit or open circuit of the NTC is easily detected by the micro-controller, but fixing the NTC in an intermediate value may not be detected. IEC60730-1:2007, Annex J has requirements for thermistors. There is a test for the drift of the R/T characteristic when the control is classified as Type 2. Should the NTC be fixed at a specific value in order to render the thermal control inoperative? What is the required condition to be checked by clause 19.102? Decision: The NTC can be fixed to a specific value. This value must be in the middle of the operating range of the NTC, in order to avoid hysteresis problems that may happen towards the end of the range. Explanatory Notes: For induction hobs that use other types of electronic controls to control the temperature in clause 11, the intent is to set up the electronics so that the appliance continues to operate without the clause 11 controls operating. Inspection of the circuit diagram may show that it is also possible, for example, to bypass the micro-controller to achieve the intent. Decision was confirmed by CLC/TC61 (June 2009). It is similar to OSM/HA 418 modified. 2b) Question: Could the following procedure be considered to improve the consistency of the method used for the electric strength test following the spillage test of sub-clause 15.102? The connector system is placed on a metal foil sheet on the bench surface. After the prescribed amount of water is deposited through the 8mm tube onto the connector surface, the electrical strength test shall be applied as soon as practical. Immediately following the deposition of the water, metal foil is smoothed over the accessible surface of connector, avoiding contact with any earth connection by means of a hole or gap in the foil. The application of the metal foil is carried out in such a manner, as to minimize any disturbance to water on the surface of the connector. The following test voltages are then applied as follows: a) 2500V a.c. between live parts and the metal foil over the surface of the connector b) 2500V a.c.between live parts and the metal foil surface under the appliance base c) 1250V a.c. Between live parts and the earth connector Decision: The above test procedure including the test voltages is confirmed. Explanatory Notes: This decision is similar to OSM/HA 429. | 2a: Noted 2b: Noted | None | 61/4042e/RM | CTL Liason |
| 60335-2-9; 60335-2-6 | 19.102; 19.104 | In 19.102 of IEC 60335-2-6:2002(Also in 19.104 of IEC 60335-2-9:2002), it is required to short-circuit the thermal control for the test. Note 5 of Sub-clause 19.1 of IEC60335-1:2006 explains that instead of short-circuiting, controls may be rendered inoperative. In induction hobs, the thermal control of hob elements is normally a NTC controlled by the micro-controller. Short-circuit or open circuit of the NTC is easily detected by the micro-controller, but fixing the NTC in an intermediate value may not be detected. IEC60730-1:2007, Annex J has the requirements for thermistors. There is a test for the drift of the R/T characteristic when the control is classified as Type 2. Should the NTC be fixed at a specific value so as to render the thermal control inoperative, which is the condition required to be checked by 19.102? | Decision: The NTC can be fixed to a specific value. This value must be in the middle of the operating range of the NTC, so as to avoid hysteresis problems that may happen towards the end of the range. Explanatory Notes: The intent is to set up the electronics so that the appliance continues to operate without the clause 11 controls operating. Inspection of the circuit diagram may show that it is also possible to bypass the micro-controller to achieve the intent. The interpretation of NTC may be extended to other sensors or controls that are working in clause 11 even these are not having a direct action of controlled parts. Decision was confirmed by CLC/TC61 (June 2009). It is similar to OSM/HA 418 modified. CTL/PDSH725 (revised): Replace the explanatory note with the following: “For induction hobs that use other types of electronic controls to control the temperature in clause 11, the intent is to set up the electronics so that the appliance continues to operate without the clause 11 controls operating. Inspection of the circuit diagram may show that it is also possible, for example, to bypass the micro-controller to achieve the intent.” | None | 61/3927eR/RM | CTL Liason |
| 60335-2-9 | 30.1 | In IEC 60335-2-9, According to clause 30.1, 19.101 is not taken into account, which is the same text as in the 1995 edition. The problem is that 19.101 is not the same in those two editions. The reference in the latest edition shall in my opinion be 19.102, because a new 19.101 has been introduced (moved from 22.107) Proposal In 30.1 change 19.101 to 19.102 or add 19.102 and keep 19.101. | I think we need to add 19.102 to the exclusion in 30.1 – this could be done via a corrigendum | None | 61(Kuala Lumpur/Chairman)2a | Sweden |
| 60335-2-9 | Exhibit G | 61(New Delhi/1060)6 and 61/1060/Q | DECISION: The standard requires a minimum of micro disconnection, which according to IEC 1058 requires 390 volts dielectric strength across the contract gap for 110 volt appliances and 750 volts across the contact gap for 230 volt appliances. | None | 61/1220/RM | 1060 |