| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-103 | 7.12.1 | In the standard, paragraph 7.12.1 it says: "except for horizontally moving pedestrian doors, ensure that entrapment between the driven part and the surrounding fixed parts due to the opening movement of the driven part is avoided". We would like a clarification on that point because there are 2 situations that can occur while the gate opens (For a typical sliding gate with rail on the ground): - A hit by the outside frame of the gate (the last vertical part of it and with no relation to the inner side or design of the gate) in the opening area (the area where it opens to). This can be prevented by fencing. - 2 possible hits of the same nature: - A gate is built with an outside frame and inside vertical bars, let's say 100 mm one from the other, An arm inserted horizontally threw the bars, while the person is standing in the opening gap. The person goes with the gate until the arm reaches a fixed part. Are the bars considered as a driven part or only the out side frame? b. A person drops something on the ground and it rolls over the ground. The person is inside the perimeter and the item he droped he wants is on the outside part of the perimeter. Between them - a gate. The person is lying on the ground and streaching his arm forward threw the gate, below the buttom horizontal bar of the gate frame and above the rail. Suddenly the gate opens and the wheel touches the arm. Are the bars considered as a driven part or only the out side frame? | I will refer your question to our MT25 that looks after this standard, however my initial response to your questions are: - Agreed - a) and b) The bars are also considered as a driven part. However, in the situations you describe in items 2(a) and (b) the entrapment is the result of a deliberate action, it is not as a result of an inadvertent action. I am not sure that the intent of the installation instruction you refer to takes into account entrapment resulting from a deliberate action since there may be many more scenarios that would need to be to covered if it did. It can be stated that injury from deliberate action entrapment is lessened due to the limit on the entrapment forces detailed in 20.107, this covers both contact and non contact entrapment protection in subsequent clauses and where it is not possible to prevent injury due to contacting the driven part, non contact sensing would be required. Israel 1: Confirmed | None | 61/4042e/RM | Israel |
| 60335-2-103 | 22.106; 22.107; 22.108; | a. Under the definition of the DRIVE (3.101) it is understood that, in the case of a sliding gate for example, a combination of (motor+gear+limit switches), or, a combination of the same with a control panel but without push buttons and without a rack is not considered a DRIVE, so someone can buy this combination for self installation and say that he will install later a button set that works like a biased off switch or that he has already safety devices at home. This puts the seller in a position in which he is subjected to non-compliance to the standard. On the other hand, (22.106) says that a DRIVE must be supplied with all the components..., again, depending on the mode of operation. Is a combinations of motor+gear+limit switches is enougth to be considered as a DRIVE? If no, what is the minimum combination for a DRIVE? b. 20.108 is talking about testing the ENTRAPMENT PROTECTION SYSTEM. If, for example, the ENTRAPMENT PROTECTION SYSTEM is a safety edge that has limit switch inside and that gives a wireless signal to the control by means of a transmitter and a receiver, is it considered as a legitimate test to open the circuit of the receiver so that the command will not reach the control panel (this receiver is wired while installation and not a build in part of the control panel)? If your answer is yes, so, all dry contact accessories like safety edges and photocells can not be considered as ENTRAPMENT PROTECTION SYSTEMS becasue they can be overrided. If your answer is yes so only operators with built in sensing devices and/or 8200 Ohms safety edges can be considered ENTRAPMENT PROTECTION SYSTEM. c. (22.107) allows to override the ENTRAPMENT PROTECTION SYSTEM. is it not supposed to be that when the ENTRAPMENT PROTECTION SYSTEM is overrided only BIASED off switch can activate it? Now, (22.107) can give any installer the opportunity to install an override switch and save all ENTRAPMENT PROTECTION SYSTEM accessories. | a) You need to take into account the note to 3.101 that states: NOTE Examples of components are gears, controls, brakes, trolleys and entrapment protection systems. b) I do not understand your reasoning – the standard is very clear on this point. Please read the test specification c) I do not understand your reasoning – the standard is very clear on this point. Please read the text of 22.107 carefully it does not exempt an entrapment protection system from being supplied and fitted. Israel 1: Confirmed | None | 61/4285/RM | Israel |
| 60335-2-103 | Note 104 | According to note 104 this standard doesn't apply to drives– "intended exclusively to be used by trained persons in commercial and industrial premises;" The problem is that there is not definition of "user" in the standards. The words "be used by trained persons" is intended to the operators of the gates or to the persons cross through the gates/doors? I'll appreciate a clarification regarding the above | The words "be used by trained persons" is intended to apply to the operators of the gates/doors. It does not apply to the persons crossing through the gates/doors who cause the gates/doors to open/close automatically. Israel 1: Confirmed | None | 61/4212e/RM | Israel |