| Document |  | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60335-2-55 |  | 3.1.9 | As stated in 5th paragraph of clause 3.1.9 of IEC50335-2-55, "Heaters are operated in a sufficient quantity of water to maintain the water temperature between 20ºC and 25ºC without the thermostat cycling.". In this case, the thermostat always cycles independently of its regulation and of the water temperature which means that the above test conditions will not be possible to obtain. Do you consider that this heater don’t comply the standard because it doesn’t reach the standard operating conditions? Or Do you perform the test (clause 11) in the most unfavourable and apply the standard accordingly? Rationale: see question ### Proposal (if any): If there is any kind of temperature sensing for cycling then it should be kept without cycling in clause 11 and sub-clause 19.4 may be covered in this construction. But if the cycling does not rely on the temperature of the water (energy regulator or similar) the cycling means are allowed to operate in clause 11 and shall be short-circuited in sub-clause 19.4. | 17/07 - Agreed – the standard should be amended to reflect this. TC 61 to circulate a DC for Part 2-55. | None | 61/3355/RM | CTL Inquiry – 17/07 |
| 60335-1; 60335-2-55; 60335-2-109 | Clause 15; Clause 22.6; Clause 22.101; Clause 22.103; Clause 27.1 |  | A query on UVC pond appliances We have some products that were traditionally made to -2-55 because they were for use with garden ponds but since they contain UVC sources for algae control they are now being considered as coming under -2-109 which has strict requirements for UVC access which previously were dealt with by instruction to the user and not constructional requirements. A Test house has also made an interpretation relating to Clause 15 trying to find the most unfavourable situation in 22.103 and then doing the Clause 15 tests. With -2-55 this was not a problem. My comments (Stuart MacConnacher) to the questions put to me are in red. EN 60335 – 1 Clause 15  If a product cannot be disassembled without the aid of a tool but for user maintenance you need to open up the product and then remove a detachable part such as an o-ring attached for water sealing purposes I believe that these detachable parts inside the appliance case do not need to be removed. If however the outer case is detachable without the use of a tool you keep removing detachable parts (o-rings and retaining parts) until you come to a part that needs a tool. You then go no further even if the user instructions states that tool fixed part has to be removed to change the lamp?. This is prior to the tests of 15.1.1, 15.1.2 & 16.3. I believe that the situation for clause 15 is as stated above. (CONFIRMED) Clause 22.6  For a class 1 construction electrical insulation shall not be affected if a hose ruptures or a seal leaks?  Does it mean if you use o rings for water exclusion inside the product because water flows through the product do you consider that one will fail so you need to provide 2 ?. There is no logic to needing 2 (HAVE MORE THAN ONE SEAL DOES NOT EXEMPT THE PRODUCT FROM 22.6) EN 60335-2-109 Clause 22.101  The hole is drilled such that water entering does not fall directly onto live parts – not the hole is drilled and if water can directly fall on live parts the construction does not comply? It is the accumulating water that must contact earthed metal first? YES that is what the standard says.(CONFIRMED) Clause 22.103 If the replacement of the UV-C emitter is allowed by the user, the appliance shall be constructed so that – the replacement of the UV-C emitter is easily possible; – the UV-C emitter is correctly sealed after replacement; – if screws, seals or components are omitted or incorrectly positioned or fastened, the appliance is rendered inoperable or manifestly incomplete. Compliance is checked by inspection and, for parts that are likely to be removed for the UV-C emitter replacement, by disassembling and assembling these parts ten times, followed by the tests of 15.1.2 and Clause 32.  Rendered inoperable or manifestly incomplete are two different states? Yes two states (CONFIRMED)  Inoperable means the appliance cannot be energised? YES (CONFIRMED)  Manifestly incomplete means that it is obvious to the user that something is missing, i.e the part cannot be properly tightened up and the assembly is obviously loose? Yes (CONFIRMED)  This status is visually inspected? Yes (CONFIRMED) For parts that are likely to be removed for the UVC emitter replacement, then the whole assembly/subassembly is assembled and disassembled 10 times (not omitting any parts) – then is followed by the clause of 15.1.2 and clause 32? This tests the durability of the construction rather than the possibility that someone could forget to reassemble a part – which is covered by the inspection above? YES (CONFIRMED)  The tests of clause 15.1.2 and 32 are not carried out with seals or screws omitted. YES (CONFIRMED) Clause 27.1 Addition  Is there a reason why reinforced insulation is not acceptable in the case of a class 1 pond UVC device where it is in -2-55? This has been taken from the water heater standard and I do not see why it is now applicable to pond appliances covered by this standard. (CONFIRMED) | United Kingdom 1: Clause 15 – confirmed Clause 22.6 – Having more than one seal does not exempt the product from 22.6. Clause 22.101 – confirmed Clause 22.103 – confirmed (correct reference to 5.1.2 to 15.1.2 in question) Clause 27.1 – confirmed | None | 61/4285/RM | United Kingdom |