| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-21 | 19.1 | The first dash item in 19.1 of IEC 60335-2-21 that states: – an outer enclosure of metal; NOTE 101 Non-metallic covers may be used for the supply terminals and controls. The inquiry from Australia asked if the same level of safety would be achieved if the water container was of metal and the outer enclosure was of non-metallic material. | I cannot remember the reason for the wording in the first dash item except that it had been in the standard for many years. However, I am of the opinion that a metallic water container would provide the same level of safety taking into account the other three dash items in 19.1. | None | 61/3827/RM | Australia |
| 60335-2-21 | 22.103 | Our lab received 3-different sample (30 Liter, 50 Liter, and 80 Liter) with Rated pressure 0.75MPa for all types see the attached photo, As its clear from the photo the declared rated Pressure is 0.75 MPa but the manufacturer also declared the working pressures for the Pressure-relief devices. According to 22.103 of IEC 60335-2-21/2002 which state that: “Pressure-relief devices of closed water heaters shall prevent the pressure in the container from exceeding the rated pressure by more than 0,1 MPa. And when we did the test on each one of the sample, the Pressure-relief device for each one start to work at ( 0.86 MPa, 0.88 MPa, 0.93 MPa) in consequently. According to the standard the pressure-relief device shall prevent the pressure to exceed 0.85 MPa So we need your opinion in each case of the three samples. | Clause 22.103 requires that during the test the pressure in the water container must not exceed 0.85 MPa (for a rated pressure of 0.75 MPa). IEC 60335-2-21 make no allowance for uncertainty of measurement since this is not the function of the standard – the standard states „must not exceed” | None | 61/4080ea/RM | Jordan |
| 60335-2-21 | 22.106 | Question: In sub-clause 22.106 of standard IEC 603325-2-21:2002 says that, Closed water heater shall incorporate a thermal cut-out providing all-pole disconnection and which operates independently from the thermostat. Can the thermal cut-out and thermostat control the same contactor (when contactors is used) or are two contactors needed (one controlled by the thermal cut-out and one controlled by the thermostat)?\ Rationale: see question ### Proposal (if any): 2 contactors at least are necessary or one contactor with 2 independent sets of main contacts. | 15/07 - Two contactors or relays must be used because the contactor or relay may not operate if one set of independent contacts welds closed. See also 19.14 in Part 1. | None | 61/3355/RM | CTL Inquiry – 15/07 |
| 60335-2-21 | 24.2.102 | Question: In the IEC 60335-2-21:2002 / A1:2004: Last paragraph said: The thermal cut-out shall operate before its temperature exceeds 110 °C. The water temperature shall not exceed 20 K of the maximum permitted operating temperature of the thermal cut-out. Considering the above statement, which is the maximum temperature of the water to be measured: a) 130 K (110 K + 20 K) or b) maximum measured operating temperature of the thermal cut-out during test +20 K? Rationale: see question | 30/07 - The answer is (a), modified to read degrees C instead of K. According to the text in 24.102.2, (i) the thermal cut-out shall operate before its temperature exceeds 110 °C; and (ii) the water temperature shall not exceed 20 K + 110 °C = 130 °C (since the maximum permitted operating temperature of the thermal cut-out is 110 °C according to the test specification in 24.102). | None | 61/3355/RM | CTL Inquiry – 30-07 |
| 60335-2-21 | 24.7 | We are manufacturer of Electrical Water Heaters. They are certified according to EN 60335-2-21 Our question is about the flexible hoses used to connect the appliance to the water mains. Does EIC 61770 apply on that hoses or not.(Discussed under section 24) | Your inquiry relating to the IEC regarding EN 61770 and EN 60335-2-21 has been forwarded to me to answer. My response is as follows: I cannot advise on the requirements in EN 61770 or EN 60335-2-21 since these are European standards they are not international standards. However in IEC 60335-1 Ed 5 the following is applied: 24.7 Detachable hose-sets for the connection of appliances to the water mains shall comply with IEC 61770. They shall be supplied with the appliance. Appliances intended to be permanently connected to the water mains shall not be connected by a detachable hose-set. NOTE Examples of appliances that are considered not intended to be permanently connected to the water mains are household appliances such as dishwashers, washing machines, tumble dryers, refrigerators, icemakers, steam ovens and the like. I do not know your design of water heater, but normally storage water heaters within the scope of IEC 60335-2-21 are intended to be permanently connected to the water mains and so must not be connected by detachable hose sets consequently the hose sets used need not be supplied with the appliance and are not required by the IEC 60335-1 to comply with IEC 61770. Turkey 1: Confirmed. See also note 2 to Clause 1 of IEC 61770. | None | 61/4212e/RM | Turkey |
| 60335-2-21 | 24.102.2 | a) There is reference to diagram 102a for the position of the thermostats for metallic containers. In the next paragraph there is allowance for measuring the water temperature for vertical containers with a probe in the water. Why not also for horizontal containers? b) There is reference to diagram 102b for no-metallic containers. This diagram only shows vertically orientated containers - what about horizontally orientated non-metallic containers where the probes cannot be attached to the outside of the container due to low heat transfer. - Figure 101 is only referenced in "Notes" and is thus not "Normative". As such it would be we suggest to put a comment in Fig 101 to say that "although a vertical system is shown, this also applies to horizontal systems" - The use of the wording "Low -pressure" water heater in clause 3.106 , Referring to Figure 101e, the vent pipe in many RSA installations is up to 15m high and such systems are not considered "Low pressure" (they have been part of Cistern fed systems where it is not feasible to feed the overflow back into the cistern, but is higher than the cistern). There does not seem to be a max pressure rating for such systems to be able to classify them as "Low-pressure". | 1. The third paragraph of 24.102.2 applies to both vertical and horizontal containers. Figure 102b only shows how to position the thermocouples for a vertical container. 2. See response to 1. Modify the text to delete “as shown in one of the diagrams of Figure 102b” and delete” vertically oriented.” Also delete Figure 102b. 3. The ZANC is requested to submit a proposal for change, if necessary. Note that the diagrams in Figure 101 are only examples. There are many other types of water heaters. 4. Noted. | None | 61/4080ea/RM | ZANC |