| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-31 | 7.12 | I have a question about IEC60335-2-31. As described in section 7.12, „when the range hood is located above a gas appliance, the minimum distance between the supporting surface for the cooking vessels on the hob and lowest part of the range hood shall be at least 65cm‟. Here, I am wondering why the distance of 65cm is adopted, I mean is there a specific experiment indicate the 65cm is the lowest safety distance, if there is, could you please send me a standard experiment procedure which proved the 65cm is necessary. | Generally the temperatures above a gas hob are higher than those above an electric hob. However due to the different calorific values of the gas supplied to consumers in different countries around the world it was not possible to introduce into the standard, a standardized gas hob as was done for electric hobs. Consequently, as a pragmatic solution to the problem of different gas calorific values the distance of 65 cm (as modified by A2 to IEC 60335-2-31) was selected in conjunction with the enhanced ball pressure test in the Addition to 30.1. To date this solution has proved satisfactory. China 1: Confirmed | None | 61/4042e/RM | China |
| 60335-2-31 | 7.12; 7.12.1 | 1. In IEC 60335-2-31 Clause 7.12 it reads - there shall be adequate ventilation of the room when the range hood is used at the same time as appliances burning gas or other fuels (not applicable to appliances that only discharge the air back into the room) The adequate ventilation should be necessary for the appliances. Why it's not applicable to the appliances that only discharge the air back into the room ? 2. In IEC 60335-2-31 Clause 7.12.1 it reads - the air must not be discharge into a flue that is used for exhaustin fumes from appliances burning gas or other fuels... Whether the "air" in that clause mean "exhausted air"? | 1. This is an instruction for use and is there to ensure against a negative pressure in the room occurring due to the use of a range hood that is vented to the outside. Negative room pressure can affect the safe operation of gas hobs. Range hoods that discharge air back into the room do not create a negative room pressure. 2. It is confirmed that the "air" in that clause means "exhaust air from the range hood" Thailand 1 – Confirmed | None | 61/3366/RM | Thailand |
| 60335-2-31 | 7.12.1 | In the international standard 60335-2-31, the addition part 7.12.1 states … “the minimum distance between the supporting surface for the cooking vessels on the hob and the lowest part of the range hood. When the range hood is located above a gas appliance, this distance shall be at least 65cm. If the instructions for installation for the gas hob specify a greater distance, this has to be taken into account.” We are a manufacturer of extractor hoods for domestic use in Belgium. We are in the final R&D phase with a relatively new type of extractor hood, called the “head free” hood. You may already have seen this type of hood, but in case you haven’t a picture is attached to this mail to make things clear. So my question is… Is there an exception for this kind of hoods in the pipeline? The critical parts of this hood (like motor and electrical components) are located higher than in a standard hood. We suppose the lowest part of this hood can be placed at about 50cm above the hob with a lower risk of fire than a standard hood at 65cm, because all critical parts are then at 80 cm and higher. Do you agree with our opinion, and if you do, which action do we need to take to have the standard changed? | A proposal to alter IEC 60335-2-31 clause 7.12.1, is at present being progressed through the standards amendment process. It proposed to add the following to the second dashed item: The distance of 65 cm can be reduced for - non-combustible parts of range hoods, and - parts operating at a voltage not exceeding safety extra low voltage, provided these parts do not give access to live parts if deformed. Belgium 1 – Confirmed | 61(Tokyo/Chairman)2a – Belgium 1 | 61/3655/RM | Belgium |
| 60335-2-31 | 7.12.1; 30.101 | In the last few months, we are receiving various inquiries about the acceptability of rangehoods intended to be installed at less than 65 cm above the gas range. All the products submitted to us have no live parts installed at less than 65 cm above the gas range. According to the following OSM decision: http://www.iecee.org/ctl/osm_sheets/0306OSM-HA.pdf The construction is acceptable for EN 60335-2-31. I would like to know if, in the past IEC/TC 61 meetings, was discussed a proposal of an Amendment to IEC 60335-2-31:2002, clauses 7.12.1 and 30.101, regarding the distance of installation above the gas range and a possible needle flame of flammable materials installed below the specified height. | The OSM decision applies to the 1997 edition of EN 60335-2-31it is not correct when applied to 2002 edition of IEC 60335-2-31. A proposal has never been submitted to TC 61 to amend clauses 7.12.1 and 30.101, regarding the distance of installation above the gas range and a possible needle flame of flammable materials installed below the specified height. | None | 61(London/Chairman)2a | Italy |
| 60335-2-31 | 20.2 | 61(New Delhi/Australia)12 | The requirement is necessary to give protection against injury from moving parts. | None | 61/1220/RM | Australia |
| 60335-2-31 | 22.102 | 3) In addition, as mentioned in Clause 22.102 of IEC 60335-2-31, “Compliance is checked by inspection”. We would like to ask what kind of inspection should be conducted to check the compliance of this clause and what are the criteria to fulfill this clause. Also, the word ‘cleaned’ should be carried out by user or maintenance agent ? | 3) In this clause reference to cleaning means “cleaning by the user”. The inspection means applying an engineering judgement as to whether the appliance is constructed so that it can be cleaned by the user Hong Kong 2 – Confirmed | None | 61/3355/RM | Hong Kong |
| 60335-2-31 | 30.101 | In sub-clause 30.101 of IEC 60335-2-31:2002 it is stated that : “ Internal air-ducts and parts within them, such as fans, are subjected to the needle-flame test of Annex E”. We have a range hood that has a metal grease filter and behind this filter there is a carbon filter directly in the air stream. This carbon filter does not have a function as a grease filter as the material in the metal filter is the grease filter. We have considered this carbon filter as a part within the internal air-duct and therefor it has to be comply with the needle-flame test. Between the test houses there is a different interpretation of this clause. There are test houses that do not apply this tests and others do apply. The Netherlands National Committee is of the opinion that filters within the internal air-duct stream and which are not used as a grease filter have to comply with the needle-flame test of Annex E. We would like to know the opinion of IEC TC 61. | The opinion of the NLNC is agreed. | None | 61/2898/RM | Netherlands |

- Attachment 61(Tokyo/Chairman)2a – Belgium 1: