| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-35 | 7.1 | We were asked to approve or otherwise the importation into Mauritius of the bare element type water heater. A search for specifications for this type of equipment inevitably led me to the IEC 335-2-35 which was of great assistance in assessing the apparatus. I should, however, like to obtain clarification regarding the value of 1 300 ohm.cm quoted in section 7, paragraph 7.1, and would be most grateful if you could advise how this figure was arrived at. | I was unable to determine the origin of the 1 300 ohm.cm value in Clause 7 of the standard referred to. However, based on information I received, it appears that this value was based on a survey carried out within Europe, perhaps during the development of the CEE standard, on these appliances. In general, it was found that the domestic water supplies in Europe had a resistivity exceeding 1 300 ohm.cm. Therefore, testing the appliance having a resistivity not exceeding 1 300 ohm.cm would ensure safety in an installation. The advice is that any country adopting this standard should ensure that the 1 300 ohm.cm limit is relevant for the local water supply. If not, then a deviation should be included to ensure that their water supply is covered. | None | 61(Christchurch/Secretariat)2 | Mr. Guy Martin, Main Committee, Mauritius Standards Bureau, Mauritius |
| 60335-2-35 | 19.11.4 | Now in Thailand is already adopt the new version of IEC60335-1:2001 + Amd1:2004 to national standard and also for IEC60335-2-35:2002 I have in doubt about cl.19.11.4 for instantaneous water heater testing according to this paragraph, 19.11 "Appliances having a switch with an off position obtained by electronic disconnection, or a switch that can place the appliance in a stand-by mode, are subjected to the tests of 19.11.4" Do we think the instantaneous water heater is in stand-by mode or electronic disconnection or not? (Please see attached picture) If No, what is the reason? If YES, please indicated of function "switch with an off position obtained by electronic disconnection or a switch that can place the appliance in a stand-by mode" Then it shall be test according to cl.19.11.4. But some of EMP test is limited to 16 A. If rated current of water heater is over 16 A, How do we applied EMP test for this appliance? | 1. Because the appliance is an open outlet instantaneous water heater and because the appliance may not always be turned off by the user via the push switch it is concluded that the appliance has an electronic off via the triac and hence must be subjected to the 19.11.4 2. The test can be done with the heater load disconnected or with the heater load supplied from a separate source. | 60335-2-35:2002 | 61/2971/RM | Thailand |
| 60335-2-35 | Page 21 | Clarification was requested regarding the value of 0,25 mA as stipulated in the first line of page 21 of IEC 335-2-35. | This value was selected to align with that allowed for class II appliances - it being one-half the threshold of perception for 15-100 Hz currents given in IEC 479. | None | 61(Christchurch/Secretariat)2 | Mr. Guy Martin, Main Committee, Mauritius Standards Bureau, Mauritius |
| 60335-2-35 | 22.107; 5.7 | Can you explain to me the back ground of the requirement, "The temperature of the outlet water shall not be higher than 95 °C and shall not exceed the temperature of the inlet water by more than 75 K"? The absolute value of 95 °C I can understand. The situation is as follows? In Clause 22.107 of the EN/IEC 60335-2-35: Water shall not attain an excessive temperature in normal use. Compliance is checked by the following test. The appliance is operated at rated power input. Any regulating valve is fully opened and the water flow is adjusted so that the flow switch or pressure switch is on the verge of operating. The temperature of the outlet water shall not be higher than 95 °C and shall not exceed the temperature of the inlet water by more than 75 K. While in clause 5.7 of the EN/IEC 60335-2-35: Water having a temperature of 15 °C ± 5 °C is used for the tests. Meaning that for the test of clause 22.107 the test can be carried out with inlet water of 10 °C and with inlet water of 20 °C or any value in between. Most protective devices react on absolute values. So we have now a situation that when we: - carry out the test at 10 °C we have a FAIL on a product. - carry out the test at 20 °C we have a PASS on a product I though that the test conditions indicated in the standard should be unambiguous. How should we treat this? | This has been a requirement for many years. If I remember correctly it was agreed at the Dubrovnik meeting of TC 61 in 1991 and again if I remember the discussion correctly because the requirement states “Water shall not attain an excessive temperature in normal use.” it is not expected that a thermal cut-out will operate because that indicates “abnormal use”. So compliance with the 95 oC and the 75 K limits taking into account 5.7 has to be achieved in the design by the flow switch or pressure switch settings along with a rated power input limit value. The situation that applied when this clause was written is as follows: All operator controls are set to most onerous position (clause 5.6) then the test lab externally adjusts the water flow to the appliance such that ‘Any regulating valve is fully opened and the water flow is adjusted so that the flow switch or pressure switch is on the verge of operating’ Rated power input is then applied. With the given water flow and with the controls adjusted as stated the temperature rise of the water should be the same regardless of the inlet water temperature. Since the water flow and input power are fixed and are independent of the inlet water temperature. This is just simple physics. Netherlands 1: Confirmed | 61(Bali/Chairman)02a – Netherlands 1 | 61/4212e/RM | Netherlands |
| 60335-2-35 | 22.109 | An open-outlet water heater with pressure switch, refer to circuit below. It is fitted with a self-resetting pressure relief device, so go to second dash of 22.109, that is say 22.109.1 and 22.109.3 shall be tested. 22.109.1 pass, but for testing as 22.109.3, we have different understanding: 1. All 3 thermostats and 1 pressure switch are short circuited at the same time if pressure switch is as control ( since short circuit more unfavourable) 2. 22.109.3 is not applicable if pressure switch keeps open (heating elements do not work) after water frozen inside water tank, considering note 2 in 22.109. | From my interpretation of the circuit, 1 is correct, since the pressure switch operates in normal use and is therefore considered a control rather than a protective device. China 2: Confirmed | 61(Tel Aviv/Chairman)02a – China 2 | 61/3927e/RM | China |
| 60335-2-35 | 25.3 | Figure 1: The sample is an instantaneous water heater ( according to the standard IEC 60335-2-35 ). The type of the supply connection is subjected to the requirements in cl. 25.3 of the standard. The substance of cl.25.3 is: “A set of terminals allowing the connection of a flexible cord;” and “NOTE 1 In this case it is allowed to connect the supply cord before the appliance is fixed to its support. The appliance may be provided with a flexible cord.” (See Attachment 61(Oslo/Chairman)02a – China 3-1) An addition, the enclosure of the appliance is fixed with special screws. And in the instruction，it is shown that：The type of the supply connection is type Y attachment，and the users should not replace the supply cord on their own. It is subjected to the requirements for type Y attachment in 25.5 of the standard. Figure 2：Internal terminals of the appliances are screw-type terminals, which are used to fix the supply cord and internal conductors (See Attachment 61(Oslo/Chairman)02a – China 3-2) Question 1：Do the requirements of the standard, especially the requirements in cl.26.2 apply to the structure? The substance of cl. 26.2 is: “Appliances having type X attachment, except those having a specially prepared cord, and appliances for connection to fixed wiring shall be provided with terminals in which the connections are made by means of screws, nuts or similar devices, unless the connections are soldered.” “The screws and nuts shall not be used to fix any other component except that they may also clamp internal conductors if these are arranged so that they are unlikely to be displaced when fitting the supply conductors.” The connection of the appliance is a set of terminals allowing the connection of a flexible cord, which is provided with a supply cord. The enclosure of the appliance is fitted with special screws. And there is some explanation about type Y attachment in the instruction for use. Therefore, the type of the supply connection is type Y attachment, not type X attachment. And the appliance is permanently connected to fixed wiring. Question2:, Do the requirements in cl.26.2 apply to the appliance? Question3: Do the requirements in cl .26.2 apply to the appliances shown in Figure 3 and Figure 4? (See Attachment 61(Oslo/Chairman)02a – China 3-3) Question4：If the requirements in cl.26.2 apply to the appliance shown in Figure 1, does the appliance subjected to the requirements in 26.2? And can we consider that：The enclosure of the appliance is fixed with special screws, so the supply cord must be replaced by the professionals who can restore to the original state. Therefore, the internal conductors can not be displaced when fitting the supply conductors? | The answer to all question is clause 26.2 is not applicable, provided the requirements for a Type Y attachment are fulfilled and the appliance is supplied with the supply cord fitted – this will be clarified in the 5th edition of Part 1 see 61/3771/DC and 61/3814/INF both of these are on the Oslo agenda. China 3: Confirmed | 61(Oslo/Chairman)02a – China 3-1; 61(Oslo/Chairman)02a – China 3-2; 61(Oslo/Chairman)02a – China 3-3 | 61/3827/RM | China |

Attachment 60335-2-35:2002:

Attachment 61(Oslo/Chairman)02a – China 3-1:

Attachment 61(Oslo/Chairman)02a – China 3-2:

Attachment 61(Oslo/Chairman)02a – China 3-3:

Attachment 61(Tel Aviv/Chairman)02a – China 2:

Attachment 61(Bali/Chairman)02a – Netherlands 1: