| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-29 | 19.13 | Question: A lot of battery charger control the charging process with help of electronic circuits. Also safety functions (limiting of current, timer, etc.) are controlled by electronic circuits. In addition the cells of corresponding accu packs include a valve for opening if the pressure inside the cell will be to big. This pressure increases for example if the electronic of the charger does not work in correct way and the accu pack will be overloaded. Is it possible to consider this valve like a safety valve for protection against abnormal conditions if the safety valve function is tested separately? Rationale: see question ### Proposal (if any): Yes, but it is necessary to put in the instructions that only these batteries can be used. | 31/07 - Agree in principle but at present Part 2-29 says nothing about battery rupture – this is a battery function not a charger function. The principle was discussed under agenda item 18c in 61/3561/DA in Tokyo | None | 61/3355/RM | CTL Inquiry – 31/07 |
| 60335-2-29 | 22.26 | 61(Nuremberg/New Zealand)20 | In response to the question from New Zealand - “no connection” means no conductive connection. | None | 61/1424/RM | New Zealand |
| 60335-2-29 | 30.2.3.1 & 30.2.3.2 | Question: Is the plastic material fixing the plug of an accu-pack of a hand-held power tool evaluated for insulation material to subject to the glow-wire tests in 30.2.3.1. and 30.2.3.2 even this accu-pack is not included in the tests of IEC 60335-2-29 if the rated output current is less than 20A? In General: Is IEC 60335-2-29 also applicable for a package (battery charger + accu-pack) if the accu-pack is a special one (e.g. for hand-held tools) only chargeable by this batter charger and with safety relevant components (e.g. thermal protector) inside. Rationale: see question ### Proposal (if any): ### 1.- For the figure, it is clear in the standard that sub-clauses 30.2.2 and 30.2.3 applies to any plastic material supporting connection regardless the distance to the point of the connection. 2.- If the battery is special for the appliance in such a way that it is needed for the compliance with the requirements of the standard then, it shall be considered as a part of the appliance and shall comply with all relevant clauses of the appliance standard. | 32/07 - Agreed | 61(Tokyo/CTL Liaison)02d – 32/07: | 61/3355/RM | CTL Inquiry – 32/07 |
| 60335-2-29 | 30.2.3.1; 30.2.3.2 | Question: Is the plastic material fixing the plug of a battery pack of a hand-held power tool evaluated as insulation material and subjected to the glow-wire tests in 30.2.3.1. and 30.2.3.2. Is the battery pack is included in the tests of IEC 60335-2-29 if the rated output current is less than 20A? Is IEC 60335-2-29 also applicable to a a combined battery charger and battery pack, if the battery pack is a special one (e.g. for hand-held tools) only chargeable by the batter charger and with safety relevant components (e.g. thermal protector) inside. Decision: ### 1.- For the figure, it is clear in the standard that sub-clauses 30.2.2 and 30.2.3 applies to any plastic material supporting connection regardless the distance to the point of the connection. ### 2.- If the battery is special for the appliance in such a way that it is needed for compliance with the requirements of the standard then, it shall be considered as a part of the appliance and shall comply with all relevant clauses of the appliance standard. ### Explanatory notes: The same decision (n.414) is present in the OSM/HA decision list. | Q1: Clause 30.2.2 and 30.2.3 of Part 2-29 apply to the battery charger not to the battery pack. Q2: It applies to the complete appliance with the battery in it. | 61(Bangkok/CTL Liason)02d - 4 | 61/4042e/RM | CTL Liason |

Attachment 61(Tokyo/CTL Liaison)02d – 32/07:

Attachment 61(Bangkok/CTL Liason)02d – 4: