| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-53; 60335-2-32; 60335-2-27; 60335-2-17 | Scope | 1. Electric steam sauna in which someone can take a bath by giving off steam to have electric thermal devices installed in such as wooden bathroom, etc. 2. Vibrator which can massage someone by revolutions of electric motors and is household. 3. Appliance giving off ultraviolet rays which can sunburn a human skin and is household. 4. Electric pack which can be heated by electric thermal devices in the inside of pack and is household. | 1. The applicable standard is IEC 60335-2-53 + IEC 60335-2-98 (see London meeting agenda item 18). 2. The applicable standard is IEC 60335-2-32 3. The applicable standard is IEC 60335-2-27 4. The applicable standard is IEC 60335-2-17 | None | 61(Jeju/Chairman)2a | Korea |
| 60335-2-27 | Scope | The scope of IEC 60335-2-27 state: 1 Scope This clause of Part 1 is replaced by the following. This International Standard deals with the safety of electrical appliances incorporating emitters for exposing the skin to ultraviolet or infrared radiation, for household and similar use, their rated voltage being not more than 250 V for single-phase appliances and 480 V for other appliances. Appliances not intended for normal household use but which nevertheless may be a source of danger to the public, such as appliances intended to be used in tanning salons, beauty parlours and similar premises, are also within the scope of this standard. As far as practicable, this standard deals with the common hazards presented by appliances that are encountered by all persons in and around the home. However, in general, it does not take into account - the use of appliances by young children or infirm persons without supervision; - playing with the appliance by young children. NOTE 101 Attention is drawn to the fact that - for appliances intended to be used in vehicles or on board ships or aircraft, additional equirements may be necessary; - in many countries additional requirements are specified by the national health authorities, the national authorities responsible for the protection of labour and similar authorities; - IEC 60598-1 is applicable as far as is reasonable. NOTE 102 This standard does not apply to – appliances for medical purposes; – appliances intended to be used in locations where special conditions prevail, such as the presence of a corrosive or explosive atmosphere (dust, vapour or gas). In Germany are UV lamps on the market, which use UV radiation to harden a gel which is used to extend the natural fingernails. The DENC would like to propose to exclude nail hardener from the scope of IEC 60335-2-27: - The UV is not intended to irradiate the skin. - The irradiation times are very short. - Markings and instructions of part 2-27 do not fit to these kind of appliances. E.g. 7.12.: The instructions for appliances having UV emitters shall include the substance of the following: - a statement that UV appliances are not to be used by - children and young adolescents; - persons who tend to freckle; - persons with a natural red hair colour; - persons having abnormal discoloured patches on the skin; - persons having a large number of moles; - persons suffering from sunburn; - persons not able to tan at all or not able to tan without burning when exposed to the sun; - persons having a history of frequent severe sunburn during childhood; - persons suffering from or previously suffering from skin cancer or predisposed to skin cancer; - persons under a doctors care for diseases that involve photosensitivity; - persons receiving photosensitising medications. - The DENC would like to propose to publish an interpretation of the scope of IEC 60335-2-27 with respect to the exclusion of nail hardener from its scope. | Agreed that nail hardeners are excluded from scope of –2-27. Circulate Q document asking NCs to confirm decision. If confirmed, will published as an interpretation sheet. | None | 61/3777e/RM | Germany |