| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-1 60335-2-34 | Clause 24 | I would like to get your comment for interpretation of IEC standard. There are under texts for requirement regarding components. <IEC60335-1-2000> <IEC60335-2-34-2002> If I have already got the third parties recognition(that is VDE authorization) for motor-protector device, I think that I can execute the requirement of Part1 prior to the requirement of 2-34 for the 24 clause according to above text. In case of this subject, 1. When I execute it, what is this relevant IEC standard described on Part1 ? I believe that they are IEC 60730-1 and 60730-2-4. 2. But, some third party in china insists that this relevant standard includes the requirement of the 24 clause of 2-34. | 1. This is the correct interpretation 2. If you have certification to iec 60730-1 and 60730-2-4, for it to be valid, it must be for at least the number of operations specified in the addition to 24.1.4 of part 2-34. Japan 1 – confirmed | None | 61/3366/RM | Japan |