| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-45 | NOTE 103 | 1.According to the NOTE 103 of IEC 60335-2-45,tools using high-frequency heating is not under the scope of this standard, in this sense, what standard is applicable if a soldering station with soldering iron is high frequency induction heating type instead of traditional ceramic heating type? The way it works is, When the soldering station is turned on, high frequency AC current is made and induced to the coil winding on the tip of soldering iron and by this HF current on the coil, the eddy current is made and heating element is magnetized. The magnetized metal has feature that current pass through the surface of metal and that is how the eddy current heat up the heating element. 2.What is the temperature limit of supply cord sheath during the test of clause 11? Is 35K for cord sheath used as supplementary insulation applicable as a limit? | 1: As far as i am aware there is no standard to cover these appliances for household and similar use. Maybe we should consider adding requirements to part 2-45 to cover them. 2: Yes if the sheath is used as supplementary insulation inside the appliance. It is based on the temperature that pvc cords are tested to. If a better cord is used then it would be safe to allow a higher temperature rise. Korea – confirmed | None | 61/3355/RM | Korea |