| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-11 | Scope | Thank for your answer, but we need to know if the IEC 60335-2-11 is applicable for gas clothes dryers. | IEC 60335-2-11 is only applicable to a gas clothes dryer if it also incorporates electric heating sources. However, IEC 60335-2-102 (currently 61/2157/CDV) is applicable if the gas clothes dryer also contains electrical connections. All other safety aspects are covered by the specific standard covering the gas clothes dryer | None | 61(Beijing/Chairman)2a | Chile |
| 60335-2-11 | 19.102 | Question: Sub-clause 19.102 requires a short circuit between live parts and a short circuit of live parts to other metal parts located below holes in the drum that permit the access of test probe C of IEC 61032. Assuming that in the attached drawing: The live parts can be short circuited by the 1 x 50mm pin; Live parts 2 and 6 can be touched by the test probe; Which live parts are required to be tested? Live Part 1 ? Live Part 2 ? Live Part 3 ? Live Part 4 ? Live Part 5 ? Live Part 6 ? | The drawing shows live parts in a duct and in respect to the drawing, live parts 1, 2, and 3 are tested and 4, 5, and 6 are not. | 60335-2-11, 19.102 | 61/2800A/RM |  |
| 60335-2-11 | 22.101 | 61(Nuremberg/New Zealand)25 Clause 22.101 states: “Heating elements shall be located or guarded so that they cannot be contacted by textile material” The NZNC asks whether lint is regarded as “textile material” | In response to the question from New Zealand - lint is not considered to be textile material. | None | 61/1424/RM | New Zealand |

Attachment 60335-2-11,19.102: