| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-4 | 20.103 | We, the ICONTEC (National Standardization Body)'s Electrical Appliances Committee, are reviewing the IEC 335-2-4 "Safety of household and similar electrical appliances. Part 2: Particular requirements for spin extractors" in order to adopt it as Technical Standard in Colombia. But we have a question about the formula, included on the Note 2. clause 20.103, that states: E = mv2/4, Please, indicate us if this formula is right; if so, which is the reason for it, because we had known that E = mv2/2.. | The energy in a rotating drum of a spin extractor full of clothes is given by E = ½ I  2 where I = moment of inertia  = angular velocity The moment of inertia of a solid cylinder (mass M, radius R) about the cylinder axis is I = ½ M R2 and since  = V/R Then E = ¼ M V2 | None | 61(Kagoshima/Chairman)1 | Columbia |
| 60335-2-4 | 20.104 | In the 5th and 6th paragraphs of 20.104, it is specified as follows: For appliances incorporating two lids where the second lid opens independently of the first lid, the first lid is gradually opened and with an opening greater than 50mm, the motor shall be disconnected from the supply and within 2s the drum speed shall not exceed 20m/s. The second lid is gradually opened and – with an opening of 4 mm to 10 mm, it shall not be possible to touch parts rotating at a speed exceeding 60 rev/min with the test probe 12 of IEC 61032; – with an opening greater than 10 mm, but not more than 12 mm, it shall not be possible to touch parts rotating at a speed exceeding 60 rev/min with a test rod 3 mm in diameter and 120 mm long. In addition, the test probe B of IEC 61032 is applied and shall not come within a distance of 20 mm from the rotating parts; – with an opening greater than 12 mm and within 7s, the drum speed shall not exceed 60 r/min. We would like to clarify how the actual testing is to be applied especially related to the opening of second lid from the opening of the second lid. | The second lid is gradually opened after 2 second from the following condition. - . The first lid is opened and with an opening greater than 50mm, and, - . The motor is disconnected from the supply Namely, the test procedure is as follows. 1. Opening of the first lid greater than 50 mm - (2 second) - Opening of 4 mm to 10 mm for the second lid - Checking to touch parts rotating at a speed exceeding 60 rev/min by test probe 12 2. Opening greater than 50 mm for the first lid - (2 second) - Opening greater than 10 mm, but not more than 12 mm for the second led - Checking not to come within a distance of 20 mm from the rotating parts by test rod and test probe B 3. Opening greater than 50 mm for the first lid - (2 second) - Opening greater than 12 mm for the second lid -(7 second) – Checking the drum speed not exceeding 60 r/min Explanatory note: See attached photos Confirmed | None | 61/3827/RM | CTL |
| 60335-2-4 | 20.104 | In the 5th and 6th paragraphs of Clause 20.104, it is specified as follows: For appliances incorporating two lids where the second lid opens independently of the first lid, the first lid is gradually opened and with an opening greater than 50mm, the motor shall be disconnected from the supply and within 2s, the drum speed shall not exceed 20m/s. The second lid is gradually opened and – with an opening of 4 mm to 10 mm, it shall not be possible to touch parts rotating at a speed exceeding 60 rev/min with the test probe 12 of IEC 61032; – with an opening greater than 10 mm, but not more than 12 mm, it shall not be possible to touch parts rotating at a speed exceeding 60 rev/min with a test rod 3 mm in diameter and 120 mm long. In addition, the test probe B of IEC 61032 is applied and shall not come within a distance of 20 mm from the rotating parts; – with an opening greater than 12 mm and within 7s, the drum speed shall not exceed 60 r/min. We would like to clarify how the actual testing is to be applied especially related to the opening of second lid from the opening of the second lid. | Decision: The second lid is gradually opened 2s after the following conditions: - The first lid is opened with an opening greater than 50mm, and - The motor is disconnected from the supply. The test procedure is as follows. 1. Opening of the first lid greater than 50 mm - (2s) - Opening of 4 mm to 10 mm for the second lid – Check whether test probe 12 touches parts rotating at a speed exceeding 60 rev/min. 2. Opening greater than 50 mm for the first lid - (2s) - Opening greater than 10 mm, but not more than 12 mm for the second lid - Check whether test rod and test probe B come within a distance of 20 mm from the rotating parts. 3. Opening greater than 50 mm for the first lid - (2s) - Opening greater than 12 mm for the second lid -(7s) – Check whether the drum speed exceeds 60 r/min Explanatory Notes: See attached photos Sub-clause 22.33 (3 layers): Noted. The text will be included in edition 5 to part 1. | 61(Tel Aviv/CTL Liason)02d – 2 | 61/3927eR/RM | CTL Liason |
| 60335-2-4 | 20.104 | In Argentina certification of electrical household appliances with national or IEC standards is mandatory to place such products into the local market. This circumstance lead us as National Standardization Body to give advice and interpretation on the correct application of standards to several national actors: manufacturers, laboratories, certification bodies and governmental agencies. Nowadays we are dealing with the interpretation of the most severe working conditions for spin extractors that should be represented by the standard test conditions. Under such approach, our national mirror committee would like to know the reasons by which in paragraph 20.104 of IEC 60335-2-4 the test is carried out with the drum empty. A drum loaded with the maximum mass of dried cloth would not represent the most severe condition from the point of view of the system inertia? | The spin extractor is tested without a clothes load in 20.104 in order to obtain reproducibility and repeatability of the test result - testing with a load leads to a result that is too variable for certification purposes. Experience has shown that spin extractors that comply with the test in the standard do not cause a safety problem in the field. | None | 61(Brisbane/Chairman)2a | Argentina |

Attachment 61(Tel Aviv/CTL Liason)02d – 2: