| Document |  | Reference |  | Inquiry |  | Response |  |  | Attachment |  | RM |  | Country |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60335-2-6 |  |  |  | In the fourth edition of the International Standard IEC 60335-2-6: 1997 absent requirements for electric ranges (electrical appliances with one or some hobs or heating elements of the working surface for use on table or similar support, in which there are not electrical ovens or grill). Such requirements were presented in the third edition of the IEC 335-2-6:1986. At present there are no additional safety requirements for these widely distributed household electrical appliances in the IEC Standards. |  | These appliances are covered by IEC 60335-2-6 or IEC 60335-2-9 dependent upon whether they are stationary appliances or portable appliances. Please refer to definitions 2.6.1, 2.6.3 and 2.6.4 in IEC 60335-1. |  |  | None |  | 61(Kagoshima/Chairman)1 |  | Belarus |
| 60335-2-6 |  | 3.1.6 |  | EN60335-2-6 gives a diversity factor of (0.35+0.65/sqrtN) that can used to reduce the supply cord and terminal sizes on appliances with N heating units, where N>3. Alongside this, IEE regs gives a different diversity factor which recommends a current rating of 10A plus 30% of the remaining load. The problem is that these two approaches give quite different answers. For example, as shown in the attached table, for an oven and hob combination with a total wattage of 16.3kW and 7 heating units (which is a situation we are involved in), EN60335 would give 40.46A whereas IEE say 27.38A would be OK. Another problem is that the EN60335 formula gives a rating for a 4 zone hob which is less than a 3 zone hob, which seems odd. This is important in the context of what manufacturers say in their instruction books regarding cable sizes. Have you come across this before? I don’t know whether you remember our Wiring Regs but do you have the reason behind the TC 61 diversity statement in note 101 to 3.1.6. |  | The origin of the formula in IEC 60335-2-6 (I cannot comment on EN 60335-2-6) is lost in the mists of time. However if you plot it on a log/linear graph it is almost a straight line for N=4 and greater. To me this indicates that the committee in the early days considered that element usage decreased almost exponentially according to the number of heating elements available if there were 4 or more. Hence it was OK to use the formula to determine the current used to establish the size of the terminals and the nominal cross-sectional area of the supply cord. See the diversity factor plot below – applicable for integer values of N. This pragmatic approach also takes account of the fact that heating elements are not all simultaneously operating on full load or for long periods. I am not sure about the IEE wiring rules but the AU & NZ wiring rules are for the installation wiring as are the IEC wiring rules, the supply cord is not part of the installation wiring. I see no reason other than one of nicety for any wiring rules diversity factor to be the same as the appliance standard diversity factor since they are used for different purposes. In any case the question about the origin of the formula has been raised before and even John Humphries, Gert Bukkjaer and Jan Erik Pettersen could not shed any light on its origin other than it came from the old CEE part 2. Its continuing use was confirmed by TC 61 at that time. United Kingdom 3: Confirmed |  |  | None |  | 61/4042e/RM |  | United Kingdom |
| 60335-2-6 |  | 3.1.6 |  | Taking into account the definitions stated in clause 3: 3.1.6 Note 101: "For appliances having more than three heating units per phase, a diversity factor is applied to the rated current or rated power input when determining the current used to establish the size of the terminals and the nominal cross-sectional area of the supply cord. The diversity factor F is calculated from the following formula, where N is the number of heating units per phase that can be energized together : F = 0,35 + 0,65/√N 3.108 induction hob: hob containing at least one induction hob element 3.112 induction hob element: hob element that heats metallic vessels by means of eddy currents 3.109 heating unit: any part of the appliance that fulfils an independent cooking or warming function How to consider the application of diversity factor to the following induction appliance (see schematic circuit) where: - the induction hob is composed of 2 induction generators which fulfil an independent cooking and - each induction generator is composed of 2 induction hob elements which are not independent together. (Distribution of the power input of the induction generator between 2 induction hob elements) ? |  | For this case, the diversity factor is not applicable if the power input does not vary when the hob elements of the same generator are working together or separate, but it is applicable if the power input with both operating is the sum of each unit operating alone 41c) Agreed |  |  | 61(Penang/CTL Liason)02d- 09/07 |  | 61/3508/RM |  | CTL Inquiry - 10/07 |
| 60335-2-6 |  | 3.1.9.101 |  | Question: In note 3 of clause 3.1.9.101 it is specified the use of a non-circular vessel, for non-circular cooking areas. In fact, only the characteristics of circular vessels are specified in the standard. Furthermore, the last part of the text in note 3 require the use of information regarding circular vessels (amount of liquid depending on smallest diameter of the cooking area) 1- Should note 3 say "the smallest circular vessel" instead "the smallest non circular vessels"? 2. If the answer to 1 is no. What characteristics should have the non-circular vessels to be used in this test? 3. When the shape of the cooking zone is elongated. It is possible to use more than one circular vessel for the same cooking zone, if this condition is closer to cover the shape of the cooking zone than using only one big vessel? Rationale: see question ### Proposal (if any): 1: Note 3 has to be applied. To help in the common understanding the above sketch can be referred. In case of need the manufacturer can be asked to supply the suitable vessel. 2: The material has to be the one indicated in Figure 102. 3: No, it is not possible to use more than one circular vessel for the same cooking zone. |  | 19/07 - Agreed accept for answer 2. The material specified in Figure 102 is only applicable to pots for induction hob elements. For other hob elements the pot material is unpolished commercial quality aluminium as specified in the first paragraph of 3.1.9.101 |  |  | 61(Tokyo/CTL Liaison)02d –19/07 |  | 61/3355/RM |  | CTL Inquiry – 19/07 |
| 60335-2-6 |  | 11.101 |  | 1. Sub-clause 11.101 In note 2 is specified that , if a door is protected by a guard, the temperature rise specified for the front surface of oven door apply to the guar. Question : since the definition of “guard” is missing in the standard, is correct to consider as “guard” an external additional (accessory) part intended to prevent the contact of the user against the hot surface of the oven door, supplied with the appliance by the manufacturer and which shall be mounted according to the instructions by the user? If yes, this means that an internal protection mounted by the manufacturer in the factory, has not to be considered as a “guard” in respect of this sub-clause, even if this guard is detachable for cleaning purposes according to the instructions?” 2. Clarification on testing methods The Italian committee would like to have clarification about the testing methods for the heatings during the cooking function: "GRILL with HOT AIR FAN". THE ITALIAN COMMITEE SUGGESTS: The tests with this kind of function should be performed as stated for Grills; without considering what is prescribed in clause 3.1.9.102 for ovens ¦with forced air circulation Remark: In the European \standard EN50304 (electric ovens for household use: methods for measuring the energy consumption) the clause 4.2: definition for forced air circulation function, it doesn't include those functions which operate a Grill element only. Example: clause 11; Function: "GRILL with HOT AIR FAN" Temperature setting: Maximum position or as specified by the manufacturer Test duration: 30 minutes Grill pan and food supports:unfavourable position. |  | Q1 – The answer to the first part of the question is “Yes”, and to the second part the answer is also “Yes” because temperatures are only measured on the outside surface of the oven door or the guard on the outside of the oven door. Q2 – The ITNC interpretation is correct. |  |  | None |  | 61/2898/RM |  | Italy |
| 60335-2-6 |  | 11.101 |  | 1. Subclause 11.101 of IEC 60335-2-6, draft edition 5 The text of 11.101 (see document 61/2021/Q) is based on documents 61/1732/CDV, 61/1794/RVC and the decisions taken in Bangkok following ad hoc group proposals (see minutes of Bangkok 61/1891/RM, page 19). The question concerns the deletion of the paragraph after table 102 of 61/1932/CDV: “The values specified for oven doors only applies to front surfaces of the door which are accessible to test probe 41 of IEC 61032 applied perpendicularly to the surface.” The reasons do not appear in the Bangkok minutes. The existing standard includes this requirement and we do not see any reason to delete it. |  | 1) Subclause 11.101 of IEC 60335-2-6 – Draft Edition 5 The inquiry was noted, but no action was necessary. |  |  | None |  | 61/2093/RM |  | France |
| 60335-2-6 |  | 11.2 |  | The last sentence of the requirements of sub-clause 11.2 of IEC 60335-2-6 5th Ed 2002 states that: - Detachable parts that are intended to be used to reduce the temperature of control panels are removed NOTE101: A retractable part is not considered to be a detachable part. QUESTION We would like to know: 1. If the term “control panels”, for which no definition is given neither in Part 1 nor in Part 2-6, defines only the kind of panels used for the aim of sub-clause 7.12.4 or also includes, more in general, handles, knobs, grips and similar parts. 2. If the answer to 1. would be that handles, knobs, grips and similar parts have to be considered, we question if it is really the intention to allow, as indicated in the requirement and the note, only retractable panels (and not the detachable ones, used without problems since many years) in order to protect handles, knobs, grips and similar from excessive temperatures originating from the oven during the grill function test. 3. If the answer to 2. would be that only retractable panels are allowed, we would like to know how to consider the first sentence of 3.1.9.103: “Grills are operated empty with the grill pan and food support in the most unfavourable position, the door and any other accessories (Note from IT NC: like the detachable panel) being positioned in accordance with the instructions” |  | 1. Yes 2. Yes 3. Follow the instruction if they state pull retractable guard. If the instructions say nothing then position the guard according clause 5 |  |  | None |  | 61/2971/RM |  | Italy |
| 60335-2-6 |  | 11.3 |  | In the recent time more often the situation occurs that due to high material prices that the manufacturer want to use wire made out of a combination of aluminium core (85%) and copper sheath (15%). With this kind of wire the traditional resistance method to find the temperature in a winding as listed above does not work. If you do worst case calculation by using the copper value then most of the appliance fail and when you use the aluminium value then the material requirements for the insulation will be too low as the calculated temperature is lower than the real one. A complicated reversed calculation could be done by considering this special wire as parallel resistor network when the diameter of the copper and aluminium are measured. The intention is to use the wire in motor and transformer of standard frequency of 50/60Hz. Up to now we have observed that it is used in motor of kitchen appliances and microwave oven. |  |  |  |  | None |  | 61/3777e/RM |  | China |
| 60335-2-6 |  | 19.11.2 |  | During discussions with a test lab in Spain about the approval of an electronic cooktop the following question aroused: When the hob is tested according IEC 60335-2-6 Ed. 5.1 clause 19.11.2 we simulate one fault (for example shorting a transistor). After that, it is possible to switch off the electronic. Therefore we implement according to 19.11.3 a second fault (for example shorting another transistor). What is the compliance criteria in that situation. To answer this question I found in IEC/61/3296A/INF the following comment from Australia: "the compliance criteria of 19.11.3 is in 19.13. The Compliance criteria for 19.11.2 is contained in 19.11.2 ... " Is this statement still correct? or is it necessary to be able to switch off even after implementing the second fault. |  | The statement is correct and the standard is very clear on this point. For Part 2-6: 19.11.2 Addition states in part During simulation of the fault conditions, it shall be possible to switch off any energized hob element. 19.1 in part 1 states in part Unless otherwise specified, compliance with the tests of this clause is checked as described in 19.13. Since nothing additional is said in 19.11.3 in part 2-6, then 19.13 is the pass fail criteria for 19.11.3. Germany 1 – Confirmed, with the following addition at the end of the response: “However, for the future, the compliance criteria in the original test clause will also be applied in 19.11.3 and the standard will be amended to make this clear.” |  |  | None |  | 61/3777e/RM |  | Germany |
| 60335-2-6 |  | 19.13 |  | The new edition of IEC 60335-2-6 (Edition 5) had incorporated the requirement of 19.4 last edition in subclause 19.13. I think it was only an editorial exercise to regroup all the requirements of 19 in subclause 19.13, but it leads to interpretation from testing stations. The problem arose when the condition of double fault was applied to an oven with a latch door system (pyrolitic self cleaning oven) when performing 19.11 in electronic circuit. A failure was made to the electronic circuit which causes the fault on the latch door system (and had the normal consequence the door could be opened at any time) because the electronic temperature sensor was defeated. After doing this fault condition the 19.4 was applied and of course the door could be opened when the temperature at the center of oven exceeded 425°C. It has been considered by the testing station that the appliance didn't fulfilled the standard requirements because according to the 19.13 the door was able to be opened when the temperature was over 425°C, even if all other requirements of 19.13 was fulfilled (temperature rise of wood, components...). |  | The failure of the door interlock is only taken into account during the test of 19.4 and not during the test of (19.11 + 19.4). However, in terms of functional safety when amendment 1 is published there may be a conflict with the introduction in 19.13 of the criteria that “The appliance shall not undergo a dangerous malfunction”. d. France 1: The response in 61(Toulouse/Chairman)2a was confirmed, however the dangerous malfunction criteria only applies when applying the test of 19.11. The CTL is asked to prepare a decision sheet on this matter. |  |  | None |  | 61/2568/RM |  | France |
| 60335-2-6 |  | 19.102 |  | In our lab we have been faced with difficulties when performing tests in accordance with the standard IEC 60335-2-6:2002, Clause 19.102. We have conducted an investigation. The result can be found in the attachment. In discussion with the German Institute VDE (and also some other institutes and producers of household appliances) we found out that they have also been confronted with the same problem. VDE shares the same opinion, which can be seen from the letter attached. We think that it is not tolerable to have requirements in standard that can not be fulfilled. Such requirements cause a lot of problems, do not contribute to reproducibility and repeatability and introduce a confusion. According to all our contacts and discussions with different labs and manufacturers, it seems that we all have the same problem which needs some clarification. In summary: At dimensions to 210 mm (diameter) there was no problem. At dimensions between aproximately 210mm and 240mm vessels went convex but not so much to loose power so temperatures (heating of oil) could be reached but cooking zone power went off before reaching 270K (so the test would be positive if vessels would not be convex). For dimensions over aproximately 240mm vessels went so much convex that we could not reach oil temperature as cooking zone lost power before we could end the test. And there we can also have a case when vessel becomes too concave and loses contact in the middle of cooking zone and sensor does not function (and oil goes above 270K). |  | Thank you for this information. A CDV for a new edition of IEC 60335-2-6 will be prepared following the meeting of TC 61 in Bangkok. I shall arrange for your inquiry to be presented as a tabled document at that meeting and we shall discuss the problem with national committees. We will also take into account what is specified in C.11 of IEC 61000-3-2. However please be aware that the requirements in the current edition of the IEC standard were introduced based on submissions from IEC national committees in Europe (France and Germany) so I can only presume that your concerns were taken into account when the submissions were formulated. It is never the intention of IEC/TC 61 to consciously introduce requirements in the 60335 series of standards that do not allow reproducible and repeatable test results since that is of no value to any stake holder within the IEC/TC 61 community. TC 61 confirmed the following response: The dimension of the pan is confirmed as in the standard. If the pan goes convex during the test this is ok. |  |  | None |  | 61/4042e/RM |  | TCL |
| 60335-2-6 |  | 19.102 |  | Question: In 19.102 it is required to short-circuit the thermal control for the test. Note 5 of Sub-clause 19.1 of IEC60335-1 explains that short-circuit of controls may be render them inoperative instead. In induction hobs, the thermal control of hob elements is normally a NTC controlled by the microcontroller. Short-circuit or open circuit of the NTC is easily detected by the micro, but fixing the NTC in an intermediate value may not be detected. IEC60730-1 Annex J has the requirements to comply with by thermistors. Among them there is a test for the drift of the R/T characteristic when the control is classified as Type 2. Question: Shall we consider fixing the NTC in a specific value so as to render the thermal control inoperative which is the condition consequently required to be checked by 19.102? Rationale: (See question) ### Proposal (if any): The NTC can be fixed to a specific value, but this value has to be found in the middle of the range of characteristic of operation of the NTC, to avoid hysteresis problems may be happen towards the end of the range. EXPLANATION: Decision confirmed by CLC/TC61 (November 2007) This decision is similar to OSM/HA 418 |  | The intent is to set up the electronics so that the appliance continues to operate without the clause 11 control operating. If what is proposed achieves this aim then it is OK. Inspection of the circuit diagram may show that it is also possible to bypass the micro to achieve the aim – this could also be considered. |  |  | None |  | 61/3777e/RM |  | IMQ |
| 60335-2-6; 60335-2-9 | 19.102; 19.104 |  | In 19.102 of IEC 60335-2-6:2002(Also in 19.104 of IEC 60335-2-9:2002), it is required to short-circuit the thermal control for the test. Note 5 of Sub-clause 19.1 of IEC60335-1:2006 explains that instead of short-circuiting, controls may be rendered inoperative. In induction hobs, the thermal control of hob elements is normally a NTC controlled by the micro-controller. Short-circuit or open circuit of the NTC is easily detected by the micro-controller, but fixing the NTC in an intermediate value may not be detected. IEC60730-1:2007, Annex J has the requirements for thermistors. There is a test for the drift of the R/T characteristic when the control is classified as Type 2. Should the NTC be fixed at a specific value so as to render the thermal control inoperative, which is the condition required to be checked by 19.102? |  | Decision: The NTC can be fixed to a specific value. This value must be in the middle of the operating range of the NTC, so as to avoid hysteresis problems that may happen towards the end of the range. Explanatory Notes: The intent is to set up the electronics so that the appliance continues to operate without the clause 11 controls operating. Inspection of the circuit diagram may show that it is also possible to bypass the micro-controller to achieve the intent. The interpretation of NTC may be extended to other sensors or controls that are working in clause 11 even these are not having a direct action of controlled parts. Decision was confirmed by CLC/TC61 (June 2009). It is similar to OSM/HA 418 modified. CTL/PDSH725 (revised): Replace the explanatory note with the following: “For induction hobs that use other types of electronic controls to control the temperature in clause 11, the intent is to set up the electronics so that the appliance continues to operate without the clause 11 controls operating. Inspection of the circuit diagram may show that it is also possible, for example, to bypass the micro-controller to achieve the intent.” |  |  | None |  | 61/3927eR/RM |  | CTL Liason |  |
| 60335-2-6; 60335-2-9; 60335-2-15 |  | 19.102; 19.104; 15.102 |  | 2a) Question: In clauses 19.102 of IEC 60335-2-6:2002 and 19.104 of IEC 60335-2-9:2002), it is required to short-circuit the thermal control for the test. Note 5 of Sub-clause 19.1 of IEC60335-1:2006 explains that instead of short-circuiting, controls may be rendered inoperative. In induction hobs, the thermal control of hob elements is normally a NTC controlled by the micro-controller. Short-circuit or open circuit of the NTC is easily detected by the micro-controller, but fixing the NTC in an intermediate value may not be detected. IEC60730-1:2007, Annex J has requirements for thermistors. There is a test for the drift of the R/T characteristic when the control is classified as Type 2. Should the NTC be fixed at a specific value in order to render the thermal control inoperative? What is the required condition to be checked by clause 19.102? Decision: The NTC can be fixed to a specific value. This value must be in the middle of the operating range of the NTC, in order to avoid hysteresis problems that may happen towards the end of the range. Explanatory Notes: For induction hobs that use other types of electronic controls to control the temperature in clause 11, the intent is to set up the electronics so that the appliance continues to operate without the clause 11 controls operating. Inspection of the circuit diagram may show that it is also possible, for example, to bypass the micro-controller to achieve the intent. Decision was confirmed by CLC/TC61 (June 2009). It is similar to OSM/HA 418 modified. 2b) Question: Could the following procedure be considered to improve the consistency of the method used for the electric strength test following the spillage test of sub-clause 15.102? The connector system is placed on a metal foil sheet on the bench surface. After the prescribed amount of water is deposited through the 8mm tube onto the connector surface, the electrical strength test shall be applied as soon as practical. Immediately following the deposition of the water, metal foil is smoothed over the accessible surface of connector, avoiding contact with any earth connection by means of a hole or gap in the foil. The application of the metal foil is carried out in such a manner, as to minimize any disturbance to water on the surface of the connector. The following test voltages are then applied as follows: a) 2500V a.c. between live parts and the metal foil over the surface of the connector b) 2500V a.c.between live parts and the metal foil surface under the appliance base c) 1250V a.c. Between live parts and the earth connector Decision: The above test procedure including the test voltages is confirmed. Explanatory Notes: This decision is similar to OSM/HA 429. |  | 2a: Noted 2b: Noted |  |  | None |  | 61/4042e/RM |  | CTL Liason |
| 60335-2-6 | 21.1; 21.102 |  | We are a glass hob-top producer for household applications and our laboratory would like to know the meaning of the note 101 ("if the hob top surface comprises a single piece of material except for the outer frame this test is not carried out") Mechanical Strenght IEC 60 335-2-6 2002-2006 . Is it applicable the spring hammer test in case of single piece or not ? In case of positive answer is it for 0,5 or 0,7 joule ? Because we couln't find an answer to the question we will really appreciate if somebody can explain the point or kindly give us an indication where we can find an answer . |  | The standard is very clear. The tests are carried out as follows. 1) Apply the 0,5 J hammer test according to IEC 60335-1 clause 21.1. 2) Apply the test of 21.102 IEC 60335-2-6 3) If the hob surface does not comprises a single piece of material except for the outer frame, apply the 0,7 J hammer test according to IEC 60335-2-6 clause 21.1 addition. Italy 1 – Confirmed |  |  | None |  | 61/3777e/RM |  | Italy |  |
| 60335-2-6 |  | 21.101 |  | We agree with IEC/TC61 position (61/3956/INF) about IMQ Primacontrol letter concerning sub-clause 22.122 of IEC 60335-2-6 +A1+A2. Now we would like to have your feed back about sub-clause 21.101 of A2 which states: 21.101 Add the following to the test specification: Ovens with shelves that can be withdrawn and that are fitted with stops are also tested as follows. The shelves are fully extended to the maximum distance allowed by the stops. An evenly distributed force of 80 N is applied to each shelf, at locations along the front edge of the shelf, using a vessel having side dimensions of 200 mm, one side of the vessel being aligned along the front edge of the shelf. During this test, the shelf shall not tilt downwards by more than 6 o. The question is: shall the test with 80 N be performed to shelves, including roasting trays, which are not fitted with stops? Our understanding is NO because in the test specification is stated “Ovens with shelves that can be withdrawn and that are fitted with stops….” |  | If they are not fitted with stops how do they comply with 22.122 ? They all have to be fitted with stops to comply with 22.122. Italy 2: Confirmed. However, add the following “they all have to be fitted with stops to comply with 22.122”. |  |  | None |  | 61/4042e/RM |  | Italy |
| 60335-2-6 |  | 21.101 |  | In your role as Chairman of the IEC TC 61 committee, can you please provide Choice with a written interpretation of IEC 60335-2-6 Shelf Tilt Cl.21.101? |  | The standard is very clear and it states that the test is carried out as follows: The shelves are fully extended to the maximum distance allowed by the stops. An evenly distributed force of 80 N is applied to each shelf, at locations along the front edge of the shelf using a vessel having side dimensions of 200 mm, one side of the vessel being aligned along the front edge of the shelf. During this test, the shelf shall not tilt downwards by more than 6 deg. So the complete test is "fully extend the shelf then load as stated" The allowed downward tilt of 6 deg is the sum of any tilt that occurs due to fully extending the shelf plus the additional tilt due to the load. It is NOT the additional tilt that occurs due to the loading of the shelf after it is in its fully extended position. Australia 1 – Confirmed |  |  | None |  | 61/3777e/RM |  | Australia |
| 60335-2-6 |  | 22.122 |  | In IEC 60335-2-6 Ed.5.2 it is stated: 22.122 ovens with shelves that can be withdrawn shall be fitted with rest positions to prevent the inadvertent withdrawal of the shelves Compliance is checked by inspection and manual test. We want to know which of the following oven racks are considered to be shelves in sense of the standard. Examples of different "shelves": A) - Stylish, durable glass tray can go straight to the table from the oven - Practical and easy to clean - Dishwasher safe - Provides a better view of the food inside the oven when cooking at different levels - Dimensions: 46.3 cm x 37.5 cm x 3 cm B) - Ideal for grilling or roasting on a big scale - Perfect for big grills, roasting turkeys or cooking up party quantities of lasagne - With the addition of an optional enamelled lid 437803 - Dimensions: 46,5 cm wide x 37,5 cm deep x 8 cm high - Capacity: 10 litres C) - Consists of deep enamel pan and stainless steel perforated tray - Includes two small trays (one perforated, one solid) - Large glass cover Large tray is ideal for steaming fish, meat or poultry - Small tray is ideal for vegetables and sauces - Healthy way to prepare food - The system steamer can be used in ovens with either a steam setting or a bottom heat only function D) |  | All of the examples given are considered to be shelves within the meaning of the IEC standard |  |  | None |  | 61/4080ea/RM |  | DENC |
| 60335-2-6 | 22.122 |  | Please refer to the letter from 'IMQ' and a copy of Clause 22.122 from AS/NZS 60335.2.6 (attached). Is the proposed amendment to EN 60335.2.6 a 'flow-on' from decisions taken within TC61 (and as such will ultimately become an amendment to AS/NZS 60335.2.6 : 2008 Clause 22.122 ) or is it solely a 'EN' deviation. To date no relaxation has been offered for 'Roasting Trays' for approval in Australia. |  |  |  | The proposed amendment to EN 60335-2-6 is in direct conflict with the decision taken by TC 61 at its meeting in Kuala Lumpur in response to an identical proposal from Australia. Consequently, the interpretation given by IMQ concerning IEC 60335-2-6 is not correct and the proposed amendment to the EN is not for inclusion in the IEC standard. The relevant documents relating to the proposal from Australia are: 61/2398/DC that stated (in part). 22.1XX Ovens with a capacity exceeding 20 l and having withdrawable shelves shall be fitted with stops to prevent the inadvertent withdrawal of the shelves. The shelves shall be capable of being withdrawn so that when fully extended to the maximum distance allowed by the stops, the front edge is not less than 160 mm from the front surface of the oven on which the door comes to rest in the closed position. The shelves shall also be constructed to prevent cooking dishes or the like from sliding over the rear edge. These requirements do not apply to shelves that are designed to contain liquids, such as roasting trays and the like. Compliance is checked by inspection and by manual test. This proposal from Australia was rejected based on a comment from Germany (see comment 19 in 61/2545A/INF) that stated: Australia 2: Confirmed. Prepare INF document circulating IMQ letter and refuting the information included in that letter. | 61(Tel Aviv/Chairman)02a – Australia 2 |  | 61/3927e/RM |  | Australia |  |
| 60335-2-6 | 24.2 |  | We have a portable induction hotplate with a separate SELV control box(control panel). The control box is at the end of the SELV cord, with a connecter in the middle of the cord. One of the certificate bodies thinks it's not acceptable because: 1) The SELV control box is a control in the flexible cord, not complying with clause 24.2 2) A portable induction hotplate shall not have separate control box, even though SELV control box. Only built-in appliances can have separate control box. They say: 1) The standard doesn't allow control in flexible cord, even SELV cord complying with clause 8.1.4 and clause 17. So the control box and the appliance has to be fixed, and shall comply with 60335-2-6. 2) According to clause 7.12.4, they think only built-in appliance can have separate control panel. No other appliances are mentioned to have separate control panel. 60336-2-6 7.12.4 Addition: The instructions for built-in appliances having separate control panels shall state that the control panel is only to be connected to the heating units specified in order to avoid a possible hazard. |  | In terms of the published standards the answers are Q1: Correct does not comply – when it is stated in Part1 “in a flexible” this also means at the end of a flexible cord. Q2: Correct for Part 2-6 but not explicitly stated in Part 1 to be applied in general. China 1: Modified as follows: Q1: Correct, does not comply. When it is stated in part 1 “in a flexible cord” this also means at the end of a flexible cord Q2: Correct for Part 2-6 but not explicitly stated in part 1 to be applied in general. |  |  | 61(Rimini/Chairman)02a – China 1 |  | 61/4285/RM |  | China |  |
| 60335-2-6 | 29.2 |  | Clause 29.2 of IEC 60335-2-6 states that 29.2 Addition: The microenvironment is pollution degree 3 unless insulation is enclosed or located so that it is unlikely to be exposed to pollution during normal use of the appliance. Our questions are: What is the reason for this requirement? Is it the atmosphere in the kitchen which may pollute with grease and steam that require a classification of the appliance in pollution degree 3 or Is it the opinion of the committee that a cooker will produce a dirty atmosphere inside the appliance by itself? We consider the atmosphere in a commercial kitchen as always polluted. Therefore the standards under the scope of IEC 61E require pollution degree 3. For us is the air in a normal kitchen in a house considered to fall under pollution degree 2 conditions where non conductive pollution may occur on a temporally basis. Kitchen machines room-heaters, storage room heaters, fans in air ducts also located in a normal kitchen are considered to fall under pollution degree 2 In Europe a discussion started between manufacturer and test houses on ovens designed as shown in the hand made drawing below Figure 2 and figure3. The cavity and the exhaust of it are completely separated from the outside of the cavity. There is a separate duct sucking the air on the bottom of the appliance and blow it around the cavity. The cooling air leave the appliance in the gap between the panel and the door. In figure 2 the cavity is vented direct through the door so that the polluted steam leaves the door through vents. In figure 3 the steam from the cavity is mixed with the cooling air in the duct before leaving the appliance. [See attached graphic: 60335-2-6 (Figs 2 & 3)] A hob is normally not exposed to the atmosphere. During production the components are mounted and then the hob is closed. There are only small holes in the enclosure allowing the ingress of air from outside [See attached graphic: 60335-2-6 (hob)] We are of the opinion that for separate hobs , switching boxes for controlling the hobs and for cooker designed such that the cavity is sealed and polluted steam can not enter electrical components Pollution degree 2 is suitable To give more evidence we provide a separate document showing the inside of above mentioned appliances manufactured for more than 22 years and still in use without problems. According to the old IEC 60335-2-1 2nd edition severe condition applied |  | 1. The macroenvironment in a domestic kitchen is pollution degree 2 2. The microenvironment inside the appliance with respect to positioning of the insulation maybe 2 or 3 depending on the design and pollution produced by operation of the appliance itself |  |  | 60335-2-6 (Figs 2& 3); 60335-2-6 (hob); Attachment 1 - 61(Cape Town/DE)36; Attachment 2 - 61(Cape Town/DE)36; Attachment 3 – 61(Cape Town/DE)36; Attachment 4 – 61(Cape Town/DE)36; Attachment 5 – 61(Cape Town/DE)36; Attachment 6 – 61(Cape Town/DE)36; Attachment 7 – 61(Cape Town/DE)36; Attachment 8 – 61(Cape Town/DE)36; Attachment 9 – 61(Cape Town/DE)36; Attachment 10 – 61(Cape Town/DE)36; Attachment 11 – 61(Cape Town/DE)36; Attachment 12 – 61(Cape Town/DE)36 |  | 61/2971/RM |  | Germany |  |

Attachment 60335-2-6 (Fig 2 & 3):

- Attachment 60335-2-6 (hob):

- Attachment 1 - 61(Cape Town/DE)36:

- Oven +Micro+switching+box

Attachment 2 -  61(Cape Town/DE)36:

- Electronic PCB fromMicro

- Attachment 3 -  61(Cape Town/DE)36:

- The switching – box

- Behind the switching box

Attachment 4 -  61(Cape Town/DE)36:

- Inside the Switching – box

Attachment 5 -  61(Cape Town/DE)36:

Attachment 6 -  61(Cape Town/DE)36:

- Inside of the induction box

Attachment 7 -  61(Cape Town/DE)36:

- The induction-generator

Attachment 8 -  61(Cape Town/DE)36:

- The oven power board

Attachment 9 -  61(Cape Town/DE)36:

- The oven power board

Attachment 10 -  61(Cape Town/DE)36:

- The mixed hob

Attachment 11 -  61(Cape Town/DE)36:

- The mixed hob inside

- Attachment 12 -  61(Cape Town/DE)36:

Attachment 61(Penang/CTL Liason)02d- 09/07:

Attachment 61(Tokyo/CTL Liaison)02d –19/07:

Attachment 61(Tel Aviv/Chairman)02a – Australia 2:

Attachment 61(Rimini/Chairman)02a – China 1: