| Document |  | Reference |  | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60335-2-3 60335-1 |  |  | It concerns an iron with a thermal protection inside. When putting the iron on the table for more than 8 sec, it will shut off (in the case the user is interrupted during ironing and does not touch the iron for more than 8 sec). When the user re-uses it, it turns on again automatically. Then, if the user pushes the steam button immediately, hot water is pouring out of the iron until the water tank is empty. This action (to push the steam button immediately after a period of non-use) is according to our interpretation a normal reaction of the user. Furthermore, the quantity of hot water is considerable. On the other hand, in the instructions is stipulated that the user has to wait until the red lamp is lit, before pushing the steam button, otherwise no steam will come out. The question in general is : Can an instruction cover a dangerous situation even if it is clear that the user will encounter the situation in real use ? Or otherwise explained, can a product be in compliance with the standard, even if a dangerous situation can occur by following the instructions ? What will happen if a child is near his mother who is ironing and gets hot water over him ? |  | The relevant clause to consider in this situation is 22.102 and according to the wording it would seem that this construction fulfils the requirement. However, it also could be considered that the instruction in this situation is not enough and that such an iron does not fulfil the intention stated in the penultimate paragraph of the Introduction that states: “An appliance that complies with the text of this standard will not necessarily be considered to comply with the safety principles of the standard if, when examined and tested, it is found to have other features that impair the level of safety covered by these requirements” TC 61 is asked to answer this inquiry. Belgium 2 – The iron is in compliance with the standard and it is not considered necessary to modify the standard. | None | 61/3655/RM | Belgium |
| 60335-2-3; 60335-2-44; 60335-2-51; 60335-2-85 |  |  |  | The last years there are coming on the market various kinds of steam generators for household use, for which the accessories like electric iron accessories, floor treatment accessories, sprayguns for cleaning, fabric treatment accessories for curtains, carpets, upholstery etc. are sold separately or at the same time with the mainsteam unit. It is not clear to the Israeli committee under which part of IEC 335 they should be tested. | There is no part 2 standard for separate steam generating units. Requirements for appliances that use steam are covered by the specific parts 2, for example: IEC 60335-2-3 (for irons) IEC 60335-2-44 (for ironers) IEC 60335-2-51 (for spray guns for cleaning - see Amendment 1) IEC 60335-2-85 (for fabric steamers) This response will form part of the Chairman's Report to the next meeting of TC 61 which is to be held in Bangkok 22-26 May 2000. If TC 61 requires any modification to this response, you will be informed shortly after 26 May. | None | 61(Bangkok/Chairman)1 | Israel |
| 60335-2-3 |  | 3.5.2; 25.14 |  | I have a problem of interpretation concerning a iron because of the different interpretation as follow: 1. Which reference to Cl.3.5.2 (IEC60335-1) Cl.21.101 and Cl.25.14 this appliance is considered as handheld appliance - normal use is held in hand - Cl.21.101 the test set up is look like during ironing, it's held in hand. - Cl.25.14 It is applicable for appliance that supply cord move in normal use. Handheld appliance is within this situation. Or 2. If refer to Cl.11 (heating test) and Cl.3.1.9 (normal operation), it shall be consider as portable appliance because the appliance need not held in hand for operation. Question1: Do we consider dry iron as portable or handheld appliance? Question2: Is Cl.30.2.2 applicable for dry iron or not? | 1) A hand-held appliance is a special type of portable appliance as per definition 3.5.2 of IEC 60335-1. Concerning a dry iron, in normal use it is held in the hand when actually ironing but is placed on its stand between ironing operations, so in my opinion is it considered to be a portable appliance rather than strictly a hand-held appliance. Throughout IEC 60335-2-3, except for 27.6 and 30.2.2, there is no need to decide if a dry iron is to be considered a hand-held appliance because for all clauses where part 1mentions special requirements for hand-held appliances part 2-3 clarifies the requirement and/or the test specification. 2) For the reasons given in 1) above, in my opinion: - in 27.6 a dry iron should be considered as a hand-held appliance (which means that printed conductors of printed circuit boards shall not be used to provide earthing continuity); - in 30.2.2 a dry iron should not be considered as a hand-held appliance (which means that the test is applicable). This response will be submitted to TC 61/CAG and to TC 61 at it London meeting in May 2006 for confirmation or modification. If it is modified I shall inform you by the end of May 2006. NOTE: The CAG did generally not agree with my answer given in the first dash item and had a divided view given to my answer in the second dash item | None | 61(London/Chairman)2a | Thailand |
| 60335-2-3 |  | 25.7 |  | On the prohibition of PVC sheathed supply cords in 25.7 of IEC 60335-2-3 lead to the circulation of 61/2046/DC. | We decided to circulate a DC proposing an amendment to 25.7 of IEC 60335-2-3 on the basis that TC 20 had been asked by TC 61 to detail requirements for highly flexible cords that were suitable for use in high temperature applications. This proposal 61/2046/DC, has been supported (see 61/2079/INF) and will be circulated as a CDV during the next maintenance cycle for IEC 60335-2-3. | None | 61(Chicago/Chairman)1 | Israel |