| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-59 |  | I’m Eng. Asma’a from Jordan, we were in contact before and we need your help again about a handheld insect killer with racket shape, This insect killer works using a 2-dry batteries connected in series ( 3V DC). According to IEC 60335-2-59 for insect killers the classification for the appliance under this standard shall be Class I or Class II, and according to IEC 60335-1 cl. 3.3.12 the definition of Class III “class III appliance: appliance in which protection against electric shock relies on supply at safety extra-low voltage and in which voltages higher than those of safety extra-low voltage are not generated “ The first problem with this appliance is that it takes low voltage as input but generate high voltages as an output. What is the right classification for this appliance and according to which standard shall we test it. | This appliance can be tested to IEC 60335-2-59 as a class II appliance. Jordan 1: Confirmed | None | 61/4212e?RM | Jordan |
| 60335-2-59 | 8.1.1 | I have been asked a question concerning the application of 8.1.1 to insect killers according to 60335-2-59. The dilemma is as follows: 22.103 would imply that it is acceptable to be able to touch the any part of the grid with Test probe B of IEC 61032 during user maintenance such as replacing a lamp or emptying the dead insect collection tray. Application of 22.101 and 24.101 also allows the use of an interlock to provide protection when accessing the grid during user maintenance. However, 8.1.1 in part 2-59, does not in general allow access to the grid (since by definition it is a live part) except as stated in the addition for this part 2 (applicable only to class I appliances) Since it is, apparently, intended to allow the grid to be touched during user maintenance under the conditions in 22.101, 22.103 and 24.101 then for clarification there should probably be additions to 7.12 and 8.1.1 stating the substance of the following ? In 7.12 add If the grid is accessible when replacing a lamp or starter, an instruction to disconnect the appliance from the supply before replacing a lamp or starter shall be given (this instruction is not necessary if the appliance is fitted with an interlock in accordance with 22.101 and 24.101) In 8.1.1 add If the instructions state that a part is to be removed when replacing a lamp and a tool is required for its removal, the part is not considered to be a detachable part provided that after removal of the cover, access to the grid is allowed, but access to other live parts is prevented by at least basic insulation For further clarification, in 22.103 the substance of the following text should be added to the second sentence of the test specification: “or for appliances fitted with an interlock switch to protect against access to live parts during user maintenance, the interlock switch is operated to disconnect the grid”. | Australia 1 – The decision is modified as follows: a) Lamps are always considered replaceable by the user unless the appliance is destroyed when trying tore place the lamp and the instructions state that the lamp cannot be replaced. b) Circulate a DC to indicate this. | None | 61/3366/RM | Australia |
| 60335-2-59 | 22.104 | We need to test a handheld insect killer with racket shape, so we decide to test it according to IEC 60335-2-59/2002 For insect killer. What I need to Know first about measuring the short circuit current in Cl. 22.104: the right way to measure that current and the suitable device for such measuring for current about 10 mA. As may be you Know that kind of appliances which being in contact to the mains source for charging purposes and is still be able to work during the charging are having 3-grids, I need to know the middle grid is being accessible by hand are to be considered as dangerous accessible part in normal use or we decide according to the current passing throw it during the short circuit test. | One way to measure the current is to measure the voltage drop across a current shunt and then use Ohm‟s law to calculate the current. Under all conditions of normal use (including charging) you must ensure that live parts are not accessible. To decide if an accessible part is a live part, please apply 8.1.4 of IEC 60335-1. Jordan 1: Confirmed | 61/4080ea/RM – Jordan 1 | 61/4080ea/RM | Jordan |
| 60335-2-59 | 32 | I have an example of air cleaner as attachment picture that incorporated UV lamp inside. The appliance has 2 independent interlock switches to detected filter in position. If I remove filter out of position, UV cannot operate. According to IEC 60335-2-65 or IEC 60335-1 standard. There are no requirements for measurement of UV. | Yes – in this case is acceptable to apply IEC60335-2-59 Cl.32 Thailand 3: Confirmed | 61/4042e/RM – Thailand 3 | 61/4042e/Rm | Thailand |

Attachment 61/4080ea/RM – Jordan 1:

Attachment 61/4042e/RM – Thailand 3: