| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-60 | 22.103 | Concerning the hair entrapment test of 22.103, it is stated in part that "The hair is moved from side to side for up to 2,5 min in an attempt to get it sucked completely into the opening." A test lab has reported (not surprisingly in my view) that if the hair is swept side to side in a circular motion in the direction of the vortex, the hair is easily sucked in opening but not if it is swept across a chord or diameter of a circular opening. So the question is Does side to side mean for example across a chord or diameter of a circular opening or is it correct to sweep it around the perimeter in/or against the direction of the vortex ? | All responders were of the opinion that the reason of this test (with the cover removed) is to prevent the entrapment of the hair when the user is in the whirlpool bath, so the test should be carried out moving the testing hair from side to side and not following the direction of the vortex, if any. A number of responders considered that if the committee agreed that the hair should be swept in a direction following the vortex when the test is carried out with the cover removed, then this should be made clear in the standard. The intent of the test with the cover in place is to check if the hair, when sucked, is clutched by the protection grid of the opening or by other parts of the suction hose and also to see if it can displace the cover. | None | 61(Jeju/Chairman)2a | Australia |
| 60335-2-60 | 22.103 | I have been asked about the details of the tests in clause 22.103 in IEC 60335-2-60 Ed 3.1. I am having difficulty in understanding the methodology expressed by the words. The second to last paragraph of the test specification states that the hair test is also carried out with detachable covers in place. This would imply that the test in the preceding paragraphs is carried out with detachable covers not in place. Is this correct ? If this is the case then I wonder what is purpose of the second test. I would then suggest that it be stated for the first test that detachable covers are removed and to delete the second test. If this is not the case then I suggest that the word "also" be deleted from the second to last paragraph and to state that during this sweeping test if the cover is displaced then the first test (the force test) is repeated with the cover removed. | 1) It was agreed that the first test is carried out with detachable covers removed, therefore at the next amendment we should add some text to clarify this. - 2) One member of the CAG stated that this was not necessary because of 5.5 of part 1 that states “The tests are carried out with the appliance or any movable part of it placed in the most unfavourable position that may occur in normal use”. However, in my opinion this is an incorrect interpretation because - operating an appliance after removing a detachable part (which is defined) is not "normal use". - “normal use" does not include "user maintenance" (which is defined) | None | 61(London/Chairman)2a | Australia |