| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-53 |  | I will like to find out whether the attached product can be tested to IEC 60335-2-53. See photos and relevant extracts from the instruction booklet. (See Attachment 61(Oslo/Chairman)02a-South Africa 1) | This appliance can only be tested to IEC 60335-1 at this time, it is not possible to test it to IEC 60335-2-53. However, you need to take into consideration the requirements detailed in IEC 60335-2-105 as far as is reasonable. Possible entrapment aspects may be covered by IEC 60335-2-27 by applying for example 22.101 and 22.113. The ozone generation aspects will need to be covered by applying the tests in IEC 60335-2-65 clauses 16.101 and 32. At this time requirements for infrared radiation exposure are under consideration and hopefully will be finalized by the end of this year for infrared sauna cabins. It should be noted that IEC 60335-1 does not cover any electro-medical aspects. South Africa 1: Confirmed | 61(Oslo/Chairman)02a-South Africa 1 | 61/3827/RM | South Africa |
| 60335-2-53; 60335-2-32; 60335-2-27; 60335-2-17 | Scope | 1. Electric steam sauna in which someone can take a bath by giving off steam to have electric thermal devices installed in such as wooden bathroom, etc. 2. Vibrator which can massage someone by revolutions of electric motors and is household. 3. Appliance giving off ultraviolet rays which can sunburn a human skin and is household. 4. Electric pack which can be heated by electric thermal devices in the inside of pack and is household. | 1. The applicable standard is IEC 60335-2-53 + IEC 60335-2-98 (see London meeting agenda item 18). 2. The applicable standard is IEC 60335-2-32 3. The applicable standard is IEC 60335-2-27 4. The applicable standard is IEC 60335-2-17 | None | 61(Jeju/Chairman)2a | Korea |

Attachment 61(Oslo/Chairman)02a-South Africa 1:

60