| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-65 |  | I have got a message that our ozone limit in 60335-2-65 (5X10-8 %) is very low and very much less than the normal percentage outdoor. Is there something wrong in the standard or is this guy simply doing a mistake. | Yes – the value of the ozone limit should be 5 x 10 -6 %. This equates to a limit of 0,05 ppm that was specified in the previous edition. The Directives prohibit the use ‘’ppm’’ so % must be used. We should correct the standard via a corrigendum. | None | 61(Kuala Lumpur/Chairman)2a | Denmark |
| 60335-2-65 | 8.1.4 | Nowadays, sc ado-d belong to TC household appliances mentioned, develop activities around the adoption and adapting international standards as national standards (NMX) with our national deviations. our 1999 annual work program include the adoption of iec 60335-2-65 as mexican national standard; throught the international standard review process to define mexican national deviations to it, surged a doubt that I would like you could help me clarify as follows: around subclause 8.1.4 - addition, we would like to know, which is the test method to evaluate or measure discharge from the parts which are accessible and having a peak value over 15 kV and like consequence to get an discharge not exceed 350 mJ? by the way the second one, subclause 16.102 which indicate that high voltage transformers shall have adequate internal insulation. we think that both subclauses are related; there is a technical relationship between them. in mexican market, there are several air cleaning appliances which have an electronic integration components for sections that use high voltage values, for example transformers which raise household utility voltage up to 15 kV values used for air ionization to purified air; as a common practice physical dimensions to this transformers keep values around 2 cm3 or 3 cm3 of volume, designed and constructed with wires that have very little cross section area, sometimes equal or equivalent to a thin hair and in addition them are placed over print board circuits. is impossible to get discharge values or discharge measurements in components like this because the little size of it. how could we measure test methods results under specification with these technical design conditions?. we think that the iec 60335-2-65 subclauses 8.1.4 and 16.101 don't looks forward to particular cases like described above. | In 8.1.4, for voltages above 15kV, this discharge energy should be measured as the energy of the discharge through a 2000 (+/- 100) resistor. We are also of the opinion that this value should be specified for voltages 450V to 15kV. There is no relationship between 8.1.4 and 16.102. 8.1.4 is the test to decide whether an accessible part is a live part. If it is than the appliance is unsafe because the live parts are not permitted to be accessible. 16.102 is the test to ensure that there is adequate interturn insulation within both primary and secondary transformer windings. | None | 61(Rhodes/Chairman)1 | Mexico |
| 60335-1; 60335-2-65 | 8.1.4; 16.102 | We had established relationship between 8.1.4 and the relevant electric diagram on iec 60335-1, but we would like have a brief description of the way how is the test develop in this case and some recommendations about measurement equipment to discharge measure (we think that the right equipment could be an oscilloscope with memory or similar). why iec 60335-1 indicate discharge values results in microcoulombs and iec 60335-2-65 require it mJ? second one, about 16.102 of iec 60335-2-65, we agree that 16.102 is the test to ensure adequate interturn insulation within both, primary and secondary, transformer windings, but we return at the original inquiry: how to make measurements in primary and secondary transformer windings with volumetric dimensions of 1 cm3 (core including) which are constructed with wires so thin than a hair, and located over print board circuits | If it is not possible to do it by calculation, it is normal to use a storage oscilloscope with pre-trigger data storage, so that the scope can be triggered on single shot just prior to triggering the discharge. As is clearly stated in the standard, C are measured for voltages in the range 450 V – 15 000 V and mJ are measured for voltages > 15 000 V. The reason for this is explained in IEC 60479-2. Any breakdown during the double induced voltage test on a transformer will be obvious due to the sudden increase in input current and accompanying “bang”. For such a small transformer as is described it will probably be destroyed. | None | 61(Rhodes/Chairman)1 | Mexico |
| 60335-2-65 | 32 | In IEC 60335-2-65 clause 32 NOTE 101 "If the instructions state that the appliance is to be fixed in a room having a volume exceeding 30 m3, the dimensions of the test room are increased accordingly." Could you please explain about increasing of the dimension of the test room? Does it increases only height of the test room or all dimension (width, length and height)? | It is your choice on how to increase the volume - increase whichever linear dimensions are most convenient. | None | 61(Cape Town/Chairman)2a | Thailand |