| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-3; 60335-2-44; 60335-2-51; 60335-2-85 |  | The last years there are coming on the market various kinds of steam generators for household use, for which the accessories like electric iron accessories, floor treatment accessories, sprayguns for cleaning, fabric treatment accessories for curtains, carpets, upholstery etc. are sold separately or at the same time with the mainsteam unit. It is not clear to the Israeli committee under which part of IEC 335 they should be tested. | There is no part 2 standard for separate steam generating units. Requirements for appliances that use steam are covered by the specific parts 2, for example: IEC 60335-2-3 (for irons) IEC 60335-2-44 (for ironers) IEC 60335-2-51 (for spray guns for cleaning - see Amendment 1) IEC 60335-2-85 (for fabric steamers) This response will form part of the Chairman's Report to the next meeting of TC 61 which is to be held in Bangkok 22-26 May 2000. If TC 61 requires any modification to this response, you will be informed shortly after 26 May. | None | 61(Bangkok/Chairman)1 | Israel |
| 60335-2-85 | 13.2 | During adopting the text of the IEC 60335-2-85 1997 ’particular requirement for fabric steamers ’ , I have a problem with understanding item 13.2 of this standard, i.e. ”For electrode-type appliances and appliances having bare heating elements, the leakage current is also measured between a metallic mesh placed in the steam, 10mm from the outlet, and accessible metal parts.” The text is a little confusing to me, and I wonder if it can be understood in the following way. “For electrode-type appliances and appliances having bare heating elements, the leakage current is also measured between any pole of the supply and accessible metal parts connected to metallic mesh placed in the steam, 10mm from the outlet.” | Your understanding is not correct. The leakage current experienced by the user is best measured by the method described in the standard. The circuit back to the supply is via the metallic mesh and the steam path. | None | 61(Durban/Chairman)2a | Hong Kong |