| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-5 | 20.101 | 20.101 The appliance is placed on a horizontal surface and a mass of 23 kg is placed on, or suspended from, the centre of the open door or any fully opened drawer, whichever is more unfavourable. Any castors are turned to the most unfavourable position. For appliances normally used on a table or similar support and incorporating a door having horizontal hinges and a horizontal rest position, a mass of 7 kg is used instead of a mass of 23 kg. Appliances normally used on a table or similar support and which have a drawer are additionally tested with the drawer placed in the most unfavouorable position and loaded with the maximum number of place settings in accordance with the instructions. If a dishwasher is combined with a hob, the test is carried out with the appliance loaded as specified in IEC 60436, the point of application of the load being at the centre of the outer edge of the open door or drawer. To avoid any interpretation in the fourth paragraph, we propose to modify the text in the same way as the first paragraph, to read as follows: “the point of application of the mass being at the centre of the outer edge of the open door or fully opened drawers, whichever is more unfavourable.” | It was agreed to change the word “load” to “mass” before the FDIS is released for voting. The Central Office will be notified. | None | 61/2093/RM |  |
| 60335-2-5 | Annex BB | 61(Secretariat)837 and 61(London/Secretariat)9 | Compilation of comments prepared for this document, 61(London/Secretariat)9, was reviewed. After discussion, it was decided that the interpretation as queried by the British National Committee on Annex BB of IEC 335-2-5 was as follows: 1. No change is to be made to the full immersion test at this time. 2. Only the hose material is checked by the tests of Annex BB, not the couplings or other connections | None | 1994-11-London | Secretariat |