| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-13 |  | Could you help me to answer the question please? Frying pan. (IEC 60335-2-13) Please see the attached files, picture 1. (see attachment 61/4080ea/RM – Thailand 3 – Picture 1) How to consider “protection against access to live part”? Position of the appliance in normal use, test finger can‟t touch live part. (pictures 2 and 3) (see attachment 61/4080ea/RM – Thailand 3 – Pictures 2 & 3) But in the other positions – tilt angle 10 to 15 or overturn (picture 4), test finger can touch live part. (see attachment 61/4080ea/RM – Thailand 3 – Picture 4) | Clause 8.1 of IEC 60335-1 requires that “Appliances shall be constructed and enclosed so that there is adequate protection against accidental contact with live parts.” The third paragraph of the test specification in 8.1.1 of IEC 60335-1 states (in part) “Test probe B of IEC 61032 is applied with a force not exceeding 1 N, the appliance being in every possible position except that appliances normally used on the floor and having a mass exceeding 40 kg are not tilted”. Picture 4 in your inquiry would indicate that you have carried out the test as detailed in the third paragraph of the test specification in 8.1.1 of IEC 60335-1. If during this test the test probe contacts live parts, then this is not in compliance with the fourth paragraph of the test specification in 8.1.1 of IEC 60335-1 that states “It shall not be possible to touch live parts or live parts protected only by lacquer, enamel, ordinary paper, cotton, oxide film, beads, or sealing compound except self-hardening resins, with the probe.” Thailand 3: Confirmed. | 61/40/0ea/RM – Thailand 3 – Picture 1; 61/4080ea/RM – Thailand 3 – Pictures 2 & 3; 61/4080ea/RM – Thailand 3 – Picture 4 | 61/4080ea/RM | Thailand |
| 60335-2-13 | 19.101 | According to 19.101 stated that “Deep fat fryers incorporation a thermal cut-out of the capillary type are tested as specified in 19.4 but with the capillary tube ruptured”. I tested clause 19.101 by change position of thermostat (render inoperative thermostat) and break the capillary tube of thermal cut-out. The heater of deep fat fryer operated all the time and after 30 min, oil ignited. Base on IEC 60335-1, clause 19.1. The testing is simulated only one fault at a time. But clause 19.101 of IEC 60335-2-13 is double fault testing. (Short thermostat and break capillary tube thermal cut-out) I’m not sure that which one is right (1 or 2) for clause 19.101. 1. One fault. - Thermostat operate - Break capillary tube thermal cut-out 2. Double fault. - Short thermostat or render inoperative thermostat and - Break capillary tube thermal cut-out If 2 is right. Why standard test double fault with thermal cut-out capillary type? | The second option is correct. It was agreed at the meeting in Bali that the thermal cut-outs of the capillary tube type must comply with IEC 60730-2-9 for a type 2K action – this will overcome the “double fault situation” and if approved this will not be part of the standard until A1 of IEC 60335-1 Ed 5 is published in about 2 years. Thailand 1: Confirmed with minor change of putting quotation marks around “double fault situation.” | None | 61/4285/R M | Thailand |

Attachment 61/4080ea/RM – Thailand 3 – Picture 1:

Attachment 61/4080ea/RM – Thailand 3 – Pictures 2 & 3:

Attachment 61/4080ea/RM – Thailand 3 – Picture 4: