| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-1; 60335-2-30 |  | I have been asked (by the Australia/New Zealand Safety Authorities) which standards apply to the following appliances: 1) A water sterilizer - one type that I have seen pumps water through a tube where it is exposed to UV radiation from a UV lamp. These are used to sterilize aquarium water and also drinking water from rain water collection tanks. They sit external to the water tank and have inlet & outlet hoses. I also know that there other types on the market or that maybe coming on the market that operate on a different principle. If we cannot identify a suitable standard - do we need a new part 2 ? 2) Which standard should apply to a "3 in 1" fan, room heater, luminaire and ”2 in 1” room heater, luminaire (see picture) – these are typically used in a bathroom recess mounted in the ceiling. The room heater aspect is provided (typically) by 275 W – 375 W IR lamps. | 1.. Some members of the CAG considered that a new part 2 is required, while others considered that for UV types additional requirements should be included in part 1. 2. For the heater we should apply part 2-30 (heaters for mounting at high level) but this will need a change as per part 2-71 concerning 8.1.1, 21.1, 22.39 and possibly some other instructional requirements. Also note that basic insulation is accessible while replacing the heater lamp (this is allowed in 60598 but not allowed in part 2-71). | 61(Cape Town/Chairman)2a – Australia/New Zealand 1 | 61(Cape Town/Chairman)2a | Australia/New Zealand |
| 60335-2-30 60335-2-43 |  | Let us take the example of a room heater, equipped with an electronic control and containing a thermostat. During normal operation, the electronic control continuously monitors the room ambient temperature and compares this to a pre-set value. When this value is reached, the electronic control operates and the heat source is switched OFF. At some later time, the room temperature will have fallen sufficiently for the control to operate once more, this time switching the heat source ON. Our contention is that the OFF period is part of the operating cycle of the appliance, and is not to be considered a ‘standby’ mode, as the equipment is fully operational during this time. Some test houses are regarding this OFF period as a ‘standby’ mode, as referred to in Annex Q, which is clearly not applicable in this case. We therefore believe that a clarification of ‘standby’ mode is required, either in the Part 1, or the relevant individual Part 2’s. This might take the form of an additional note, to the effect that a thermostat is not a switching device to set a heating appliance into a ‘standby’ mode | (NOTE : The Highlighted text and stuck through text are the changes recommended by MT23) In response to your question: The cycling of the thermostat is not a standby mode. It is normal operation. Clause 19.13 states (in part) that Appliances tested with an electronic switch in the off position, or in the stand-by mode, shall – not become operational, or – if they become operational, not result in a dangerous malfunction during or after the tests of 19.11.4. In the situation that you described when the room heater switches back on when the room temperature drops this is not dangerous. In the same way if it switches back on by EMP when the appliance is being used normally in my opinion this is not a dangerous malfunction consequentially there is no need to even do the 19.11.4 tests. The second paragraph of 5.3 in part 1 can be applied in this case. You need to look at also in part 1: 3.8.2 and 7.10. However, please note that in room heaters 22.109 states "The disconnection of the supply by a switch in the off position shall not rely on electronic components." This is fulfilled provided that the temperature controller does not have a position marked "O" or "Off" or similar. I see no need to change the standards at this time. Ireland – Confirmed, as modified by MT23. | None | 61/3355/RM | Ireland |
| 60335-2-30 | 5 | Testing a series of stationary radiant room heaters of different size, is it correct to test the heater with the lowest and highest input and the one with highest input/m²? If the power input/ m² is the same for all in a series, is it correct to test the one with highest input only? | Yes, both statements are correct, but it is necessary to evaluate construction and components used to determine the correct family representative. The evaluation shall be made by the body A and it shall be explained in the test report 41c) Certification issue, not a matter for TC61 | None | 61/3508/RM | CTL Inquiry - PDSH 629 |
| 60335-2-30 | 5.6 | Q1. According to the standard, for the heating test in the normal operation like the temperature rise and the test corner, shall the thermostat be short-circuited or not? According to the definition of normal operation in the standard, the thermostat shall not be short-circuited yet, to our understanding. The thermostat is a must safety device to control the temperature. If the thermostat is SC, the temperature cannot be below 85K – please kindly clarify. Q2. In the “5.6 addition, IEC 60335-2-30”, “Sensitive” is the key word. The thermostat in the heater is not sensitative to the room air temperature (why not ambient temperature used here?) at all and not like the one fixed on an air-conditioner, which is really sensitive to and can almost exactly tell the room air temperature. It is not located in the inlet either. So the thermostat is not short-circuited. Is such an interpretation correct? – please kindly clarify. We have done consultations on this issue with different notified bodies like TUV Rheinland, SGS, ITS and STR, but surprised that there are two answers. TUV Rheinland, ITS and STR say the thermostat shall not be short-circuited in the heating test of the normal operation, except for Australia, but SGS says that the thermostat must be short-circuited in the heating test of normal operation. So which shall be correct? – please kindly clarify. Please kindly refer to the construction of the oil-filled radiators as attached below, in which six parts are marked. To our knowledge, in the EU market now, there are both heater model 1 with parts (1)(2)(5)(6) and model 2 with parts (1)(2)(3)(4)(5)(6). But for Australia, one extra thermostat is required specially by Australia Electric Safety Bureau to be fixed either in place (3) or (4). The thermostat (3) or (4) is cycling but not adjustable, and the thermostat (2) is both cycling and adjustable. All are the safety devices to limit the temperature rise to be below 85K but can be affected by the airflow. And then there is (5)thermal link to stop heating when it feel a temperature of 157 degrees in the case that (2) or (3)/(4) doesn’t work properly. But there are different interpretations to the standard on those two models from different notified bodies: For model 1, notified body like TUV Rheinland and STR issue CE/GS to manufacturers under the condition that the thermostat (2) shall not be short-circuited according to their 2nderstanding to the standard, but SGS thinks thermostat (2) shall be short-circuited. So model 2 will be the only choice for SGS. But TUV Rheinland thinks model 2 is up to the buyer’s requirement or local requirement like Australia for double safety. For such a case, we write to your to see whether there is a final answer to end this I. 1. which model shall be applied to the EU market? Or are both OK?; 2. what is the technical reason that the thermostat must be short-circuited? | Q1: It is difficult to answer your specific question without knowing the details of the heater construction to which the questions apply. But a general answer to your questions are: 1) According to the general conditions of the tests unless otherwise specified all tests are carried out by applying the conditions detailed in clause 5. So when carrying out the test of clause 11, according to 5.6 (taking 60335-1 and 60335-2-30 together) Thermostats sensitive to the room air temperature, such as those having a sensing element located in the air inlet, are short-circuited. However, if the thermostat can be set so that it does not cycle, it is not short-circuited but is set so that it does not cycle. All other types of thermostats are adjusted to their most unfavourable setting, if the setting can be altered by the user. 2) The intent of the standard is to render inoperative all thermostats that operate according to the air temperature or are influenced even partly by the air temperature. Consequently if the sensing element is situated anywhere in the air stream (such as is the normal situation in the type of heater that you have described) then the thermostat should in general always be rendered inoperative. If the thermostat sensing element is directly measuring, for example, the accessible surface temperatures or liquid temperatures (in say oil filled heaters) then they are not rendered inoperative. As the parts that they are directly measuring are parts that have safety limits applied. In this case they are adjusted to their most unfavourable setting, if the setting can be altered by the user. Q2: From what I can see in the photo of the design you have submitted it appears that item 2 is sensitive to the room air temperature and so must be shorted as a general condition of the tests according to IEC 60335-2-30 – I cannot see the sensing element for 3 or 4 so cannot say whether they should be shorted or not. Denmark 1 - Confirmed | 61(Helsinki/Chairman)02a- Denmark 1 | 61/3366/RM | Denmark |
| 60335-2-30 | 11.8 | According to Cl. 11.8 of IEC 60335-2-30/2002, “Temperature rises of surfaces of heaters shall not exceeding values in Table 101 “ and using test rod with (75 mm in diameter, of unrestricted length with hemispherical end) Q1: Is there any consideration to take into account to determine these surfaces except the use of the test rod? Q2: Is there are minimum distance between the heating source and the heating surface to take in account during the measuring? For example , the heaters with metal surfaces above the heating source (See Photo1), can we measure the temperature on these metal surfaces ? We have under test room heater(See photo 2) with 2-parts one works using gas, the other using electrical supply(1800W quartz heaters). Q3: Can we measure the temperature rise of the metal surface above the heating source directly knowing that the test rod can touch these parts. | The answers to the questions are: Q1: No – however, the part you have identified may be within the boundary of the immediate surround of the fireguard (see definition 3.104 and 3.105) and if so no limit applies to this part. Limits do apply to parts outside the immediate surrounds of the fireguard Q2: No Q3: Yes – but see answer to Q1 you need to check if the surface identified is within the immediate surround of the fireguard (see definition 3.105) or what looks like an air outlet grille just above the arrow head. Of course when conducting the test of clause 11 the gas part of the heater must be in operation (see clause 5.4 of IEC 60335-1) if possible. Jordan 2: Confirmed | 61(Rimini/Chairman)02a – Jordan 2 | 61/4285/RM | Jordan |
| 60335-2-30 | 19.103 | Clause 19.103 of IEC 60335-2-30 Ed 4 states in part: The rear surface of the heater is completely covered with strips down to the floor if – the heater is constructed to stand away from the wall; – for fixed heaters, the gap between the heater and the wall exceeds 30 mm, and the horizontal distance  between any two fixing points or spaces exceeds 200 mm, or  between any fixing point or spaces and the end of heater exceeds 100 mm, otherwise the rear surface is covered to a distance of approximately one-fifth of the height of the heater from the top. Assuming the distance to the wall is designed as equal or less than 30 mm a covering with 30 mm strips is then not possible. Our committee is therefore consequently of the opinion that the felt strip will not be pushed into a gap to the wall of 30 mm or less depth. The top rear surface of other heaters is covered to a distance of approximately one -fifth of the height of the heater. However in which way is the covering of "other heaters" done: - if the distance to the wall is equal to or less than 30 mm - or/and the horizontal distance between spacers or fixing points is less than 100 mm - or/and the heaters have fixing points or spacers with a vertical distance to the top of less than one-fifth of the height of the heater. | 1) The felt strip plus the copper disk plus the textile material gives a thickness of a little over 26 mm. By application of the second paragraph of 5.3 of Part 1, the opinion expressed (the felt strip will not be pushed into a gap) could be accepted for distances less than 30 mm. 2) If the heaters are such that the horizontal distance between spacers or fixing points is less than 100 mm then subject to the answer 1) the rear surface of the heater is covered down to the floor at other points where the felt strips can fit and subject to answer 3) are covered to a distance of approximately one-fifth of the height of the heater where the felt strips are too wide. .3) If the heaters have fixing points or spacers with a vertical distance to the top of less than one-fifth of the height of the heater then subject to the answers 1) and 2) the rear surface of the heater is covered from the top for a distance to the fixing points at the fixing points and down to the floor at other points The paragraph stating “The top rear surface of other heaters is covered to a distance of approximately one-fifth of the height of the heater. ” Can probably be deleted as it seems to be redundant text. Germany 1: Confirmed. Also, convert response to an interpretation sheet. | None | 61/4212e/RM | Germany |
| 60335-2-30 | 19.109; 19.110 | Fan assisted visibly glowing radiant heaters | 1. This type of heater was not considered by the committee when the requirements in IEC 60335-2-30 were written. 2. Subclause 19.109 should not be applicable, to the type of heater that you describe "if the production of fan assisted convected heat is associated with and dependent on the operation of the radiant heat section". 3. Consideration may be given to amending the standard to ensure that the type of heater that you describe can pass a modified 19.110 test. A test having a 150 K temperature rise limit on the test corner wall with the heater at a distance of 250 mm has been suggested. 4. This response will be submitted to the TC 61 for ratification at its meeting to be held in Bangkok 22 – 26 May 2000. If ratified, it will result in an amendment arising from point 2 and possibly an amendment arising from point 3. | None | 61(Bangkok/Chairman)1 | Australia |
| 60335-2-30 | 20.1 | I have been requested to write to you to find out if there is a reason why stationary appliances over 18 kg under IEC 60335-2-30 clause 20.1 are not subject to a stability test. | The reason for exempting stationary room heaters over 18 kg from the stability test has been lost in the mists of time. According to my records it has been that way since at least the first edition of part 2-30 (1979). However, I wonder if stationary room heaters over 18 kg actually exist in the market or are they theoretical appliances. If they do exist, they would be difficult to move around and would probably be installed once and never moved again, hence stability would not be an issue. | None | 61(Jeju/Chairman)2a | United Kingdom |
| 60335-2-30 | 22.109 | There is a question about Cl. 22.109 of IEC/EN 60335-2-30: The disconnection of the supply by a switch in the off position shall not rely on electronic components. If we have such switch: Our switch (push button) is for the modes (stop, comfort, economical (3, 5°C under comfort) , no frost) it is not general switch, this electronic switch (push button) is not the general switch because when you are on stop position if the temperature of room is between + 3 to + 7°C the radiator heat. When the radiator has not mechanical general switch (to cut the line and neutral), the complementary test on the amendment of NF EN 60335-1/A1 must be made. Can this switch pass this clause of IEC/EN 60335-2-30? Fixing the different temperatures : 1. Fixing the comfort temperature : The comfort temperature is the temperature that you would like while the room is occupied a) Push button (1) – First press, you are in comfort mode. The mode indicator light (2) is green continue. b) Set the control knob on half (+/- 23°C), the heating indicator light (3) comes on (red) if the temperature is below the required temperature. c) Wait a few hours for the temperature to stabilize. d) If the setting is satisfactory (if necessary use a thermometer to check), mark the position for future use. If the stting is not satisfactory, adjust it and start from point c. 2. Fixing the economical temperature : This is the required temperature during periods when the room unoccupied. It is recommended threat this mode should be used if the room is unoccupied for more the 4 hours a) Push button (1) – Second press, you are in economical mode. The mode indicator light (2) is green blinking at 50% b) If you don‟t change the position of control knob, the temperature in economical is 3.5°C under the comfort temperature. c) It is possible to adjust the temperature with the control knob in this mode but when you come back at the comfort temperature, the temperature will be the economical temperature more 3.5°C. 3. No Frost mode : This mode is used to keep the temperature at between 3 to 7°C in the room when you are absent from the house for a prolonged period (i.e. more than 24 hours). a) Push button (1) – Third press, you are in no frost mode. The mode indicator light (2) is green blinking at 25% 4.Stop a) Push button (1) – The fourth press, you are in stop mode. The mode indicator light (2) and Heating indicator light(1) go out NTC defect: On the thermostat, there is a NTC sensor. If this sensor is broken, the radiator heat at 50% and the heating indicator light red (3) is blinking. | It is not within the scope of IEC/TC 61 to state whether a particular design complies with the standard, this is the task for certification bodies and you should submit your design to one of these organizations for an opinion. However, it appears from the information that you provided that the stop position gives an off position via an electronic disconnection and this is not permitted by IEC 60335-2-30 clause 22.109 irrespective of compliance with the tests in Clause 19 China 3: Confirmed | 61/4080ea/RM – China 3 | 61/4080ea/RM | China |
| 60335-2-30 | 22.109 | we kindly ask you to explain to us how following Clause of IEC/EN 60335-2-30/2002 has to be interpretated: 22.109 The disconnection of the supply by a switch in the off position shall not rely on electronic components. As the previous Clauses 7.12 Text: …. The instructions for visibly glowing radiant heaters, other than heaters for mounting at high level, shall include the substance of the following: “Do not use this heater with a programmer, timer or any other device that switches the heater on automatically, since a fire risk exists if the heater is covered or positioned incorrectly” ……… ……. and 22.108 Text: Visibly glowing radiant heaters, other than heaters for mounting at high level, shall not incorporate thermostats, timers or similar means which switch on heating elements automatically, unless at least one heating element is already visibly glowing refer exclusively to Visibly Glowing Radiant Heaters, other than heaters for mounting at high level, we wonder if the following Clause 22.109 shall be intended as a general requirement for all heaters or only for Visibly Glowing Radiant Heaters, other than heaters for mounting at high level. | The requirement in 22.109 applies to all heaters because it must always be possible to switch a heater off. Switching them off cannot rely on the correct functioning of an electronic component. This has been a requirement for many years i.e since the publication of the third edition in 1996. It has nothing to do with and is not related to the instruction in 7.12 or the requirement in 22.108. The instruction in 7.12 and the requirement in 22.108 do not apply to heaters for mounting at high level as it is unlikley that such a heater will be inadvertently covered or positioned by a third person when it is set to come on automatically. | None | 61(Kuala Lumpur/Chairman)2a | Italy |

Attachment 61(Helsinki/Chairman)02a- Denmark 1:

Attachment 61/4080ea/RM – China 3:

Oil radiators heater control panel:

Attachment 61(Rimini/Chairman)02a – Jordan 2: