| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-2 |  | I've recently received an interesting vacuum cleaner from a customer. The vacuum cleaner has a PTC heating element for sterilizing blankets or duvets. The PTC heating element is covered with metal plate. And the metal plate is exposed to the outside. I think the metal plate is dangerous because it's temperature can be over 100oC. But I can't explain why it is dangerous by the standards (IEC60335-1, IEC60335-2-2). Could you tell me how I can apply the standards? | This type of appliance is not yet standardized but as an interim measure, IEC 60335-2-2 should be applied with the vibrator and heating plate operated as in normal use according to the manufacturer’s instructions. During the clause 19 tests the temperature rise of the heating plate should not exceed 150 K. Korea 1: Confirmed | None | 61/4212e/RM | Korea |
| 60335-2-2 |  | 2) Are rechargeable vacuum cleaners, which are being charged with 12V DC output of vehicles, in the scope of IEC 60335-2-2 ? If not which IEC publication is applicable for them? | 2) This type of vacuum cleaner is within the scope of IEC 60335-2-2. They are tested as class III appliances, taking into account Annex B of part 1. Please note that in some countries (for example those within the EU) additional requirements apply to appliances intended for use in automobiles. 41b) Iran 2 - Confirmed | 61(Penang/Chairman)02a – Iran 2 | 61/3508/RM | Iran |
| 60335-2-2 | 10 | Regarding to power input test of vacuum cleaner in IEC 60335-1 and IEC 60335-2-2, clause 10. the deviation of power input was ± 15% in Table 1 But in the successive issue (2002) the –ve deviation is not mentioned and there was a note that says no –ve deviation exist . The latest issue (2009) does not include neither the –ve deviation nor the note . This case causes confusion in testing bodies, for example when a manufacture writes 1800 W as the rated input power in the main plate of a vacuum cleaner. and when this vacuum cleaner is tested and the measured power input is found to be 1000 W. In this case is this vacuum cleaner accepted or rejected?? | There is no negative deviation limit specified in the standard – so the result is of course acceptable. For your information a negative deviation for motor-operated appliances has never been specified in IEC 60335-1. The negative deviation for vacuum cleaners was removed when the fourth edition was published in 1993 since it was in conflict with IEC 60335-1 and there is no safety issue involved with negative deviations for motor-operated appliances. | None | 61/4080ea/RM | Egypt |
| 60335-2-2 | 20.1 | The questions are regarding an issue discussed on an earlier meeting – stability requirements on hand-held “standing” vacuum cleaners. The previous discussion reported in the minutes didn´t precise all types of interests. Electrolux Floor Care and Small Appliances are manufacturing several types of vacuum cleaners for a worldwide market. The different types of vacuum cleaners, mainly stick type vacuum cleaners, has now developed features that we think should be clarified in clause 20.1 of IEC 60335-1/60335-2-2. IEC 60335-1 requires stability test for portable and stationary appliances and we now seek clarification whether following products shall be classified as hand held or portable (or both?) depending on the normal use mode or parking/storage/charging mode. Case A hand held battery operated vacuum cleaner, weight less than 2kg, height less than 50cm: A battery operated hand held cleaner is hand held during normal use (cleaning mode) but can also be equipped with a table stand charger/parking stand. Shall this product pass the stability requirement when it is placed in its parking/charging stand? Case B hand held battery operated stick cleaner, weight less than 3kg, height 100-120cm A battery operated stick type vacuum cleaner is hand held during normal use (cleaning mode). b1) it can be equipped with a temporarily parking mode where the cleaner can stand upright without the help of the user (balancing on the nozzle) b2) it can be equipped with an external parking stand (for long time storage), where it can stand upright without the help of the user. b3) it can be placed in charging mode in either option b1 or b2. Charging time can be 1-16h Shall this product pass the stability requirement in when: - It is placed in temporarily parking mode (b1)? - It is placed in it external parking stand (b2) for long time storage? - It is being charged in either b1 or b2 position? Case C corded stick cleaner, weight more than 3kg, height 110-130cm The corded stick type vacuum cleaner is hand held during normal use (cleaning mode) c1) It can be equipped with a parking mode where the cleaner can stand upright without the help of the user (balancing on the nozzle) c2) It can be equipped with an external parking stand, where it can stand upright without the help of the user. Shall this product pass the stability requirement when: - it is placed in its short time parking mode (c1)? - it is placed in its long time external storage/parking stand (c2)? | This response is based on the standard as currently written and takes into account the decision related to the Chairman‟s report inquiries as recorded in 61/3927/RM item 25b “China 4”. Battery operated appliances where the battery is charged within the appliance are covered by Annex B in the 60335 series, the complete appliance (during charging) being the charging base plus the hand-held part. Charging of the battery in the hand-held part is considered normal use. If the charging base is fixed then the stability test is not applicable during charging of the hand-held part since the complete appliance in this mode of operation is considered a fixed appliance. So based on the above considerations the answers to your questions are: Case A: Yes because the charging base is not fixed Case B b1: Yes if the stand is a charging base and it is not fixed, no if it is just a stand b2: Yes if the stand is a charging base and it is not fixed, no if it is just a stand b3: See b1 and b2 Case C c1: Yes (aligns with answer to China 4 from Tel Aviv) c2: Because of the answer to c1, to test in this mode (based on the construction in the photos) is not necessary because it is tested in the c1 mode which will probably be a more onerous condition than c2. As is usual your inquiry and these draft answers will be included in my Chairman‟s report to the TC 61 meeting in Bangkok 17 – 21 May 2010. If TC 61 requires any modification to this draft response you shall be informed after the meeting. Sweden 1: Confirmed | 61/4042e/RM – Sweden 1 | 61/4042e/RM | Sweden |

Attachment 61/4042e/RM – Sweden 1:

Case A

Case B

b1

b2

Case C

c1

c2	2