| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-98; 60335-2-101 | Scope; 3.101 | I as an Expert Working in research in Institute of Standard and Industrial Research of Iran (ISIRI, www.isiri.org), want to use the IEC 60335-2-98 standards for testing and proving safety certificate of some vaporizers on March 09 2004. In fact, I'm going to select one of these two standards: . IEC 60335-2-98 . IEC 60335-2-101 I'd be very grateful if you provide me with ample information. | Scope of IEC 60335-2-98 This International Standard deals with the safety of electric humidifiers for household and similar use, their rated voltage being not more than 250 V for single-phase appliances and 480 V for other appliances. NOTE 101 Examples of appliances that are within the scope of the standard are – appliances that atomize water; – appliances that evaporate water by heating; – appliances that blow air through a moist element. Scope of IEC 60335-2-101 This International Standard deals with the safety of electric vaporizers for household and similar purposes, their rated voltage being not more than 250 V. NOTE 101 Examples of appliances that are within the scope of this standard are air fresheners and insect repellers. They may be plugged directly into a socket-outlet or connected to the supply mains by means of a flexible cord and plug. - NOTE 103 This standard does not apply to – appliances intended exclusively for industrial purposes; – appliances intended to be used in locations where special conditions prevail, such as the presence of corrosive or explosive atmosphere (dust, vapour or gas); – appliances for heating liquids (IEC 60335-2-15); – insect killers (IEC 60335-2-59); – fabric steamers (IEC 60335-2-85); – humidifiers (IEC 60335-2-98). 3.101 vaporizer appliance incorporating a heater for vaporizing a medium in order to affect the air in a room | None | 61(Kuala Lumpur/Chairman)2a | Iran |