| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60384-14 |  | I appreciate to receive your feed back about the following request: - must the capacitor C1 of the attached wiring diagram (extended to all similar application) be class X2, standard IEC 60384-14? I point out that the resistor R1 is a fuse device. | In my opinion it doesn't need to be X2. The resistor R1 can be considered an intentionally weak part. Also I think it could be argued that the capacitor is an energy limiter rather than a voltage divider. It is also not connected directly to the mains. Italy 2: Confirmed | 61/BALI/cHAIRMAN)02A- Italy 2 | 61/4212e/RM | Italy |
| 60384-14 | 3.4 | I have a question concerning the IECQ certification of Y1 capacitors. The back ground and my question is as follows: IEC 61140 requires that protective impedance comprise two components. However it allows one component if it is IECQ certified. This standard is the basic safety publication concerning protection against electric shock The standard for capacitors used as protective impedance is IEC 60384-14 (QC 302400). Clause 3.4 “Approval testing” of IEC 60384-14 has two alternatives. 3.4.1 “Safety tests for approval” 3.4.2 “Qualification approval” Which alternative has to be followed for IECQ certification. | I was informed verbally by Chris Agius, Executive Secretary, that for IECQ certification of Y1 capacitors to IEC 60384-14 (QC 302400) 3.4.2 “Qualification approval” was applicable rather than 3.4.1 “Safety tests for approval”. See also January/February 2010 E-Tech article IECQ stresses importance of "green" products TC 61 Chairman 2: Confirmed | None | 61/4042e/RM | TC Chairman |

- Attachement 61/BALI/cHAIRMAN)02A- Italy 2: