| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-7; 60335-1; |  | A twin tub washing machine. Where the supply cord is including 3-conductors which are the Line, Natural and Earth conductors. The problem is that the earthing conductor is floating inside the appliance and according to its design the earthing conductor shall be connected to the washing and spinner motors external enclosures which are not accessible. Knowing that according to the design, the external parts of the appliance doesn’t need to be connected to the earth since most of the external parts are plastic and the accessible metal parts are only screws for fixing purposes which doesn’t need to be earthed - First what is the classification for this appliance? Its class I appliance? - What about the floating earthing conductor? Doesn’t cause any clause of the IEC 60335-1, and IEC 60335-2-7 not to be fulfill? Knowing that the floating earthing conductor can touch the insulation of the internal wiring like Line Natural wires, plastic parts and the external enclosure of the spinner motor. I mean by floating that one end of the earthing conductor is not connected to the earthing terminal(not fixed in any position) floating on the air inside the appliance but when its move can’t touch any live parts, only plastic parts and parts which should be earthed. This is Class I appliance which have accessible metallic parts with only class II construction. | Without actually seeing the appliance these questions are difficult to answer. However, if it is a class II appliance then it must be marked with the double square symbol and it must not be provided with two core plus earth (three core) flexible cord. If it is not marked with then it is a class I appliance and must fulfil all of the requirements of the standard that are applicable to class I appliances. | 61(Rimini/Chairman)02a – Jordan 1 | 61/4285/RM | Jordan |
| 60335-2-7 | 2.2.9 | Asked if the addition to 2.2.9 of IEC 60335-2-7:1993 as introduced by amendment 1 meant that the rated clothes load should also be reduced 50 %. | No. | None | 61(Chicago/Chairman)1 | China |
| 60335-2-7 | 3.1.9 | We are a Lab in Iran that working on safety of some household electrical appliance according to IEC 60335-1 and its particular requirements standards. in IEC 60335-2-7 standard about washing machine we have some obscurity in definition of normal operation specially about temperature of inlet water. In fact we want to know the direct effects of water temp in safety tests. In other words if we use a water with difference temp like water mains (pipe) whether all the safety tests will be invalid or not. | We confirm that for the test result to be valid according to the standard, the temperature of the inlet water must be as specified in 3.1.9. Iran 1: Confirmed | None | 61/4212e/RM | Iran |
| 60335-2-7 | 11.7 | According to sub-clause 11.7 of part 2-7, appliances incorporating a programmer are operated with the program that results in the highest temperature rise. Sub-clause 3.1.9 requires the use of 50% of the load if this gives higher power input or current. Some new fully electronic programmers of washing machines automatically perform different programs when half a load is used even if the program selected is the same used for full load. The difference between the programs, automatically selected by the appliance, do not allows to comply the requirements of 11.7 and 3.1.9 at the same time (worse load and most unfavourable program) in such a way that using the two possibilities the limits may be exceeded in one case and not in the other. The question is how to perform clause 11 in this case. | If the power input or current is higher when only 50 % of the textile material is used, the appliance is operated with this load instead if this gives more unfavourable conditions than the full load during the test of Clause 11. 41c) Agreed, also refer to 61/3358/CDV - Ed 7 of IEC 60335-2-7, this requires the most onerous condition to be applied | None | 61/3508/RM | CTL Inquiry - PDSH 628 |
| 60335-1; 60335-2-7 | 19.13, Tables 16, 17. 18; 29.2 | A motor thermal protector manufacturer indicated to UL that they are experiencing differing national interpretations of IEC60335-1 (4th edition) and -2-7 with respect to clearance and creepage requirements. We request clarification of the creepage and clearance distance requirements that apply to thermal motor protectors in motor operated washing machines. A typical motor thermal protector construction is illustrated. The device is placed within a mylar sleeve to insulate the live case from motor windings. An alternative version of the device has leads entering and exiting from opposite ends. When the sleeved protector is in direct contact with pump motor windings, clearance and creepage distances of interest are those across the functional insulation (contacts open), the electrically live case / terminations and windings, and the electrically live case / terminations and accessible parts of the appliance. The required distances are determined by test (per Clause 19.13 of Part 1 for functional insulation) and application of Tables 16, 17 and 18 of Part 1, as appropriate. The required creepage distances are dependant on the pollution degree of the environment in question. Clause 29.2 of Part 2-7 indicates that the microenvironment is pollution degree 3 unless the insulation is enclosed or located so that it is unlikely to be exposed to pollution during normal use of the washing machine.Our questions are as follows: - Does an open-end mylar sleeve afford adequate protection from exposure to pollution such that pollution degree 2 distances are applicable? If it extends beyond the physical dimensions of the thermal protector (e.g. 3.2 mm)? - Does Note 1 of Part 1 Clause 24.1 limit application of Tables 17 and 18 to distances between live and accessible parts of the appliance? - If the answer to question 2 is Yes, does this mean that measured clearance and creepage distances between the terminations of the thermal protector are not specified in IEC60335-1 but instead in IEC60730-2-2 (per Clause 24.1.4 of Part 1)? - If the answer to question 2 is No for either Table 17 or 18, does this mean that the table distance applies between the live thermal protector case / termination and motor winding? Between the terminations of the thermal protector? | - THIS WOULD APPEAR TO BE ACCEPTABLE IN THE SITUATION DESCRIBED. BUT IN GENERAL EACH CASE MUST BE TREATED ON ITS MERITS AND THE CIRCUMSTANCES IN WHICH THE PROTECTOR IS USED MUST BE INVESTIGATED FOR EACH CASE. FOR EXAMPLE SUCH AN OPEN ENDED SLEEVE WOULD NOT BE ACCEPTABLE IN THE PRESENCE OF CONDUCTIVE DUST SUCH AS MIGHT BE PRODUCED BY A MOTOR WITH BRUSHES. - YES FOR THE PROTECTOR ITSELF BUT NOT FOR OTHER PARTS OF A PROTECTOR CIRCUIT SUCH AS ON A PCB ASSOCIATED WITH THE PROTECTOR. - YES The Clause 19.13 test of IEC60335-1 would still apply. YES THIS APPLIES WITH THE PROTECTOR IN THE OPEN POSITION SEE AM 2 TO IEC 60335-1. | None | 61(Jeju/Chairman)2a | United States |
| 60335-2-7 | 20.104; 20.105 | - 20.104: the test to verify the compliance of the requirement states that the appliance is operated empty and is supplied at rated voltage. The force determined during the test of 22.104 with the lid interlocked is applied to the lid or door in an attempt to open it. It shall not be possible to open the lid or door while the drum speed exceeds 60 r/min. If the appliance is loaded from the front and the door can be opened, the motor shall be de-energized before the opening exceeds 50 mm. We do not understand the reason for allow the opening of the door to the front-loading machines. IT IS NOT ALLOWED TO OPEN THE DOOR WHILE THE DRUM SPEED EXCEEDS 60 R/MIN Although the motor is de-energized, the inertia of the drum will always leave the possibility of reach the drum rotating at more than 60 rpm. IN THIS CASE IT WILL FAIL THE TEST CRITERIA RELATING TO THE LIMIT ON THE DRUM SPEED BEFORE OPENING THE DOOR We also believe that the worst condition is when the drum is full. SEE ANSWER TO 20.105 - 20.105: The appliance is supplied at rated voltage and operated empty. A force not exceeding 50 N is applied to the lid or door in an attempt to open it, as in normal use. If the lid or door opens, the drum speed shall be no higher than 60 r/min within 7 s of opening the lid or door by 50 mm. In addition, if the appliance is loaded from the front, the motor shall become de-energized. As in the previous subclause, the appliance is operated empty. Why? IT IS NOT POSSIBLE TO GET A REPRODUCABLE TEST RESULT WITH THE DRUM LOADED. ALSO THERE HAVE BEEN NO REPORTED FIELD INCIDENTS WITH FULLY LOADED MACHINES THAT COMPLY WITH THE CURRENT CRITERIA OF THE TEST SPECIFICATION. Clearly, to set a time for detention is for evaluates the braking of the drum and the worst condition should be when the drum is filled. | The response is included with the questions – highlighted in YELLOW CAPITALS Argentina 1: Confirmed, with the following modification. Add at the end of the first paragraph “relating to the limit on the drum speed before opening the door.” | None | 61/4285/RM | Argentina |
| 60335-2-7 | 27.1 | Question: We have received the following product to be tested according to IEC 60335-2-7 Ed 7 Std. Drum is earthed through a ball bearing in accordance with DSH 649, however it was observed that during the water extraction, water might be in contact with the heating element but not with the drum, thus in case of a failure in the insulation of heating element, extracted water might be in contact with a live part. To solve this issue, manufacturer proposes to use an earthed non-ferrous pin (copper) at the outlet. Is this solution deemed to be in compliance with IEC 60335-2-7 Std? If so, how the measurement of Clauses 13 and 16 should be done? Rationale: IEC 60335-2-7 Standard does not establish certain conductivity to water or appropriate testing methods for leakage current measurement when water is used as conductor Proposal (if any): This construction is not acceptable since water cannot be considered a permanently and reliably mean of connection to an earthing terminal. | CTL: Most of the comments are :DSH is correct and it is clarified in the standard(No DSH is necessary) Confirm decision of CTL. Any NC is free to make a proposal to allow this type of construction. | 61(Seattle/CTL Liaison)02d – Tracking No. 0769 | 61/4080ea/RM | CTL Liason |
| 60335-2-7 | 29.2 | 1. In the clause 29.2 of IEC60335-2-7, it requires the insulation shall have a CTI not less than 250 unless the microenvironment is better than the pollution degree 3. My question is if this means all of the insulation in the washing machine must meet this requirements regardless of the working voltage ? 2. In the table 18, it says, I think this requirement is because it will be no hazard for the working voltage not exceed 50V, the CTI can be decreased. Right ? 3、Based on the two questions above, how to decide the CTI for the low voltage insulation in washing machine? Such as, what is the requirement of the CTI for the display board in washing machine, which only offer the functional insulation, and the working voltage can not exceed 50V? | Q1: YES unless it is protected from pollution during normal use of the appliance due to – condensation produced by the appliance; – chemicals, such as detergent or fabric conditioner Q2: The addition to Part 2-7 over-rides Table 18 in Part 1 for Material group IIIb and means that you cannot use IIIb material in pollution degree 3 irrespective of the working voltage. If the PCB is coated in according with 29.1 and Annex J and also 29.2 second paragraph first dash then obviously pollution degree 3 does not apply. Also the relaxation in 29.2.4 2nd paragraph concerning short circuiting of creepage distances is still applicable. Q3: See answers to Q1 and Q2. China 5: Confirmed | None | 61/4080ea/RM | China |

Attachment 61(Seattle/CTL Liaison)02d – Tracking No. 0769:

Fig. A

Fig. B

Ground pin (copper material) inside the filter

Fig. B1

Attachment 61(Rimini/Chairman)02a – Jordan 1: