| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 62115 | 5.1 | Following our corrrespondence forwarded below, I would like to thank you fi rst for your response, but the problem we feel is not why the temperature rises to that degree, as according to sub-clause 5-11, IEC 62115 and clause 9 and subclause 9-1 to 9-9, your are fully correct: In sub-clause 5-11, IEC 62115 it is said that "the tests are carried out in a draught-free location at the ambient temperature of 20 C +- 5C", but it is the general conditions for tests not test results. now as, The temperature rise of the surface of handles, knobs and other parts that are likely to be touched by hand shall not exceed the following values: 25 K, for parts of metal, 30 k, for parts of gas or porcelain, 35 k, for parts of plastice or wood and the temporature rise of other accessible parts of the toys shall not exceed the following valuse: 45 k, for parts of metal, 50 k, for parts of glass or porcelain, 55 k, for parts of plastic or wood. THE QUESTION IS THAT: - Don't you think the acceptable temperature rise assumed in above clauses is too much and can damage or burn children's skin? - With these acceptable temperature rises, if the temperature of an accessible metallic part of the toy under the test get to 65 C (338 k), according to sub-clause 9-3, it is acceptable and matches the requirements, though it burns the child's skin. - Is there any scientific or experimental reason for these high-limits of the standard or they should be revised and te limit be decreased? | - THE VALUES WERE SELECTED AND AGREED BY THE DELEGATES AND NATIONAL COMMITTEES OF TC 61 - THE RELATIONSHIP BETWEEN TEMPERATURE AND BURN (superficial partial thickness burn) DEPENDS UPON THE CONTACT TIME. THE VALUES IN THE STANDARD WERE SELECTED BASED ON THE GUIDANCE GIVEN IN EN563 AND EN13202 - LIMITS CAN ALWAYS BE REVIEWED BUT IT IS NECESSARY FOR A NATIONAL COMMITTEE TO PROPOSE CHANGED LIMITS ALONG WITH A RATIONAL AND TECHNICAL REASONS TO SUPPORT THE CHANGE | None | 61(London/Chairman)2a | Iran |
| 62115 | 5.24.2 | Our engineers have different understandings about Clause 5.15 of IEC 62115 in our internal trainning last week. We are wondering if we could have IECEE/TC61’s clarification. Standard: IEC62115 Clause 5.15 “Before starting the tests, the toys is preconditioned by subjecting it to the testing of the following sub-clauses of ISO 8124-1, the batteries being in position. 5.24.2 Drop test, for toys having a mass less than 4.5kg, including batteries, irrespective of the age group.” And in ISO 8124-1 Clause 5.24.2, drop test is done, considering the age group and the toys quality as the following table: Age group; Mass criterion kg; Number of drops; Drop Height cm. Up to and including 18 months; <1,4; 10; 138+5. 19 months up to and including 96 months; <4,5; 4; 93+5. According to the above table, the toys up to and including 97 months should not do drop test. Then is it necessary for the up to and including 97 months toys having a mass less than 4.5 kg to do the Clause 5.15 drop test, irrespective of the age group? If it should do, how do? What are the conditions for the two age groups toys (up to and including 18 months and 19 months up to and including 96 months) to do the drop test? Question There are three understandings about Clause 5.15 drop test as the followings: 1. Irrespective of the age group, if do drop test to the toys having a mass less than 1.4kg, the drop height is 138cm and 10 times; and if do drop test to the toys having a mass less than 4.5kg, the drop height is 93cm and 4 times; 2. Irrespective of the age group, do drop test to the all toys having a mass less than 4.5kg, the drop height is 93cm and 4 times according with the ISO 8124-1 Clause 5.24.2 requirement to the toys less than 4.5kg; - Irrespective of the age group, do drop test to the all toys having a mass less than 4.5kg, the drop height is 138cm and 10 times according with the ISO 8124-1 Clause 5.24.2 requirement to the toys less than 1.4kg Could you kindly point out which is the correct understanding? | For the standard as written, your understanding number 2 is the correct interpretation | None | 61(London/Chairman)2a | China |
| 62115 | 9.9 | We Think the acceptable temperature rise that is assumed in this sub- clause is very much and can effects to burn or damage child's skin. For example the temperature rise equals 45 K for metallic accessible part (subclause 3-4-2) or 25 K for metallic touchable part, if the temperature of environment be 295 K, in this way the temperature of metallic part is 340 K or 330 K and it is very hot for children as well as adults | The temperature rises specified in clause 9.9 are in relation to an ambient temperature for the tests of 20 °C ± 5 °C see clause 5.11 of the standard. | None | 61(London/Chairman)2a | Iran |
| 62115 | 14.15 | In IEC 62115:2011 clause 14.15, the maximum current should be less than 0.5mA. And in another paragraph, it indicates as below: Compliance is checked by inspection and measurement. Protective parts or parts preventing access to live parts are removed, even if the toy has to be damaged. Shall we remove the protective part and measure the touch current directly from PCB? If so, I think the value will exceed 0.5mA for all most of the product. | The answer to the question is Yes. In answer to the observation – clause 14.15 was introduced by A2 to the standard to clarify requirements for “Internal parts of a toy having a voltage exceeding 24 V”. China 3: Confirmed | None | 61/4285/RM | China |