| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-80 |  | The minimum length of supply cord for fans are not specified in IEC standards. however in some of the national standards However the minimum length of supply cord defined in SLS 814 for Table fan is 2 m and pedestal fan is 3 m. Please indicate whether the minimum length requirement is important or not and if it is important what are the factors needs to be considered? what would be the reasonable length considering safety and performance? | The answer to your questions are as follows: - The minimum length of a supply cord is generally a performance matter and not a safety matter so no minimum length is specified in IEC 60335-2-80 because this deals only with safety. - Since this is not a safety matter, the length of the supply cord is over to the manufacturer to decide according to the needs of the market in which the fan is sold. 41b) Sri Lanka 1 - Confirmed | None | 61/3508/RM | Sri Lanka |
| 60335-2-80 |  | IEC 60335-2-80 : Household and similar electrical appliances – Particular requirements for fans. Clause 25 : Supply connections and external flexible cords IEC standard does not specify the length of supply cords or cables required for electric fans. In fact, Sri Lanka Standard (SLS 814) specifies the length requirement for supply cables 2 m minimum for Table fans and 3 m minimum for Pedastal fans. We shalll be thankful, if you could let us know why IEC 60335-2-80,.has not specified the length of supply cables for electric fans. We particular on this requirement as the fans imported to Sri Lanka carry cables with different lenghts. And also some of them are not within the specified length of the Sri Lanka standard. | a) The minimum length of a supply cord is generally a performance matter and not a safety matter so no minimum length is specified in IEC 60335-2-80 because this deals only with safety. b) Since this is not a safety matter, the length of the supply cord is over to the manufacturer to decide according to the needs of the market in which the fan is sold. 41b) Sri Lanka 2 – Confirmed | None | 61/3508/RM | Sri Lanka |
| 60335-2-80 |  | IEC 60355-2-80 Scope covers partition fans and duct fans also. We would be thankful, if you provide us, following information. - Can they be included for “household and similar” category. - Can same test methods be applied to measure performance requirements like air delivery for above mentioned fans ? (Test Methods included in *IEC 60879) Could you please comment on moving louver fans (Box fans) This type can be included under Table / Pedestal type? and same test methods are applicable ? Your co-operation in this regard will be very much appreciated. Angelo Perera Stds Engineer Sunitha-lethousehold-a Performance Requirements for Electric fans & Regulators. | 1. Partition fans and duct fans are included in the household and similar category, however there are some very large models that are clearly intended for industrial use and these models are not within the scope of 60335-2-80. 2. Moving louver fans can be tested to IEC 60335-2-80, whether they can also be classified as table type fans or pedestal fans depends on the design. 3. IEC 60879 concerning performance requirements for fans is dealt with by IEC/TC 59, your inquiry relating to air delivery etc has been passed on to this committee to answer. They have responded as follows: ‘’The standards for fans for household use, which are handled by TC59 are * IEC 60535 * IEC 60879 * IEC 60665 All three will be substituted soon by the new IEC 60879, which will refer to ISO 5801:1997, Industrial fans-Performance testing using standardised airways. Hoping that this answers your question sufficiently’’ i. Sri Lanka 1: The response in 61(Toulouse/Chairman)2a was confirmed | None | 61/2568/RM | Sri Lanka |
| 60335-2-80 | 21.101 | How the test of sub-clause 21.101 of EN 60335-2-80 shall be performed? | The standard does not require to use the test finger for the pull and push test, but it is agreed that the intention of this test is to check the mechanical resistance of the grid wires and not to check the introduction of the finger through the grid gap, that are covered by the test of 20.2. 41c) Agreed – but refer to the decision made on 61/3368/DC in Penang | None | 61/3508/RM | CTL Inquiry - 11/07 |
| 60335-1; 60335-2-80 | 23.3 | In clause 23.3 of such standard, is the fan sample only required to fulfill the "The appliance shall not be damaged to the extent that compliance with this standard is impaired and it shall be fit for further use. In particular, the wiring and its connections shall withstand the electric strength test of 16.3, the test voltage being reduced to 1 000 V and applied between live parts and accessible metal parts only."? If a 42 strands cord have 7 strands broken after test, and sample is operational and fulfill the high-voltage test above; is it necessary to consider the no. of broken strands is greater than 10% of total strands? | DRAFT RESPONSE (1) To answer this question, we need to know the following: 1) Were the conductors with the broken strands internal wiring (i.e. not accessible to test finger) or were they part of an interconnection cord (i.e accessible to the test finger and running between the fan base and the oscillating head. The broken strands found between the base and the oscillation body of the tower fan, they are only accessible if its base cover is open; 2) What was the rated current of the fan and the value of current in normal use through the conductors with the broken strands. The rated current is less than 0.2 A during normal use, through the cord with broken strands. DRAFT RESPONSE (2) From your description, even with 7 strands out of 42 broken, it appears that the fan withstood the HV test and is apparently still fit for further use so far as the user is concerned. It therefore must be ascertained whether it is still in compliance with the standard. i.e. would it still comply with the standard if the cross-sectional area of the internal wiring conductors were reduced by 16.67% ?. It is OK if it still complies with at least clauses 8, 11, 19, 21, 27 (if the fan is Class I) and 29 with 16.67% of the conductors broken. The limit of 10% only applies to the supply cord (and by inference an interconnection cord). | None | 61(Jeju/Chairman)2a | Hong Kong |
| 60335-2-80 | 29.2 | Recently, we arranged safety testing on samples of desk fans supplied in the local market. It has brought to our attention that several models of desk fan were found failed to comply with Clause 8.2, 16.3, 29.1 and 29.2 of IEC60335-2-80 in conjunction with IEC60335-1. We note that the manufacturers and our test laboratory have different interpretations about the safety standards. The question is on the interpretation of "detachable part". The definition of "detachable part" is given in Clause 2.7.2: Part which can be removed without the aid of a tool, a part which is removed in accordance with the instructions for use, even if a tool is needed for removal, or a part which does not fulfill the test of 22.11. As you know, the desk fan has fan guard and fan blade. Our test laboratory opined that the fan guard and fan blade are considered to be detachable parts according to Clause 2.7.2. There is not necessary to distinguish instructions for use, instruction manual, assembly instruction and maintenance instruction .. etc., as long as the literature provides the direction on how to assemble or disassemble the product. It is the fact that the fan guard and fan blade are able to be detached for cleaning and maintenance during its life time. Therefore when testing to Clause 8.2, 16.3, 29.1 and 29.2, the detachable parts (fan guard and fan blade) should be removed. The reinforced insulation between live parts and accessible part are required, unless the accessible metal parts are earthed effectively. Descriptions of non-compliance for those failed samples are as follows: i) Clause 8.2 - The motor shaft, which became accessible after the fan guard and the fan blade were removed, was found isolated from the motor winding with basic insulation only. ii) Clause 16.3 - Breakdown was occurred when a test voltage of 3750V was applied between live parts and motor shaft. iii) Clause 29.1 - The creepage distance and clearance between motor winding and stator core were found not complying with the minimum requirement of 6.0mm. iv) Clause 29.2 The thickness of the insulation between motor winding and stator core was found not complying with the minimum requirement of 2.0mm for reinforced insulation. However, the manufacturer has different opinion that the instruction for use is considered not including assembly or maintenance instructions. Thus, instructions inside the instruction manual describe the assembly of the appliance and disassembly of the appliance for maintenance or cleaning needs, which may not be considered as instruction for use. Thus, the fan guard and fan blade are not considered as detachable parts. If the fan guard and fan blade are not considered as detachable parts, there is no need to remove them during the tests for Clauses 8, 16 and 29. The tested samples will then comply with the requirements of Clause 8, 16 and 29 of the safety standards. For your information, among the failed samples, some models use guard clips (can be removed without the aid of a tool) while some models have screws to fix the fan guard (a tool is necessary to remove it). Removal of fan guard and fan blade has been mentioned in the instruction manual for maintenance and cleaning needs. The questions we came across are: i) the interpretation of detachable part and instruction for use; and ii) the test how to be conducted on the product we concerned. | Question 1: The instructions for user maintenance are a subset of the instructions for use therefore they are taken into account when deciding if a part is a detachable part. The instructions for installation (or assembly) are not taken into account when deciding if a part is a detachable part (see Note 1 in the definition of detachable part) – in this case 8.1.5 applies Question 2: If the manufacturer in the instructions for use, informs the user to remove a guard, cover or other part in order to carry out user maintenance, then clause 8 is carried out with the guard, cover or other part removed. The results of the clause 8 tests will then clarify whether class II construction is required in order for the appliance to comply with the standard. f. Hong Kong 1: The response in 61(Toulouse/Chairman)2a was confirmed | None | 61/2568/RM | Hong Kong |