| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-11 | 22.103 | I have question concern the interpretation of 22.103 of IEC60335-2-11:2012 22.103 If the instructions state that the tumble dryer can be placed on top of a washing machine, this shall be possible without the tumble dryer tilting or falling. Compliance is checked by inspection and by the following test. The washing machine and tumble dryer are assembled together in accordance with the instructions. The combination is placed in the most unfavourable orientation on a surface that is inclined at 5° to the horizontal. Each appliance is supplied at rated voltage and operated under normal operation in turn. The appliances shall not tilt and the tumble dryer shall not fall off the washing machine. QUESTIONS: 1. The appliances meaning washing machine and tumble dryer? 2. tilting meaning (ex. 3)? Thank you for your comment and regards Mr. Veerapat Ladnongkun veerapatl@thaieei.com Energy Test Group 2, Group Manager Operation Division 2 Testing and Calibration Department Electrical and Electronics Institute, Foundation for Industrial Development | Thailand 1: Confirmed DRAFT RESPONSE: Q1: “Appliances” means - the combination of washing machine and dryer; and - the dryer mounted on the washing machine. It should be noted that the washing machine cannot tilt without the dryer also tilting, but the dryer can tilt without the washing machine tilting. Q2: Tilting means any angle exceeding 5° from the vertical. | None | 61/5057A/RM | Thailand |
| 60335-2-11 | 20.104 | The combined washer/dryer below complies for the dryer function with IEC 60335-2-11:2008. When evaluating the construction according to Am 1:2012 20.104, additional tests on the electronic circuit and a software evaluation are required compare to the requirement of 20.101 of 60335-2-11:2008. As the standard is written, 20.104 applies, but it can be discussed if the intention of the sub-clause is to include also constructions with an opening which is 140 cm from floor level. Question We would like to know if sub-clause 20.104 of Am 1:2012 is relevant also to the construction below taking to account that the lower edge of the dryer opening is 140 cm above floor level? Best regards Leif Mattsson leif.mattsson@intertek.com | Sweden 1: Confirmed. DRAFT RESPONSE The intention of the standard is to protect against injury to children who climb into or are forcibly placed into the dryer followed by an intentional or unintentional closing of the dryer door. This can happen irrespective of the height of the dryer door above the floor. The introduction of electronic circuit requirements does not change this. If electronic circuits are not involved, the text in 20.104 is only an editorial rewrite of 20.101 in Ed 7. So the answer to the question is YES. Note: The dimension specified (door opening and volume) were chosen to provide protection without impacting too much on the functionality of the dryer. | 61(Washington/Chairman)02a - Sweden 1 | 61/4953/RM | Sweden |

Attachments

61(Washington/Chairman)02a – Sweden 1