| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-35 | 22.50 22.107 | I have some questions about clause 22.50 and 22.107 in IEC 60335-2-35: 2012(Ed5.0). Clause 22.50 states that: ……If the maximum temperature of the water from the system exceeds 55 °C in normal use then the requirement is not applicable provided that the system is such that a shower outlet normal use water temperature control takes precedence in setting the system temperature……. Clause 22.107 states that: For appliances intended to supply water for showering the test is carried out under normal operation and with a water pressure of 0,2 MPa. The temperature of the water at the outlet shall not exceed 55 °C. According to clause 22.50, I think the temperature of the water is allowed to exceed 55 °C for appliances intended to supply water for showering. But, According to clause 22.107, the temperature of the water at the outlet shall not exceed 55 °C. Question1: I want to know: for appliances intended to supply water for showering, whether the temperature of the water is allowed to exceed 55 °C? Question2: How to explain the conflict about water temperature between clause 22.50 and 22.107? Chen Wei Chenw@Cheari.com | China 9: Confirmed DRAFT RESPONSE The answer to your questions are: Q1: NO – IT CANNOT EXCEED 55 °C Q2: THERE IS NO CONFLICT TO EXPLAIN – PLEASE READ THE COMPLETE TEXT OF 22.50 IN PART 2-35. YOU CAN HAVE A SYSTEM WHERE ONE INSTANTANEOUS WATER HEATER SUPPLIES A NUMBER OF OUTLETS – SOME USED FOR SHOWERS AND SOME USED FOR OTHER PURPOSES. THE SHOWER OUTLET IS THE MASTER CONTROL FOR THE SYSTEM AND WHEN IN USE IT SETS THE TEMPERATURE FOR THE COMLETE SYSTEM REGARDLESS OF THE SETTING OF THE NON-SHOWER OUTLETS. | None | 61/4953/RM | China |
| 60335-2-35 | 24.102 22.107 | Thanks so much for taking the time to answer my question. IEC 60335-2-35:2012(Particular requirements for instantaneous water heaters) 1)The 4th paragraph of 24.102: The thermal cut-out is caused to operate five times, the temperatures at which it operates are measured and the mean value determined. The thermal cut-out is subjected to 50 000 cycles of temperature fluctuation. Each cycle consists of a variation in temperature between the maximum value measured during the test of 22.107 and half this value. 2)The 4th paragraph of 22.107:The temperature of the outlet water shall not be higher than 95°C and shall not exceed the temperature of the inlet water by more than 75 K. [Question] Which temperature shall we based on for the thermal cut-out being subjected during the 50 000 cycles of temperature fluctuation, the temperature on the surface of the thermal cut-out or the temperature of the outlet water? If it is the latter, that is to say, the temperature that shall be monitored is only the temperature of the outlet water. Is it right? Thanks for your kind guidance. Best regards. Zhang li 1908529809@qq.com Ningbo Joysun Product Testing Service Company | China 10: Confirmed DRAFT RESPONSE The temperature of the outlet water | None | 61/4953/RM | China |