| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-7 | General | We have an enquiry in relation to the rated capacity of washing machines, expressed in kg of dry clothes (IEC 60456- TC 59/SC 59D and IEC 60335-2-7 TC 61) About this in particular, I would be thankful if you could confirm the following for us: a) Whether the adequate standard for the determination of the rated capacity is the IEC standard 60456 or the IEC 60335-2-7. We understand that the IEC 60456 was related to the performance (energy efficiency) and it´s not the best standard to confirm the rated capacity. b) Confirm if we should start by working with the rated capacity declared by the manufacturer, and with this value performs the tests methods of the IEC 60335-2-7 standard. In this way, the rated capacity is validated. Your answer will be very import for us. Regards, Carmela Morgan cmorgan@inacal.gob.pe> Secretary Peru NEC (IEC Affiliates programme member) | Peru 1: Confirmed DRAFT RESPONSE The answers to the questions are: - For Performance it is necessary to use IEC 60456 and the rated capacity may be different for different textile types. Clause 4.2 of that standard states: These ratings do not need to be marked on the washing machine or provided in the instructions – they are only required to be declared for performance testing. For safety it is necessary to use IEC 60335-2-7 and the manufacturer is required to specify in the instructions provided to the user the maximum load. This standard states The instructions shall specify the maximum mass of dry cloth in kilograms to be used in the appliance - Based on the information above you use the maximum mass of dry cloth in kilograms to be used in the appliance declared by the manufacturer in the instructions and test to IEC 60335-2-7. In this way the capacity of the washing machine is validated for safety. | 61(Sydney/Chairman)02a - Peru 1 | 61/5057A/RM | Peru |
| 60335-2-7 | 20-101 20-103 | I want to know the “Washing machines of the drum type” mention in IEC60335-2-7 cl.20.101 and cl.20.103 refer to TYPE 1 in particular? Or it also refer to TYPE 2? If the requirement in 20.103 also applicable to TYPE 2? PS: 20.101 Washing machines of the drum type which are loaded from the top through an opening with a hinged lid, shall incorporate an interlock which de-energizes the motor before the lid opening exceeds 50 mm. If a removable or sliding lid is provided, the motor shall be de-energized as soon as the lid is removed or displaced and it shall not be possible to start the motor unless the lid is in the closed position. The interlock shall be constructed so that unexpected operation of the appliance is unlikely unless the lid is in the closed position. 20.103 For washing machines of the drum type that are loaded from the front or from the top, the door or lid shall be interlocked so that the appliance can only be operated when the door or lid is in the closed position. Best regards Hu Runze hurz@cvc.org.cn | China 4: Confirmed DRAFT RESPONSE It is Type 1. Just for further information. Drum type washing machine do not have a vertical axis of rotation – their axis of rotation is generally horizontal. Most are loaded from the front but some are loaded from the top by opening the hinged lid that then reveals a sliding door in the drum through which the clothes load is inserted. | 61(Sydney/Chairman)02a - China 4 | 61/5057A/RM | China |
| 60335-2-7 | 3.1.9 | Dear Sir: About the following part of Sub-Clause 3.1.9 of IEC 60335-2-7: " The appliance is filled with textile material ……The textile material consist of prewashed double hemmed cotton sheets " 1) - What is the acceptable percentage of cotton in this sheet? 2) - Is 96% of cotton acceptable (for example)? Or should it be only 100%? 3) - Is there any IEC guidelines related to this subject? Best regards Asma’a Quraish asmaaqurish@yahoo.com Amman Jordan | Jordan 1: Confirmed DRAFT RESPONSE Since there is no statement of the cotton % it means 100% cotton. | None | 61/4953/RM | Jordan |

Attachments

61(Sydney/Chairman)02a – Peru 1

61(Sydney/Chairman)02a – China 4