| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-14 | 20.106 | Dear Mr. Derek Johns, Happy new year! We are a test lab and looking for IEC/TC61 to clarify maximum cross-section dimension in clause 20.106 of IEC 60335-2-14. Question: Does the special tumble-type hopper with maximum cross-section dimension exceeding 45 mm and 4mm height comply with the intent of clause 20.106 of IEC 60335-2-14? Figure 1 How the food drops to the crushing cone / special tumble-type hopper. Figure 2 Maximum dimension of cross-section of hopper with a special angle open. Maximum dimension of cross-section: Measured > 45mm with only 4mm thickness/width. Conclusion: no one can put his finger to the moving part cone, although the max. dimension exceeding 45mm. The argument is that there is no limit of width/ of cross-section of hopper Looking forward to your reply soon! Thank you very much for co-operation! | China 5: Confirmed DRAFT RESPONSE Since the clause states in part: The maximum cross-sectional dimension of the hopper, measured at least 100 mm from the upper edge of the feed screw, shall not exceed 45 mm. Assessing the design against these dimensions for compliance with the standard as written and from the information that you supplied, it is considered that this design does not comply with the standard. NOTE: Some members of the CAG are of the opinion that they would be sympathetic to a change to the standard since they consider the aim of the requirement is to allow food to enter but to avoid hands and fingers touching dangerous moving parts. For that reason it could be argued that the standard is too restrictive with its requirement. | 61(Washington/Chairman)02a – China 5 | 61/4953/RM | China |
| 60335-2-14 | 20.2 and 20.114 | We have a kitchen machine that passes clause 20.2 because the feed opening complies with all 3 dimension requirements for the feed opening: The test probe is not applied to feed openings having a throat with the following dimensions: – a height of at least 100 mm, measured from the upper edge of the cutting blade; – an average of the maximum and minimum cross-sectional dimensions of the feed opening that does not exceed 65,5 mm; – a maximum cross-sectional dimension of the feed opening that does not exceed 76 mm. However we now have a test lab that states that the same appliance does not pass clause 20.114, because with the feed pusher removed, the test probe B can touch the cutting blades. We do not agree with this test lab because of the following reasons: Clause 20.2 is about touching dangerous moving parts in general. Feed openings with specific dimensions are explicitly exempted and therefore can be regarded as safe according to this standard. Clause 20.114 has a different purpose. It investigates the situation for a food processor which is “assembled incorrectly in a manner that can occur in use, such as the incorrect location or misalignment of the parts.” Therefore the focus of clause 20.114 should be on incorrect assembly or misalignment only and not on feed openings, since they are already adequately covered by clause 20.2. Next to that it cannot be the intend of the standard that for compliance with clause 20.2 we apply the modified test probe B with circular stop face 50 mm to evaluate the feed opening and then for compliance with clause 20.114 we apply another test probe being the non-modified test probe B (non-circular stop face 50 mm) to evaluate the feed opening again. As the probes are different this could and will lead to different outcomes. Is our interpretation correct? | Yes – your interpretation is correct |  | 61/4953/RM | Netherlands |
| 60335-2-14 | 7.12 | I got the case with consumer protection association (CPA) related to interpretation of wording of the standard. Clause 7.12 of IEC 60335-2-14:2007 +A1:2009 prescribes that the user manual shall include the operating time and speed settings of the accessories. The discussion point are appliances without accessory (see table below) which perform only basic function. The in table described appliances are basic appliances without accessory. These appliances are intended for household use and tested regarding IEC 60335-1 and IEC 60335-2-14 standards. Heating test is performed in line with Clause 7 of IEC 60335-1 and IEC 60335-2-14 standards without shortening of operating times and definition of speed settings in instruction manual. Appliances have been tested also in independent institutes which granted valid certificates shoving compliance with valid cited standards. The CPA claimed non compliance with the standard because there is no indication of operating time and speed settings in instruction manual. For these appliances we do not set any limitation and user is free in use ( no speed and time limitation) of them. From my point of view the mixing rod, whisks and kneading hooks are components of appliance and not accessory. For my understanding accessory to the appliance is every attachment provided by manufacturer for performing additional function to appliance basic functions. Kind regards, Danilo Kraner danilo.kraner@bshg.com | Slovenia 1: Confirmed DRAFT RESPONSE For the standard a written it is necessary to state the speed settings and operating times for accessories. If a basic appliance is supplied with different blades to perform different mixing functions then these are accessories – within the meaning of the standard. Refer to Iran 1 in 61(Washington/Chairman)02a. However for the appliances identified it seems that for the basic appliances identified, the basic appliance is not supplied with different “blades” (accessories) – the basic appliance is the hand-held part plus the detachable supplied single set of blades which is not an accessory that changes the basic function of the appliance. The following should be considered: All of the times specified in 11.7.10x are the maximum times considered by the committee that the appliance will be operated under the most onerous normal use conditions. So if the manufacturer specifies a longer time – this would be considered performance as safety is ensured by the 11.7.10x test times. If the manufacturer specifies a shorter time again safety is ensured by the 11.7.10x test times – unless the alternative test is required due to failure of the 11.7.10x test. So specifying a time is not really necessary for safety under the risk analysis that has led to the 11.7.10x times and especially if the alternative test allowance is not needed. If the alternative test is needed then an operating time must be specified in the instructions. Concerning the speed settings. These are not needed for safety since Part 1 requires these controls to be at the most unfavourable setting (see 5.6) and 11.7.10x for two of the appliances identified requires them to be at the highest setting. See Blenders: 11.7.103 (highest setting) Cream whippers: 11.7.109 (highest setting) Food mixer (without dough kneaders): 11.7.110 (most unfavourable setting via 5.6 of part 1). Note: For food processors the operating time is required in the instructions for 11.7.110 test and for all appliances it is also required if the alternative test allowance is invoked. The operating time is also required for appliances not mentioned in 11.7.10x. May be the text in 7.12.1 should be clarified. If necessary it could be discussed by delegates at the Sydney meeting and go into Ed 6 of the standard. | 61(Sydney/Chairman)02a - Slovenia 1 | 61/5057A/RM | Slovenia |

Attachments

61(Washington/Chairman)02a – China 5

61(Sydney/Chairman)02a – Slovenia 1

| Appliance description | Appliance drive unit with cord attached | Appliance component for performing basic function | Appliance assembled for basic function | Basic function (Product description ) |
| --- | --- | --- | --- | --- |
| Wand blender |  |  |  | Blender can perform a variety of tasks including blending soups, sauces, smoothies and even baby food. It has a 400W quiet, low vibration motor, stainless steel blades and the easy eject foot is simple to remove. Features Soft touch ergonomically shaped handle for comfortable grip Dishwasher safe blender foot and attachments |
| Cream whipper or Egg beater |  |  |  | Powerful 350W motor Modern drive unit technology: powerful, silent and weight light Ergonomically shaped “soft-touch” housing 5 speed setting 2 highly effective beaters in stainless steel: for great whipping and beating results in the shortest time 2 kneading hooks Additional turbo speed setting Separate wiper ejection button |
| Handheld food mixer |  |  |  |  |