| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-65 | 32 | We are professional commercial washroom cleaning service provider and we are looking into appliances utilizing ozone technology to clean air/water (not for drinking). We note the IEC 60335-2-65 Clause 32 has exposure limit and test environment defined for Air Purifiers. However, some Air Cleaners we are studying produce ozone as an intended or bypass product over the 5 x 10^-6 % (0.05ppm) limit in Clause 32 but still under the UK HSE's 0.2ppm for 15 minutes threshold. One particular product is immersed in water pipe and small amount of ozone may be release into the air above water. We discussed with certain air quality research lab what ozone exposure level is appropriate for office/commercial environment (note: the appliances are operated by trained technician not layman). Their opinion is different from what's in Clause 32. The accredited electrical safety test labs we contacted were not able to fully support the assessment or design review either. Q1: (a) I wonder if you can kindly advise us if IEC 60335-2-65 Clause 32 is the appropriate safety requirement for our air cleaners and (b) whether the ozone limit is variable for the usage/environment we intend for? Q2: For the water cleaner with ozone bypass product, I assume Part 2-65 is not applicable? Q3: If so, what level of exposure do we need to certify the product in addition to IEC 60335-1? Your feedback would be much appreciated and it will be crucial for us to fully understand the IEC safety guidance for this type of new appliances in international market. Many thanks and Happy New Year. Kind Regards Ning Pan (ning.pan@rentokil-initial.com) | United Kingdom 1: Confirmed DRAFT RESPONSE Q1 (a): IEC 60335-2-65 applies to air cleaning appliances that produce ozone that may expose the general public to its effects. Please refer to the scope that states (in Part): This International Standard deals with the safety of electric air-cleaning appliances for household and similar purposes, their rated voltage being not more than 250 V for single-phase appliances and 480 V for other appliances. Appliances not intended for normal household use but that nevertheless may be a source of danger to the public, such as appliances intended to be used by laymen in shops, in light industry and on farms, are within the scope of this standard. Q1 (b): The level in Clause 32 is not variable for appliances within the scope. Q2: Correct – Please refer to IEC 60335-2-109 for UV water treatment appliances that produce ozone. Q3: The limit in the test specification for appliances within the scope of the standard (and of IEC 60335-2-109) is set at 50% of the generally accepted ozone odour threshold of 0,1 ppm. Please also refer to the second dashed item in Note 101 in the scope of the standard (2-65 and 2-109) that states: NOTE 101 Attention is drawn to the fact that – for appliances intended to be used in vehicles or on board ships or aircraft, additional requirements may be necessary; – in many countries additional requirements are specified by the national health authorities, the national authorities responsible for the protection of labour and similar authorities. | None | 61/4953/RM | United Kingdom |