| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-26 | General | We are now involved in harmonizing work of IEC 60335-2-26(Edition 4_1) and have following question. 1. Although it is not clarified that the appliance covered by the standard is mainly actuated by motor, we think it is of course yes isn't it. 2. I am not familiar to the construction of the clock and I cannot understand what construction or operation of the clock is assumed in the requirement of 3.1.9 of the standard (Other clocks are operated with the rotor locked). Is the rotor is liable to be locked as in normal use of the clocks other than spring-operated ones having an electrically operated winding mechanism? Please let me know on the above question. Follow up Inquiry: Just I cannot understand is if the two states of condition between rotor locked and rotor not locked is exactly same? Otherwise, is the condition, rotor locked realistic as normal operation? I still do not really understand what the merit of locking rotor as normal operation, and what the demerit of not locking rotor as normal operation. | Japan 1: Confirmed DRAFT RESPONSE: Q1: Yes - there is always a motor but it may not drive the hands of the clock Q2: It simulates a clock where the hands of the clock only move when they have to so the motor experiences starting current as part of normal operation. So the easiest way to get reproducible results is to lock the rotor. DRAFT RESPONSE The requirement in the standard has been this way since the first edition was published in 1977. | None | 61/4953/RM | Japan |