| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-98 | 22-33 | I tested a humidifier according to IEC 60335-2-98 . top of humidifier have an hole . Water into the container because of contact with the electrode is energized. when we pour water inside humidifier up to marked level probe B don’t touch the water. but when we pour water inside humidifier over marked level probe B touch the water. in according to 22-33 if probe B or 13 to touch water , this appliance failed. clause 22-33: Conductive liquids that are or may become accessible in normal use and conductive liquids that are in contact with unearthed accessible metall parts shall not be in direct contact with live parts. Mojtaba Masoumi m.masoumi66@gmail.com | Iran 10: Confirmed DRAFT RESPONSE: There is an addition to 22.33 in Part 2-98 that states: “Liquids may be heated using electrodes and may be in direct contact with their live parts”. Normal operation of the appliance is with it filled with water to its rated capacity – see 3.1.9 and 7.1. So the answer to the question is: The appliance does not fail 22.33 but from the photo provided it does not comply with 22.103. | 61(Sydney/Chairman)02a - Iran 10 | 61/5057A/RM | Iran |

Attachments

61(Sydney/Chairman)02a – Iran 10