| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-9 | 3.122 22.114 | 60335-2-9 Ed. 6.1 added “candy floss appliances” in the scope of Clause 1. They are defined in Clause 3.122 as, “appliance comprised of a heater and spinning head intended for preparing candy floss” Additionally, Clause 22.114 states, “Heating elements in candy floss appliances shall be located so that they are not exposed to sugar during normal use of the appliance.” (Please refer to Example 1) This unit would meet the 22.114 requirement. As you can see in Example 2, the visibly glowing heating elements are located below the spinning head. This may certainly work for a small portion of candy for a child at home, but is not practical for a typical cone of cotton candy at a local county fair. Unfortunately, this virtually eliminates another design of candy floss appliances where the manufacturer specifically intends for the element to melt the sugar. Example 3 is a typical candy floss machine (minus the bowl). Example 4 and Example 5 show typical spinner head constructions that are designed for the elements to melt the sugar. This device also clearly meets the definition of Clause 3.122. Furthermore, it is portable, and in our opinion, also meets the Scope of Clause 1 as “… household and similar use and that may also be used by laymen in shops, in light industry…” It is not “… intended to be used professionally to process food for commercial consumption …” We therefore feel that 60335-2-9 is in fact the proper standard for these appliances. However, Clause 22.114 expressly forbids this type of construction as the elements are most certainly in contact with the sugar during normal use. The majority of candy floss appliances in use worldwide today are as in Example 3. Machines of this style have been in production since 1960. Our questions are as follows: - Are IEC 60335-1 and IEC 60335-2-9 the relevant standards for the specific design of Candy Floss Appliances (as stated above in Examples 3-5)? - If the answer to question 1 is “yes,” is 22.114 only applicable to the design of Examples 1 and 2 where the sugar is not intended to contact the heating elements? That is, 22.114 is not applicable to Example 3 where the intended function of the appliance is for sugar to contact the heating elements? | United States 1: Confirmed DRAFT RESPONSE: Q1. NO. From the description, Example 3 is used to “process food for commercial consumption” this means they are commercial appliances. The phrase in the standard actually states “However, if the appliance is intended to be used professionally to process food for commercial consumption, the appliance is not considered to be for household and similar use only.” It is always considered in the 60335 series that appliances for commercial use are operated by trained persons (i.e. used professionally) because of workplace laws in many countries. The phrase in the standard that states “may also be used by laymen in shops, in light industry and on farms” is intended to mean use by workers for their own use in preparing their lunch etc not use as part of their workplace requirements. See also the last paragraph of 7.12 in Part 2-9. This is generally applicable over the 60335 series of standards. Q2. Not relevant because of the answer to Q1. | 61(Sydney/Chairman)02a - USA 1 | 61/5057A/RM | USA |
| 60335-2-9 | 20-101 | According to IEC 60335-2-9 sub clause 20.101: “Ovens having doors with a horizontal hinge at their lower edge and on which a load is likely to be placed shall have adequate stability. Compliance is checked by the following test. The oven is placed on horizontal surface with the door open and a mass of 3,5 kg is gently placed on the geometric center of the door. NOTE A sandbag may be used for load. The oven shall not tilt. This is not carried out on ovens with doors having a dimension less than 225 mm from the hing to the opposite edge or on ovens with doors which cannot support dishes in the fully open position. “ where is opposite edge? A= 215 mm & B= 34.5 mm. which kind of measurement is true? A or B? or both of them? Best Regards Farhad Moqadami fmoqadami@gmail.com | Iran 1: Confirmed for IEC 60335-2-9 DRAFT RESPONSE The answer is “A” | 61(Sydney/Chairman)02a - Iran 1 | 61/5057A/RM | Iran |
| 60335-2-9 | General | I have two questions hope you can help me out? (Q1) Is remote operation of a contact grill allowed? The contact grill has no own controls, but instead is operated by means of mobile phone to fully control (start, stop) the contact grill via bluetooth. I would consider it dangerous, because appliances can be switched on without the operator having visibility on the appliance. But I have doubt due to the requirements in the standard. Contact grills are according IEC 60335-2-9 clause 30.2 attended appliances as it states: " For other appliances, the tests of 30.2.2 are applicable.". But attended for me suggest visibility on the appliance controlled, which is not guaranteed for a bluetooth remote control. Furthermore IEC 60335-2-9 clause 7.12 states: " The instructions shall state that appliances are not intended to be operated by means of an external timer or separate remote-control system." However IEC 60335-1 clause 22.49 / 22.50 / 22.51 seem to allow remote control if it does not create a hazard. But IEC 60335-2-9 has not modifications on clauses 22.49 / 22.50 / 22.51. (Q2) Is remote operation of an airfryer/actifry kind of product allowed? These small air circulating ovens are operated by means of mobile phone to fully control (start, stop, set timer) the appliance via bluetooth. <Rational see above.> Dinand van Aalderen dinand.vanaalderen@dekra.com | Netherlands 3: The answer to both questions is NO (and delete references to instructions) DRAFT RESPONSE Q1: No – because of the statement in the instructions. Q2: No - because of the statement in the instructions. All members of the CAG agreed with the principle of the draft response, but some considered that it needed to be more clearly stated as an addition to 22.40 (as per IEC 603352-6) that remote control of appliances is not allowed. More details about bluetooth can be found at www.bluetooth.com/Pages/Basics.aspx | None | 61/4953/RM | Netherlands |
| 60335-2-9 | General | Clarification on IEC 603355-2-9 Edition 6.1, 2012-11 This has reference to the discussion, we had at our mirror Committee with regard to adoption of it as a Sri Lanka Standard. Please be kind enough to clarify whether the Induction cookers are covered by this standard? If so Please indicate the location where examples of appliances given in 1 Scope Early response in this regard is very much appreciated Regards T Uthayakumar (uthaya@slsi.lk) Secretary National Committee of the IEC, Sri Lanka | Sri Lanka 1: Confirmed DRAFT RESPONSE IEC 60335-2-9 is the standard for grills, toasters and similar portable cooking appliances and in this standard an induction cooker is referred to as an induction hotplate – see definition 3.113. An induction hot plate is just a special type of hot plate. A hot plate is listed in the scope in Note 101 and is defined in 3.112. IEC 60335-2-6 is the standard for stationary cooking ranges, hobs, ovens and similar appliances and in this standard an induction cooker is referred to as an induction hob – see definition 3.108. Please realise that where examples of appliances are given in a Note in the scope of a standard, they are just examples and the list is never fully inclusive of all appliances within the scope of the standard. | None | 61/4953/RM | Sri Lanka |

Attachments

61(Sydney/Chairman)02a – USA 1

| Example 1 | Example 2 |
| --- | --- |
| Current example of household candy floss appliance | Visibly glowing elements under head. |

61(Sydney/Chairman)02a – Iran 1