| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-8 | 20.2 | Below are examples of a fabric shaver to removing pilling and fuzz from fabric such as woolen clothing. The appliance has a power input of approximately 2 W. The plastic pilling ejector fan is accessible when the pilling ejection collector is removed. The fan rotates at about 3000 r/min and has an out diameter of about 4 cm. 1) Is this appliance within the scope of IEC 60335-2-8? 2) Does it comply with Clause 20.2 i.e. is the ejector fan a dangerous moving Regards Wendy Huang huangwx@cvc.org.cn | China 8: Confirmed DRAFT RESPONSE - This appliance is not within the scope of IEC 60335-2-8 but the principles of Part 2-8 could be applied. However, there is no need to apply the cord the length requirement in 25.7. - Following the application of 20.2 in IEC 60335-2-14 for centrifugal juicers, the test of 20.2 is carried out with the container for collecting the fuzz in position. | 61(Washington/Chairman)02a – China 8 | 61/4953/RM | China |

Attachments

61(Washington/Chairman)02a – China 8

图1

图2