| Document | Reference | Inquiry | Response | Attachment | RM | Country |
| --- | --- | --- | --- | --- | --- | --- |
| 60335-2-29 | 22.26 | IEC 60335-2-29 subclause 22.26. What is the intent of this subclause for not allowing connection of the output circuit, which is already separated from the primary circuit by safety isolating transformer, to the accessible metal parts or earthing terminal even if the battery charger is a class I appliance? Is it acceptable to connect output circuit to earthing through a Y2 capacitor? cecilio.dimasacat@setec.com.au | Australia 1: Confirmed DRAFT RESPONSE The reason is that the earth terminal or earthed accessible metal parts are not double insulated from live parts on the primary side of the safety isolating transformer, so connecting the output to such parts bypasses the safety isolating transformer requirements. Connecting unearthed accessible metal parts to the secondary circuit can lead to shock hazards in the event of a fault within the charger or loss of earth if the accessible metal parts are intended to be earthed. This clause also prohibits the use of PELV on the secondary side. The use of Y2 capacitors is not prohibited but to meet the requirements for protective impedance, two Y2 capacitors of appropriate value must be used. | None | 61/5057A/RM | Australia |